# New Zealand’s Greenhouse Gas Inventory 1990–2019

# Volume 2, Annexes

Fulfilling reporting requirements under the 
United Nations Framework Convention on 
Climate Change and the Kyoto Protocol

Contents

Tables	v

Figures	xi

Annex 1: Key categories	1

A1.1 	Methodology used for identifying key categories	1

A1.2 	Disaggregation	2

A1.3 	Tables 4.2–4.3 of the IPCC 2006 Guidelines (General Guidance and Reporting)	2

Annex 1: References	20

Annex 2: Uncertainty analysis	21

A2.1 	Approach 1 uncertainty calculation	21

Annex 2: References	32

Annex 3: Detailed methodological information for other sectors	33

A3.1	Supplementary information for the Agriculture sector	33

A3.2	Supplementary information for the LULUCF sector	43

Annex 3: References	69

Annex 4: Methodology and data collection for estimating emissions from fossil fuel combustion	71

A4.1	Emissions from liquid fuels	74

A4.2	Emissions from solid fuels	79

A4.3	Emissions from gaseous fuels	80

A4.4	Energy balance	80

A4.5	Carbon dioxide reference approach for the Energy sector	81

Annex 4: References	83

Annex 5: Supplementary information for the KP-LULUCF sector	84

A5.1	Technical corrections to the FMRL	84

A5.2 	Natural disturbance	94

A5.3	Carbon equivalent forests	96

Annex 5: References	99

Annex 6: Additional information on the inventory system and completeness	100

A6.1 	Quality assurance and quality control processes	100

A6.2 	General assessment of completeness	103

Annex 6: References	110

Annex 7: Tokelau	111

A7.1	Emissions estimate data and relevant supporting information by category for Tokelau	111

Annex 8: Agricultural emissions from fertilisers and by livestock type	187

A8.1 	Agricultural emissions from fertilisers	187

A8.2 	Agricultural emissions by livestock type	189

See volume 1, chapters 1–15, for the National Inventory Report, which is presented separately.

# Tables

Table A1.3.1(a)	Results of the key category level analysis for 99 per cent of the net emissions and removals for New Zealand in 2019	2

Table A1.3.1(b)	Results of the key category level analysis for 99 per cent of the gross emissions and removals for New Zealand in 2019	5

Table A1.3.2(a)	Results of the level analysis for 99 per cent of the net emissions and removals for New Zealand in 1990 included for reference only	8

Table A1.3.2(b)	Results of the level analysis for 99 per cent of the gross emissions for New Zealand in 1990 included for reference only	10

Table A1.3.3(a)	Results of the key category trend analysis for 99 per cent of the net emissions and removals for New Zealand in 1990–2019	13

Table A1.3.3(b)	Results of the key category trend analysis for 99 per cent of the gross emissions for New Zealand in 1990–2019	16

Table A2.1.1 	Uncertainty calculation (including LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2019 (IPCC, 2006, Approach 1)	23

Table A2.1.2 	Uncertainty calculation (excluding LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2019 (2006 IPCC, Approach 1)	28

Table A3.1.1	Imputation levels and sampling errors for recent Agricultural Production surveys	33

Table A3.1.2	Characterisation of major livestock subcategories (dairy cattle, beef cattle, sheep and deer) in New Zealand’s Tier 2 livestock modelling	34

Table A3.1.3	Proportion of annual milk yield each month for major livestock categories	35

Table A3.1.4	Methane emission factors for Tier 1 enteric fermentation livestock and manure management	36

Table A3.1.5	Emission factors for New Zealand’s agriculture nitrous oxide emissions	36

Table A3.1.6	Direct nitrous oxide emission factors for urine deposited by cattle, sheep and deer, by livestock type and slope	37

Table A3.1.7	Parameter values for New Zealand’s agriculture nitrous oxide emissions	37

Table A3.1.8	Parameter values for New Zealand’s cropping emissions	38

Table A3.1.9 	Allocation of urine deposition to low slope (0–12 degrees) and steep slope (more than 24 degrees), split by the percentage of low slope and steep slope land available	39

Table A3.1.10	Share of livestock population, and amount of urine nitrogen (N) deposition in 2019, by Beef + Lamb New Zealand farm class	41

Table A3.1.11	Proportion of total sheep, beef and deer land on different hill slopes, by Beef + Lamb New Zealand farm class, for 2018/19 year	41

Table A3.1.12	Proportion of total sheep, beef and deer urine nitrogen deposited on different hill slopes, by Beef + Lamb New Zealand farm class, for 2019	42

Table A3.2.1 	Annual land-use changes (units in thousand hectares)	43

Table A3.2.2	Pre-1990 natural forest – tall forest yield table (tonnes C ha–1)1	50

Table A3.2.3	Pre-1990 natural forest – regenerating forest yield table (tonnes C ha–1)1	51

Table A3.2.4	Post-1989 natural forest yield table (tonnes C ha–1)1	52

Table A3.2.5	Pre-1990 ‘planted before 1990’ planted forest yield table (tonnes C ha–1)1	53

Table A3.2.6	Pre-1990 ‘planted between 1990 and 2009’ planted forest yield table (tonnes C ha–1)1	54

Table A3.2.7	Pre-1990 ‘planted between 2009 and 2019’ planted forest yield table (tonnes C ha–1)1	56

Table A3.2.8	Post-1989 planted forest yield table (tonnes C ha–1) 1	57

Table A3.2.9	Uncertainty analysis for the LULUCF sector	59

Table A4.1 	Gross carbon dioxide emission factors used for New Zealand’s energy sector in 2019	71

Table A4.2 	Consumption-weighted average emission factors used for New Zealand’s sub-bituminous coal-fired electricity generation for 1990 to 2019	72

Table A4.3 	Methane emission factors used for New Zealand’s energy sector for 1990 to 2019	73

Table A4.4 	Nitrous oxide emission factors used for New Zealand’s energy sector for 1990 to 2019	74

Table A4.5 	Gross calorific values (MJ/kg) for liquid fuels for 1990 to 2019	75

Table A4.6 	Carbon content (per cent mass) for liquid fuels for 1990 to 2019	76

Table A4.7 	Emission factors for European gasoline and diesel vehicles – COPERT IV model (European Environment Agency, 2007)	77

Table A4.8 	Split of energy use and non-energy use of natural gas in petajoules	80

Table A5.1.1 	Pre-1990 planted forest data used to estimate emissions for the technically corrected FMRL	88

Table A5.1.2	Summary of the technical corrections to the FMRL	92

Table A5.3.1	Breakdown of carbon equivalent forests by domestic scheme application from 2014 to 2019	96

Table A6.1.1	Quality assurance and quality control processes used in preparation of the inventory	100

Table A6.2.1 	Summary of NE (not estimated) entries in 2021 submission	103

Table A6.2.2 	Emissions reported using the ‘IE’ (included elsewhere) notation key	106

Tokelau CRF Table 1.A.1.a: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] (Part 1 of 3)	111

Tokelau CRF Table 1.A.1.a: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] (Part 2 of 3)	112

Tokelau CRF Table 1.A.1.a: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] (Part 3 of 3)	114

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] (Part 1 of 3)	115

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] (Part 2 of 3)	116

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] (Part 3 of 3)	116

CRF Table 1.A.3.b.i Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] (Part 1 of 3)	117

CRF Table 1.A.3.b.i Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] (Part 2 of 3)	118

CRF Table 1.A.3.b.i Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] (Part 3 of 3)	119

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] (Part 1 of 3)	120

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil (Part 2 of 3)	121

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil (Part 3 of 3)	121

CRF Table 1.A.4.b: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] (Part 1 of 3)	122

CRF Table 1.A.4.b: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] (Part 2 of 3)	124

CRF Table 1.A.4.b: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] (Part 3 of 3)	125

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing] [1.A.4.c.iii Fishing] [Gas/Diesel Oil] (Part 1 of 3)	127

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing] [Gas/Diesel Oil] (Part 2 of 3)	128

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing] [Gas/Diesel Oil] (Part 3 of 3)	129

CRF Table 1.AB Gasoline: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gasoline] (Part 1 of 3)	130

CRF Table 1.AB Gasoline: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gasoline] (Part 2 of 3)	131

CRF Table 1.AB Gasoline: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gasoline] (Part 3 of 3)	132

CRF Table 1.AB Gas Diesel Oil: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 1 of 3)	133

CRF Table 1.AB Gas Diesel Oil: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 2 of 3)	134

CRF Table 1.AB Gas Diesel Oil: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 3 of 3)	135

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] (Part 1 of 3)	136

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] (Part 2 of 3)	137

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] (Part 3 of 3)	138

CRF Table 1.AB LPG: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 1 of 3)	139

CRF Table 1.AB LPG: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 2 of 3)	140

CRF Table 1.AB LPG: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 3 of 3)	141

CRF Table 1.AB Lubricants: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] (Part 1 of 3)	142

CRF Table 1.AB Lubricants: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] (Part 2 of 3)	143

CRF Table 1.AB Lubricants: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] (Part 3 of 3)	144

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning] [ 2.F.1.b Domestic Refrigeration][ HFC-134a] (Part 1 of 3)	145

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning] [ 2.F.1.b Domestic Refrigeration][ HFC-134a] (Part 2 of 3)	145

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] (Part 3 of 3)	146

CRF Table 2.F.1.f: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air Conditioning] (Part 1 of 3)	147

CRF Table 2.F.1.f: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] (Part 2 of 3)	148

CRF Table 2.F.1.f: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] (Part 3 of 3)	149

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-32] (Part 1 of 3)	150

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-32] (Part 2 of 3)	150

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-32] (Part 3 of 3)	151

CRF Table 2.F.1.f HFC-125: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-125] (Part 1 of 3)	152

CRF Table 2.F.1.f HFC-125: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-125] (Part 2 of 3)	152

CRF Table 2.F.1.f HFC-125: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-125] (Part 3 of 3)	153

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-134a] (Part 1 of 3)	154

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-134a] (Part 2 of 3)	154

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] (Part 3 of 3)	155

CRF Table 2.F.4.a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] (Part 1 of 3)	156

CRF Table 2.F.4.a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] (Part 2 of 3)	157

CRF Table 2.F.4.a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] (Part 3 of 3)	158

CRF Table 2.F.4.a HFC-134a: [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] (Part 1 of 3)	159

CRF Table 2.F.4.a HFC-134a: [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] (Part 2 of 3)	159

CRF Table 2.F.4.a HFC-134a: [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] (Part 3 of 3)	160

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] (Part 1 of 3)	160

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] (Part 2 of 3)	161

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] (Part 3 of 3)	161

CRF Table 2.G.3.a: [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] (Part 1 of 3)	162

CRF Table 2.G.3.a: [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] (Part 2 of 3)	162

CRF Table 2.G.3.a: [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] (Part 3 of 3)	163

CRF Table 3.A.3 Tokelau Swine: [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] (Part 1 of 3)	163

CRF Table 3.A.3 Tokelau Swine: [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] (Part 2 of 3)	164

CRF Table 3.A.3 Tokelau Swine: [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] (Part 3 of 3)	165

CRF Table 3.A.4 Tokelau Poultry [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] (Part 1 of 3)	166

CRF Table 3.A.4 Tokelau Poultry [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] (Part 2 of 3)	167

CRF Table 3.A.4 Tokelau Poultry [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] (Part 3 of 3)	168

CRF Table 3.B.1.3 Tokelau Swine: [3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] (Part 1 of 3)	169

CRF Table 3.B.1.3 Tokelau Swine: [3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] (Part 2 of 3)	169

CRF Table 3.B.1.3 Tokelau Swine: [3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] (Part 3 of 3)	170

CRF Table 3.B.1.4 Tokelau Poultry: [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] (Part 1 of 3)	171

CRF Table 3.B.1.4 Tokelau Poultry: [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] (Part 2 of 3)	172

CRF Table 3.B.1.4 Tokelau Poultry: [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] (Part 3 of 3)	172

CRF Table 5.A: [5. Waste][5.A Solid Waste Disposal] (Part 1 of 3)	173

CRF Table 5.A: [5. Waste][5.A Solid Waste Disposal] (Part 2 of 3)	174

CRF Table 5.A: [5. Waste][5.A Solid Waste Disposal] (Part 3 of 3)	174

CRF Table 5.A.3: [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] (Part 1 of 3)	175

CRF Table 5.A.3: [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] (Part 2 of 3)	176

CRF Table 5.A.3: [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] (Part 3 of 3)	177

CRF Table 5.C.2.2.a: [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic][5.C.2.2.a Municipal Solid Waste] (Part 1 of 3)	178

CRF Table 5.C.2.2.a: [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic][5.C.2.2.a Municipal Solid Waste] (Part 2 of 3)	179

CRF Table 5.C.2.2.a: [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic][5.C.2.2.a Municipal Solid Waste] (Part 3 of 3)	180

CRF Table 5.D: 	[5. Waste][5.D Wastewater Treatment and Discharge] (Part 1 of 3)	181

CRF Table 5.D: 	[5. Waste][5.D Wastewater Treatment and Discharge] (Part 2 of 3)	182

CRF Table 5.D: 	[5. Waste][5.D Wastewater Treatment and Discharge] (Part 3 of 3)	183

CRF Table 5.D.1: 	[5. Waste][5.D Wastewater Treatment and Discharge] [5.D.1 Domestic Wastewater] (Part 1 of 3)	184

CRF Table 5.D.1: 	[5. Waste][5.D Wastewater Treatment and Discharge] [5.D.1 Domestic Wastewater] (Part 2 of 3)	185

CRF Table 5.D.1: 	[5. Waste][5.D Wastewater Treatment and Discharge] [5.D.1 Domestic Wastewater] (Part 3 of 3)	186

Table A8.1.1 	Direct and indirect emissions by fertiliser in 2019	187

Table A8.1.2 	Use of fertilisers in New Zealand in 1990 and 2019	188

Table A8.1.3 	Emissions from fertilisers in 1990 and 2019	189

Table A8.2.1	Total emissions by livestock type in 1990, 2018 and 2019	189

# Figures

Figure A1.1.1 	Decision tree to identify key source categories (figure 4.2, IPCC, 2006)	1

Figure A3.1.1	Proportion of urine nitrogen (N) applied to low (0–12 degree) slopes using nutrient transfer model (equal proportion line shown in grey for comparison)	40

Figure A3.1.2	Proportion of urine nitrogen (N) applied to steep (more than 24 degree) slopes using nutrient transfer model (equal proportion line shown in grey for comparison)	40

Figure A3.1.3	Proportion of land area, excretal nitrogen (N) and nitrous oxide (N2O) emissions by hill slope category for sheep, beef cattle and deer farms in 2019	42

Figure A3.2.1	New Zealand’s LUCAS data management system	63

Figure A3.2.2	New Zealand’s Geospatial System components	64

Figure A3.2.3 	LUCAS Management Studio	65

Figure A3.2.4	LUCAS Gateway database	66

Figure A3.2.5	Quality control procedure for implementing improvements	68

Figure A4.1 	Reference and sectoral approach carbon dioxide by fuel type (kt CO2)	82

Figure A5.1.1	Technical corrections to the FMRL by category	93

Figure A5.1.2	Comparison of the 2011 FMRL and total of technical corrections over the period to 2020	94

Figure A6.1.1 	How the quality assurance and quality control processes and products align with the preparation of the inventory	101

Figure A6.1.2 	Data processing, quality assurance and quality control processes during the inventory preparation of the data from Tokelau into New Zealand’s inventory	102

# Annex 1: Key categories

## A1.1 	Methodology used for identifying key categories

The key categories in the inventory have been assessed using the Approach 1 level and trend methodologies from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2006). The methodology applied was determined using the decision tree shown in figure A1.1.1. Because some categories in the inventory apply default uncertainties values for emission estimates, and developing country-specific uncertainty values is resource prohibitive, Approach 1 level and trend methodologies are used.

Figure A1.1.1 	Decision tree to identify key source categories (figure 4.2, IPCC, 2006)

For this inventory submission, the Approach 1 level and trend assessments were applied, including and excluding the Land Use, Land-Use Change and Forestry (LULUCF) sector (IPCC, 2006).

The level and trend assessments are calculated as per equations 4.1, 4.2 and 4.3 of the IPCC 2006 Guidelines (IPCC, 2006). Key categories are defined as those categories whose cumulative percentages, when summed in decreasing order of magnitude, contributed 95 per cent of the total level or trend.

A minor error in the formulas used to calculate trend key categories was identified after the 2020 submission and corrected for the 2021 submission. The formulas are now consistent with equations 4.2 and 4.3, volume 1 of the IPCC 2006 guidelines (IPCC, 2006). This correction made only small changes to the categories identified as trend key categories.

## A1.2 	Disaggregation

The classification of categories follows the classification of the common reporting format (CRF) tables by:

identifying categories using carbon dioxide equivalent emissions and considering each greenhouse gas from each category separately

either including or excluding LULUCF categories at the level shown in the IPCC 2006 guidelines table 4.1 (IPCC, 2006).

The level of aggregation used for the key category analysis is similar to the default aggregation used for the key category analysis within the CRF tables, with adjustments to better reflect New Zealand’s emissions profile. Specifically, a large proportion of emissions from the Energy and Agriculture sectors are disaggregated further than the key category analysis generated in the CRF tables, to allow for a more evenly proportioned analysis of categories.

## A1.3 	Tables 4.2–4.3 of the IPCC 2006 Guidelines (General Guidance and Reporting)

The following tables specify the level analyses for 2019 and 1990, and trend analyses, each including and excluding LULUCF. The tables show the categories that comprise 99 per cent of emissions for each analysis. Only the categories that comprise the top 95 per cent of emissions for the 2019 level analysis and the trend analysis are key categories. The 1990 level analysis tables are included for information only.

Table A1.3.1(a)	Results of the key category level analysis for 99 per cent of the net emissions
and removals for New Zealand in 2019

| IPCC Tier 1 category level assessment – including LULUCF (net emissions): 2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 2019 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 4.A.1 | Forest Land – Forest Land Remaining Forest Land | CO2 | –17,544.5 | 14.7 | 14.7 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 14,559.4 | 12.2 | 26.8 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 14,013.6 | 11.7 | 38.6 |
| 4.G | Land Use, Land-Use Change and Forestry – Harvested Wood Products | CO2 | –10,067.3 | 8.4 | 47.0 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 8,527.2 | 7.1 | 54.1 |
| 3.A.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 5,893.7 | 4.9 | 59.0 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | CO2 | –4,736.0 | 4.0 | 63.0 |
| 3.D.1.3 | Direct N2O Emissions From Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 3,899.3 | 3.3 | 66.3 |
| 4.C.2 | Grassland – Land Converted to Grassland | CO2 | 2,953.0 | 2.5 | 68.7 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 2,692.9 | 2.3 | 71.0 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 2,533.5 | 2.1 | 73.1 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 1,859.2 | 1.6 | 74.6 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 1,751.6 | 1.5 | 76.1 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,661.6 | 1.4 | 77.5 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air conditioning | HFCs | 1,637.7 | 1.4 | 78.9 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 1,634.8 | 1.4 | 80.2 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilizers | N2O | 1,429.3 | 1.2 | 81.4 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 1,384.2 | 1.2 | 82.6 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CO2 | 1,228.2 | 1.0 | 83.6 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 1,197.6 | 1.0 | 84.6 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,154.3 | 1.0 | 85.6 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 989.6 | 0.8 | 86.4 |
| 3.D.2.1 | Indirect N2O Emissions From Managed Soils – Atmospheric Deposition | N2O | 925.3 | 0.8 | 87.2 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and quarrying Liquid Fuels | CO2 | 834.6 | 0.7 | 87.9 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 663.7 | 0.6 | 88.4 |
| 3.D.1.6 | Direct N2O Emissions from Managed Soils – Cultivation of Organic Soils | N2O | 653.0 | 0.5 | 89.0 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 587.4 | 0.5 | 89.5 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 585.1 | 0.5 | 90.0 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 574.5 | 0.5 | 90.4 |
| 3.H | Agriculture – Urea Application | CO2 | 570.7 | 0.5 | 90.9 |
| 3.G | Agriculture – Liming | CO2 | 546.1 | 0.5 | 91.4 |
| 3.D.2.2 | Indirect N2O Emissions From Managed Soils – Nitrogen Leaching and Run-off | N2O | 513.8 | 0.4 | 91.8 |
| 3.A.4 | Other livestock – Deer | CH4 | 492.9 | 0.4 | 92.2 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 461.7 | 0.4 | 92.6 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 410.9 | 0.3 | 93.0 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 383.7 | 0.3 | 93.3 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 369.6 | 0.3 | 93.6 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 354.0 | 0.3 | 93.9 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 329.1 | 0.3 | 94.2 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 315.7 | 0.3 | 94.4 |
| 4.B.1 | Cropland – Cropland Remaining Cropland | CO2 | 315.6 | 0.3 | 94.7 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 287.7 | 0.2 | 94.9 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 283.0 | 0.2 | 95.2 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 265.4 | 0.2 | 95.4 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 253.6 | 0.2 | 95.6 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 234.3 | 0.2 | 95.8 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 222.4 | 0.2 | 96.0 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 217.9 | 0.2 | 96.2 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 210.9 | 0.2 | 96.3 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 202.8 | 0.2 | 96.5 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 159.0 | 0.1 | 96.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 156.9 | 0.1 | 96.8 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 147.5 | 0.1 | 96.9 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 135.0 | 0.1 | 97.0 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 134.4 | 0.1 | 97.1 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 126.4 | 0.1 | 97.2 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 118.9 | 0.1 | 97.3 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 115.2 | 0.1 | 97.4 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 108.0 | 0.1 | 97.5 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 107.2 | 0.1 | 97.6 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 101.2 | 0.1 | 97.7 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 99.1 | 0.1 | 97.8 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 96.8 | 0.1 | 97.8 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 95.2 | 0.1 | 97.9 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 92.2 | 0.1 | 98.0 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 90.2 | 0.1 | 98.1 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 89.1 | 0.1 | 98.2 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 87.2 | 0.1 | 98.2 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 84.5 | 0.1 | 98.3 |
| 3.B.1.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 81.4 | 0.1 | 98.4 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 80.7 | 0.1 | 98.4 |
| 5.C | Waste – Incineration and Open Burning of Waste | CH4 | 77.8 | 0.1 | 98.5 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilizers | N2O | 76.9 | 0.1 | 98.6 |
| 4.E.1 | Settlements – Settlements Remaining Settlements | CO2 | 75.7 | 0.1 | 98.6 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 73.0 | 0.1 | 98.7 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 72.7 | 0.1 | 98.7 |
| 4.B.2 | Cropland – Land Converted to Cropland | CO2 | 63.8 | 0.1 | 98.8 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 62.6 | 0.1 | 98.8 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 58.2 | 0.0 | 98.9 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | N2O | 58.0 | 0.0 | 98.9 |
| 4.F.2 | Other Land – Land Converted to Other Land | CO2 | 48.6 | 0.0 | 99.0 |
| 2.D | Industrial Processes and Product Use – Non-energy Products from Fuels and Solvent Use | CO2 | 48.0 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total. Removals from the LULUCF sector are shown as negatives in this table. The absolute values for those removals were used for the calculations.

Table A1.3.1(b)	Results of the key category level analysis for 99 per cent of the gross emissions
and removals for New Zealand in 2019

| IPCC Tier 1 category level assessment – gross emissions (excluding LULUCF): 2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC Category | Gas | 2019 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 14,559.4 | 17.7 | 17.7 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 14,013.6 | 17.0 | 34.7 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 8,527.2 | 10.4 | 45.1 |
| 3.A.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 5,893.7 | 7.2 | 52.2 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 3,899.3 | 4.7 | 57.0 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 2,692.9 | 3.3 | 60.2 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 2,533.5 | 3.1 | 63.3 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 1,859.2 | 2.3 | 65.6 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 1,751.6 | 2.1 | 67.7 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,661.6 | 2.0 | 69.7 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air conditioning | HFCs | 1,637.7 | 2.0 | 71.7 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 1,634.8 | 2.0 | 73.7 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilizers | N2O | 1,429.3 | 1.7 | 75.4 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 1,384.2 | 1.7 | 77.1 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 1,197.6 | 1.5 | 78.6 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,154.3 | 1.4 | 80.0 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 989.6 | 1.2 | 81.2 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 925.3 | 1.1 | 82.3 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and quarrying Liquid Fuels | CO2 | 834.6 | 1.0 | 83.3 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 663.7 | 0.8 | 84.1 |
| 3.D.1.6 | Direct N2O Emissions from Managed Soils – Cultivation of Organic Soils | N2O | 653.0 | 0.8 | 84.9 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 587.4 | 0.7 | 85.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 585.1 | 0.7 | 86.3 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 574.5 | 0.7 | 87.0 |
| 3.H | Agriculture – Urea Application | CO2 | 570.7 | 0.7 | 87.7 |
| 3.G | Agriculture – Liming | CO2 | 546.1 | 0.7 | 88.4 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 513.8 | 0.6 | 89.0 |
| 3.A.4 | Other livestock – Deer | CH4 | 492.9 | 0.6 | 89.6 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 461.7 | 0.6 | 90.2 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 410.9 | 0.5 | 90.7 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 383.7 | 0.5 | 91.1 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 369.6 | 0.4 | 91.6 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 354.0 | 0.4 | 92.0 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 329.1 | 0.4 | 92.4 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 315.7 | 0.4 | 92.8 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 287.7 | 0.3 | 93.2 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 283.0 | 0.3 | 93.5 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 265.4 | 0.3 | 93.8 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 253.6 | 0.3 | 94.1 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 234.3 | 0.3 | 94.4 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 222.4 | 0.3 | 94.7 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 217.9 | 0.3 | 94.9 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 210.9 | 0.3 | 95.2 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 202.8 | 0.2 | 95.4 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 159.0 | 0.2 | 95.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 156.9 | 0.2 | 95.8 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 147.5 | 0.2 | 96.0 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 135.0 | 0.2 | 96.2 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 134.4 | 0.2 | 96.3 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 126.4 | 0.2 | 96.5 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 118.9 | 0.1 | 96.6 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 115.2 | 0.1 | 96.8 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 108.0 | 0.1 | 96.9 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 107.2 | 0.1 | 97.0 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 101.2 | 0.1 | 97.2 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 99.1 | 0.1 | 97.3 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 96.8 | 0.1 | 97.4 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 95.2 | 0.1 | 97.5 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 92.2 | 0.1 | 97.6 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 90.2 | 0.1 | 97.7 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 89.1 | 0.1 | 97.8 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 87.2 | 0.1 | 97.9 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 84.5 | 0.1 | 98.1 |
| 3.B.1.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 81.4 | 0.1 | 98.2 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 80.7 | 0.1 | 98.2 |
| 5.C | Waste – Incineration and Open Burning of Waste | CH4 | 77.8 | 0.1 | 98.3 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilizers | N2O | 76.9 | 0.1 | 98.4 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 73.0 | 0.1 | 98.5 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 72.7 | 0.1 | 98.6 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 62.6 | 0.1 | 98.7 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 58.2 | 0.1 | 98.8 |
| 2.D | Industrial Processes and Product Use – Non-energy Products from Fuels and Solvent Use | CO2 | 48.0 | 0.1 | 98.8 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 47.2 | 0.1 | 98.9 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Biomass | N2O | 45.9 | 0.1 | 98.9 |
| 1.A.4.b | Other Sectors – Residential Biomass | CH4 | 45.2 | 0.1 | 99.0 |
| 1.A.3.e | Transport – Other Transportation (please specify) Gaseous Fuels | CO2 | 39.8 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total.

Table A1.3.2(a)	Results of the level analysis for 99 per cent of the net emissions and removals
for New Zealand in 1990 included for reference only

| IPCC Tier 1 category level assessment – including LULUCF (net emissions): 1990 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 1990 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 4.A.2 | Forest Land – Land Converted to Forest Land | CO2 | –18,516.4 | 20.1 | 20.1 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,557.9 | 15.8 | 36.0 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 7.8 | 43.8 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 6,147.3 | 6.7 | 50.4 |
| 3.A.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 5,950.0 | 6.5 | 56.9 |
| 4.A.1 | Forest Land – Forest Land Remaining Forest Land | CO2 | –4,808.9 | 5.2 | 62.1 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,337.8 | 3.6 | 65.8 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 3,068.6 | 3.3 | 69.1 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 3.3 | 72.4 |
| 4.G | Land Use, Land-Use Change and Forestry – Harvested Wood Products | CO2 | –2,072.9 | 2.3 | 74.6 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,761.0 | 1.9 | 76.6 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 1.4 | 78.0 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1.2 | 79.1 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1.0 | 80.2 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 1.0 | 81.2 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 1.0 | 82.1 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 0.8 | 83.0 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 735.1 | 0.8 | 83.8 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 0.8 | 84.6 |
| 3.D.1.6 | Direct N2O Emissions from Managed Soils – Cultivation of Organic Soils | N2O | 643.8 | 0.7 | 85.3 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 0.6 | 85.8 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 500.6 | 0.5 | 86.4 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 0.5 | 86.9 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 449.0 | 0.5 | 87.4 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 0.5 | 87.9 |
| 3.A.4 | Other livestock – Deer | CH4 | 445.5 | 0.5 | 88.4 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 0.5 | 88.8 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 416.6 | 0.5 | 89.3 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 395.2 | 0.4 | 89.7 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 0.4 | 90.1 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 0.4 | 90.5 |
| 4.B.1 | Cropland – Cropland Remaining Cropland | CO2 | 351.1 | 0.4 | 90.9 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 0.4 | 91.3 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 0.4 | 91.7 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and quarrying Liquid Fuels | CO2 | 331.5 | 0.4 | 92.0 |
| 4.C.2 | Grassland – Land Converted to Grassland | CO2 | 311.5 | 0.3 | 92.4 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 0.3 | 92.7 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 0.3 | 93.0 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 269.4 | 0.3 | 93.3 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 236.1 | 0.3 | 93.5 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 0.3 | 93.8 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilizers | N2O | 230.3 | 0.3 | 94.0 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 0.2 | 94.3 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 221.2 | 0.2 | 94.5 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CO2 | 219.9 | 0.2 | 94.8 |
| 3.A.4 | Other Livestock – Goats | CH4 | 196.6 | 0.2 | 95.0 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 0.2 | 95.2 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 0.2 | 95.4 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 0.2 | 95.6 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 158.8 | 0.2 | 95.7 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 152.3 | 0.2 | 95.9 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 148.8 | 0.2 | 96.1 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 142.2 | 0.2 | 96.2 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.2 | 96.4 |
| 5.C | Waste – Incineration and Open Burning of Waste | CH4 | 127.3 | 0.1 | 96.5 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | N2O | 124.3 | 0.1 | 96.6 |
| 4.B.2 | Cropland – Land Converted to Cropland | CO2 | 117.3 | 0.1 | 96.8 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 116.6 | 0.1 | 96.9 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 114.4 | 0.1 | 97.0 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 0.1 | 97.1 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 0.1 | 97.3 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 0.1 | 97.4 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 0.1 | 97.5 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 89.3 | 0.1 | 97.6 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 82.6 | 0.1 | 97.7 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 82.1 | 0.1 | 97.8 |
| 3.B.1.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 82.0 | 0.1 | 97.9 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 78.4 | 0.1 | 97.9 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CH4 | 74.2 | 0.1 | 98.0 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 0.1 | 98.1 |
| 4.E.1 | Settlements – Settlements Remaining Settlements | CO2 | 67.2 | 0.1 | 98.2 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 0.1 | 98.2 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 0.1 | 98.3 |
| 1.A.2.g.vi | Other (please specify) – Textile and leather Gaseous Fuels | CO2 | 59.2 | 0.1 | 98.4 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 0.1 | 98.4 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 0.1 | 98.5 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.7 | 0.1 | 98.6 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 49.1 | 0.1 | 98.6 |
| 1.A.4.b | Other Sectors – Residential Biomass | CH4 | 48.4 | 0.1 | 98.7 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.4 | 0.1 | 98.7 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 0.1 | 98.8 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 47.7 | 0.1 | 98.8 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | N2O | 43.2 | 0.0 | 98.9 |
| 3.A.4 | Other Livestock – Horses | CH4 | 42.3 | 0.0 | 98.9 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of machinery Gaseous Fuels | CO2 | 41.9 | 0.0 | 99.0 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 0.0 | 99.0 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 38.7 | 0.0 | 99.0 |

Note:	Removals from the LULUCF sector are shown as negatives in this table. The absolute values for those removals were used for the calculations.

Table A1.3.2(b)	Results of the level analysis for 99 per cent of the gross emissions for New Zealand
in 1990 included for reference only

| IPCC Tier 1 category level assessment – gross emissions (excluding LULUCF): 1990 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 1990 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,557.9 | 22.4 | 22.4 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 11.0 | 33.4 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 6,147.3 | 9.4 | 42.8 |
| 3.A.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 5,950.0 | 9.1 | 51.9 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,337.8 | 5.1 | 57.1 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 3,068.6 | 4.7 | 61.8 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 4.6 | 66.4 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,761.0 | 2.7 | 69.1 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 2.0 | 71.1 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1.6 | 72.7 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1.4 | 74.2 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 1.4 | 75.6 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 1.4 | 77.0 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 1.2 | 78.1 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 735.1 | 1.1 | 79.3 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 1.1 | 80.4 |
| 3.D.1.6 | Direct N2O Emissions from Managed Soils – Cultivation of Organic Soils | N2O | 643.8 | 1.0 | 81.4 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 0.8 | 82.2 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 500.6 | 0.8 | 83.0 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 0.7 | 83.7 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 449.0 | 0.7 | 84.4 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 0.7 | 85.1 |
| 3.A.4 | Other Livestock – Deer | CH4 | 445.5 | 0.7 | 85.8 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 0.7 | 86.4 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 416.6 | 0.6 | 87.1 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 395.2 | 0.6 | 87.7 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 0.6 | 88.3 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 0.6 | 88.8 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 0.5 | 89.4 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 0.5 | 89.9 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and quarrying Liquid Fuels | CO2 | 331.5 | 0.5 | 90.4 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 0.4 | 90.9 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 0.4 | 91.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 269.4 | 0.4 | 91.7 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 236.1 | 0.4 | 92.1 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 0.4 | 92.4 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilizers | N2O | 230.3 | 0.4 | 92.8 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 0.4 | 93.1 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 221.2 | 0.3 | 93.5 |
| 3.A.4 | Other livestock – Goats | CH4 | 196.6 | 0.3 | 93.8 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 0.3 | 94.0 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 0.3 | 94.3 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 0.3 | 94.6 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 158.8 | 0.2 | 94.8 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 152.3 | 0.2 | 95.0 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 148.8 | 0.2 | 95.3 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 142.2 | 0.2 | 95.5 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.2 | 95.7 |
| 5.C | Waste – Incineration and Open Burning of Waste | CH4 | 127.3 | 0.2 | 95.9 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 116.6 | 0.2 | 96.1 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 114.4 | 0.2 | 96.3 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 0.2 | 96.4 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 0.2 | 96.6 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 0.2 | 96.8 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 0.2 | 96.9 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 89.3 | 0.1 | 97.1 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 82.6 | 0.1 | 97.2 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 82.1 | 0.1 | 97.3 |
| 3.B.1.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 82.0 | 0.1 | 97.4 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 78.4 | 0.1 | 97.6 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 0.1 | 97.7 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 0.1 | 97.8 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 0.1 | 97.9 |
| 1.A.2.g.vi | Other (please specify) – Textile and leather Gaseous Fuels | CO2 | 59.2 | 0.1 | 98.0 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 0.1 | 98.0 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 0.1 | 98.1 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.7 | 0.1 | 98.2 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 49.1 | 0.1 | 98.3 |
| 1.A.4.b | Other Sectors – Residential Biomass | CH4 | 48.4 | 0.1 | 98.4 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.4 | 0.1 | 98.4 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 0.1 | 98.5 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 47.7 | 0.1 | 98.6 |
| 3.A.4 | Other livestock – Horses | CH4 | 42.3 | 0.1 | 98.6 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of machinery Gaseous Fuels | CO2 | 41.9 | 0.1 | 98.7 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 0.1 | 98.8 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 38.7 | 0.1 | 98.8 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilizers | N2O | 36.3 | 0.1 | 98.9 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 35.1 | 0.1 | 98.9 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 34.7 | 0.1 | 99.0 |
| 1.A.2.b | Manufacturing Industries and Construction – Non-Ferrous Metals Liquid Fuels | CO2 | 32.5 | 0.0 | 99.0 |

Table A1.3.3(a)	Results of the key category trend analysis for 99 per cent of the net emissions and removals
for New Zealand in 1990–2019

| IPCC Tier 1 category trend assessment – including LULUCF (net emissions) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 1990 estimate (kt CO2-e) | 2019 estimate (kt CO2-e) | Trend assessment | Contribution to trend (%) | Cumulative total (%) |
| 4.A.1 | Forest Land – Forest Land Remaining Forest Land | CO2 | –4,808.9 | –17,544.5 | 0.156 | 17.8 | 17.8 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,557.9 | 8,527.2 | 0.119 | 13.5 | 31.3 |
| 4.G | Land Use, Land-Use Change and Forestry – Harvested Wood Products | CO2 | –2,072.9 | –10,067.3 | 0.094 | 10.8 | 42.1 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | CO2 | –18,516.4 | –4,736.0 | 0.082 | 9.4 | 51.5 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 6,147.3 | 14,013.6 | 0.063 | 7.2 | 58.7 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 14,559.4 | 0.054 | 6.2 | 64.9 |
| 4.C.2 | Grassland – Land Converted to Grassland | CO2 | 311.5 | 2,953.0 | 0.028 | 3.1 | 68.0 |
| 3.A.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 5,950.0 | 5,893.7 | 0.022 | 2.5 | 70.6 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,761.0 | 354.0 | 0.022 | 2.5 | 73.0 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,337.8 | 2,692.9 | 0.019 | 2.2 | 75.2 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air conditioning | HFCs | 0.0 | 1,637.7 | 0.018 | 2.0 | 77.2 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 2,533.5 | 0.016 | 1.8 | 79.1 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 1,859.2 | 0.013 | 1.4 | 80.5 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 89.1 | 0.012 | 1.4 | 81.9 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilizers | N2O | 230.3 | 1,429.3 | 0.012 | 1.4 | 83.3 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 1,634.8 | 0.011 | 1.2 | 84.6 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 9.5 | 0.011 | 1.2 | 85.7 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CO2 | 219.9 | 1,228.2 | 0.010 | 1.2 | 86.9 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 416.6 | 1,384.2 | 0.009 | 1.0 | 87.9 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 1,197.6 | 0.007 | 0.7 | 88.7 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 570.7 | 0.006 | 0.6 | 89.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1,751.6 | 0.005 | 0.6 | 89.9 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 24.6 | 0.005 | 0.5 | 90.5 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and quarrying Liquid Fuels | CO2 | 331.5 | 834.6 | 0.004 | 0.5 | 91.0 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 663.7 | 0.004 | 0.5 | 91.4 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 72.7 | 0.003 | 0.4 | 91.8 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 585.1 | 0.003 | 0.3 | 92.2 |
| 1.A.4.c | Other Sectors – Agriculture/ Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1,154.3 | 0.003 | 0.3 | 92.5 |
| 3.A.4 | Other livestock – Goats | CH4 | 196.6 | 21.0 | 0.003 | 0.3 | 92.8 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 0.0 | 217.9 | 0.002 | 0.3 | 93.1 |
| 3.D.1.6 | Direct N2O Emissions from Managed Soils – Cultivation of Organic Soils | N2O | 643.8 | 653.0 | 0.002 | 0.3 | 93.3 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 989.6 | 0.002 | 0.3 | 93.6 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 3,068.6 | 3,899.3 | 0.002 | 0.2 | 93.8 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 410.9 | 0.002 | 0.2 | 94.1 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.0 | 0.002 | 0.2 | 94.3 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 210.9 | 0.002 | 0.2 | 94.5 |
| 1.A.4.c | Other Sectors – Agriculture/ Forestry/Fishing Solid Fuels | CO2 | 35.1 | 202.8 | 0.002 | 0.2 | 94.7 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.7 | 222.4 | 0.002 | 0.2 | 94.9 |
| 4.B.1 | Cropland – Cropland Remaining Cropland | CO2 | 351.1 | 315.6 | 0.002 | 0.2 | 95.1 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 315.7 | 0.002 | 0.2 | 95.3 |
| 1.A.4.a | Other Sectors – Commercial/ Institutional Gaseous Fuels | CO2 | 236.1 | 461.7 | 0.002 | 0.2 | 95.4 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 283.0 | 0.001 | 0.2 | 95.6 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 383.7 | 0.001 | 0.2 | 95.8 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 369.6 | 0.001 | 0.2 | 95.9 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 158.8 | 90.2 | 0.001 | 0.2 | 96.1 |
| 1.A.4.a | Other Sectors – Commercial/ Institutional Solid Fuels | CO2 | 142.2 | 80.7 | 0.001 | 0.1 | 96.2 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | N2O | 124.3 | 58.0 | 0.001 | 0.1 | 96.3 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 148.8 | 95.2 | 0.001 | 0.1 | 96.5 |
| 3.A.4 | Other livestock – Deer | CH4 | 445.5 | 492.9 | 0.001 | 0.1 | 96.6 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 47.2 | 0.001 | 0.1 | 96.7 |
| 4.B.2 | Cropland – Land Converted to Cropland | CO2 | 117.3 | 63.8 | 0.001 | 0.1 | 96.8 |
| 5.C | Waste – Incineration and Open Burning of Waste | CH4 | 127.3 | 77.8 | 0.001 | 0.1 | 96.9 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 0.0 | 87.2 | 0.001 | 0.1 | 97.1 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 156.9 | 0.001 | 0.1 | 97.2 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 1,661.6 | 0.001 | 0.1 | 97.3 |
| 1.A.4.a | Other Sectors – Commercial/ Institutional Liquid Fuels | CO2 | 500.6 | 587.4 | 0.001 | 0.1 | 97.4 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 18.2 | 0.001 | 0.1 | 97.5 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 269.4 | 287.7 | 0.001 | 0.1 | 97.6 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 47.7 | 135.0 | 0.001 | 0.1 | 97.6 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 27.6 | 107.2 | 0.001 | 0.1 | 97.7 |
| 1.A.4.c | Other Sectors – Agriculture/ Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 73.0 | 0.001 | 0.1 | 97.8 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 546.1 | 0.001 | 0.1 | 97.9 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CH4 | 74.2 | 35.5 | 0.001 | 0.1 | 98.0 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 30.5 | 99.1 | 0.001 | 0.1 | 98.0 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 29.6 | 0.001 | 0.1 | 98.1 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 735.1 | 925.3 | 0.001 | 0.1 | 98.2 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 34.7 | 101.2 | 0.001 | 0.1 | 98.3 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 84.5 | 0.001 | 0.1 | 98.3 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 152.3 | 159.0 | 0.000 | 0.1 | 98.4 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of machinery Gaseous Fuels | CO2 | 41.9 | 13.5 | 0.000 | 0.1 | 98.4 |
| 1.A.2.g.vi | Other (please specify) – Textile and leather Gaseous Fuels | CO2 | 59.2 | 37.1 | 0.000 | 0.1 | 98.5 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 221.2 | 253.6 | 0.000 | 0.1 | 98.5 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 38.0 | 0.000 | 0.0 | 98.6 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | N2O | 43.2 | 17.7 | 0.000 | 0.0 | 98.6 |
| 3.A.4 | Other livestock – Horses | CH4 | 42.3 | 17.3 | 0.000 | 0.0 | 98.7 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 25.6 | 0.000 | 0.0 | 98.7 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CH4 | 27.3 | 1.9 | 0.000 | 0.0 | 98.8 |
| 5.B | Waste – Biological Treatment of Solid Waste | CH4 | 2.7 | 38.0 | 0.000 | 0.0 | 98.8 |
| 1.A.3.e | Transport – Other Transportation (please specify) Gaseous Fuels | CO2 | 5.5 | 39.8 | 0.000 | 0.0 | 98.8 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.4 | 96.8 | 0.000 | 0.0 | 98.9 |
| 4.F.2 | Other Land – Land Converted to Other Land | CO2 | 12.6 | 48.6 | 0.000 | 0.0 | 98.9 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 265.4 | 0.000 | 0.0 | 99.0 |
| 4.C.2 | Grassland – Land Converted to Grassland | N2O | 28.5 | 8.4 | 0.000 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total. Removals from the LULUCF sector are shown as negatives in this table. The absolute values for those removals were used for the calculations.

Table A1.3.3(b)	Results of the key category trend analysis for 99 per cent of the gross emissions
for New Zealand in 1990–2019

| IPCC Tier 1 category trend assessment – gross emissions (excluding LULUCF) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRF Category code | IPCC Category | Gas | 1990 estimate (kt CO2-e) | 2019 estimate (kt CO2-e) | Trend assessment | Contribution to trend (%) | Cumulative total (%) |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,557.9 | 8,527.2 | 0.152 | 22.3 | 22.3 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 6,147.3 | 14,013.6 | 0.096 | 14.1 | 36.5 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 14,559.4 | 0.085 | 12.5 | 48.9 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,761.0 | 354.0 | 0.029 | 4.2 | 53.2 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air conditioning | HFCs | 0.0 | 1,637.7 | 0.025 | 3.7 | 56.9 |
| 3.A.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 5,950.0 | 5,893.7 | 0.025 | 3.7 | 60.6 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,337.8 | 2,692.9 | 0.023 | 3.5 | 64.0 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 2,533.5 | 0.020 | 2.9 | 66.9 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 1,859.2 | 0.018 | 2.7 | 69.6 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilizers | N2O | 230.3 | 1,429.3 | 0.017 | 2.6 | 72.2 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 89.1 | 0.016 | 2.4 | 74.6 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 1,634.8 | 0.016 | 2.3 | 76.9 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 9.5 | 0.014 | 2.1 | 79.0 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 416.6 | 1,384.2 | 0.013 | 1.9 | 80.9 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 1,197.6 | 0.010 | 1.4 | 82.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1,751.6 | 0.009 | 1.3 | 83.6 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 570.7 | 0.008 | 1.2 | 84.8 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 331.5 | 834.6 | 0.006 | 0.9 | 85.7 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 24.6 | 0.006 | 0.9 | 86.7 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 663.7 | 0.005 | 0.7 | 87.4 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 585.1 | 0.005 | 0.7 | 88.1 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 72.7 | 0.005 | 0.7 | 88.7 |
| 3.A.4 | Other livestock – Goats | CH4 | 196.6 | 21.0 | 0.003 | 0.5 | 89.3 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 0.0 | 217.9 | 0.003 | 0.5 | 89.7 |
| 1.A.4.c | Other Sectors – Agriculture/ Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1,154.3 | 0.003 | 0.5 | 90.2 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.0 | 0.003 | 0.4 | 90.6 |
| 1.A.4.a | Other Sectors – Commercial/ Institutional Gaseous Fuels | CO2 | 236.1 | 461.7 | 0.003 | 0.4 | 91.0 |
| 3.D.1.6 | Direct N2O Emissions from Managed Soils – Cultivation of Organic Soils | N2O | 643.8 | 653.0 | 0.002 | 0.4 | 91.3 |
| 1.A.4.c | Other Sectors – Agriculture/ Forestry/Fishing Solid Fuels | CO2 | 35.1 | 202.8 | 0.002 | 0.4 | 91.7 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.7 | 222.4 | 0.002 | 0.4 | 92.1 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 410.9 | 0.002 | 0.4 | 92.4 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 283.0 | 0.002 | 0.3 | 92.7 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 210.9 | 0.002 | 0.3 | 93.0 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 989.6 | 0.002 | 0.3 | 93.4 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 369.6 | 0.002 | 0.3 | 93.7 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 315.7 | 0.002 | 0.3 | 94.0 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 158.8 | 90.2 | 0.002 | 0.3 | 94.2 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 383.7 | 0.002 | 0.2 | 94.4 |
| 1.A.4.a | Other Sectors – Commercial/ Institutional Solid Fuels | CO2 | 142.2 | 80.7 | 0.002 | 0.2 | 94.7 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 148.8 | 95.2 | 0.001 | 0.2 | 94.9 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 47.2 | 0.001 | 0.2 | 95.1 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 546.1 | 0.001 | 0.2 | 95.3 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 156.9 | 0.001 | 0.2 | 95.5 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 0.0 | 87.2 | 0.001 | 0.2 | 95.7 |
| 5.C | Waste – Incineration and Open Burning of Waste | CH4 | 127.3 | 77.8 | 0.001 | 0.2 | 95.9 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 47.7 | 135.0 | 0.001 | 0.2 | 96.0 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 18.2 | 0.001 | 0.2 | 96.2 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 27.6 | 107.2 | 0.001 | 0.2 | 96.4 |
| 3.A.4 | Other livestock – Deer | CH4 | 445.5 | 492.9 | 0.001 | 0.2 | 96.5 |
| 1.A.4.c | Other Sectors – Agriculture/ Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 73.0 | 0.001 | 0.1 | 96.7 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 30.5 | 99.1 | 0.001 | 0.1 | 96.8 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 34.7 | 101.2 | 0.001 | 0.1 | 96.9 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 269.4 | 287.7 | 0.001 | 0.1 | 97.0 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 29.6 | 0.001 | 0.1 | 97.2 |
| 1.A.4.a | Other Sectors – Commercial/ Institutional Liquid Fuels | CO2 | 500.6 | 587.4 | 0.001 | 0.1 | 97.3 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 84.5 | 0.001 | 0.1 | 97.4 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 265.4 | 0.001 | 0.1 | 97.5 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of Machinery Gaseous Fuels | CO2 | 41.9 | 13.5 | 0.001 | 0.1 | 97.6 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Gaseous Fuels | CO2 | 59.2 | 37.1 | 0.001 | 0.1 | 97.6 |
| 3.A.4 | Other Livestock – Horses | CH4 | 42.3 | 17.3 | 0.001 | 0.1 | 97.7 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 38.0 | 0.001 | 0.1 | 97.8 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.4 | 96.8 | 0.001 | 0.1 | 97.9 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 329.1 | 0.001 | 0.1 | 98.0 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 25.6 | 0.001 | 0.1 | 98.0 |
| 5.B | Waste – Biological Treatment of Solid Waste | CH4 | 2.7 | 38.0 | 0.001 | 0.1 | 98.1 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 115.2 | 0.001 | 0.1 | 98.2 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 152.3 | 159.0 | 0.001 | 0.1 | 98.3 |
| 1.A.3.e | Transport – Other Transportation (please specify) Gaseous Fuels | CO2 | 5.5 | 39.8 | 0.001 | 0.1 | 98.3 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CH4 | 27.3 | 1.9 | 0.001 | 0.1 | 98.4 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilizers | N2O | 36.3 | 76.9 | 0.000 | 0.1 | 98.5 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 78.4 | 126.4 | 0.000 | 0.1 | 98.6 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 221.2 | 253.6 | 0.000 | 0.1 | 98.6 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Solid Fuels | CO2 | 19.9 | 0.0 | 0.000 | 0.1 | 98.7 |
| 5.B | Waste – Biological Treatment of Solid Waste | N2O | 2.0 | 27.2 | 0.000 | 0.1 | 98.7 |
| 1.B.2.c.1.iii | Venting – Combined | CH4 | 1.4 | 26.0 | 0.000 | 0.1 | 98.8 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 234.3 | 0.000 | 0.1 | 98.8 |
| 3.B.1.1 | Option A – Non-Dairy (Beef) Cattle | CH4 | 82.0 | 81.4 | 0.000 | 0.1 | 98.9 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Gaseous Fuels | CO2 | 10.7 | 35.1 | 0.000 | 0.0 | 98.9 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Liquid Fuels | CO2 | 20.3 | 4.3 | 0.000 | 0.0 | 99.0 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 3,068.6 | 3,899.3 | 0.000 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total.

## Annex 1: References

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 1. General Guidance and Reporting. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

# Annex 2: Uncertainty analysis

Uncertainty estimates are an essential element of a complete emissions inventory. The purpose of uncertainty information is to help prioritise efforts to improve the accuracy of inventories in the future and guide decisions on methodological choice (IPCC, 2006).

New Zealand has followed Approach 1 for uncertainty analysis, as required by the inventory reporting guidelines under United Nations Framework Convention on Climate Change (UNFCCC, 2013) and Intergovernmental Panel on Climate Change (IPCC) methodological guidelines (IPCC, 2006). Uncertainties in the categories are combined in the uncertainty analysis to provide uncertainty estimates for the entire inventory in any year and the uncertainty in the overall inventory trend over time. Uncertainties for the categories themselves are described in the sector chapters 3-8 and chapter 11, as well as in chapter 1 section 1.6. Land Use, Land-Use Change and Forestry (LULUCF) sector categories have been included using the absolute value of any removals of carbon dioxide (table A2.1.1). Table A2.1.2 calculates the uncertainty only in emissions, that is, excluding LULUCF removals.

## A2.1 	Approach 1 uncertainty calculation

The uncertainty in activity data and emission and/or removal factors shown in tables A2.1.1 and A2.1.2 are equal to half the 95 per cent confidence interval divided by the mean and expressed as a percentage. The reason for halving the 95 per cent confidence interval is that the value corresponds to the familiar plus or minus value when uncertainties are loosely quoted as ‘plus or minus x per cent’.

Where uncertainty is highly asymmetrical, the larger percentage difference between the mean and the confidence limit is entered. Where only the total uncertainty is known for a category, then:

if uncertainty is correlated across years, the uncertainty is entered as the emission or the removal factor uncertainty and as zero in the activity data uncertainty

if uncertainty is not correlated across years, the uncertainty is entered as the uncertainty in the activity data and as zero in the emission or the removal factor uncertainty.

In Approach 1, uncertainties in the trend are estimated using two sensitivities.

Type A sensitivity is the change in the difference of total emissions between the base year and the current year, expressed as a percentage. Further, this change results from a 1 per cent increase in emissions of a given source category and a greenhouse gas in both the base year and the current year.

Type B sensitivity is the change in the difference of total emissions between the base year and the current year, expressed as a percentage. Further, this change results from a 1 per cent increase in emissions of a given source category and gas in the current year only.

Uncertainties that are fully correlated between years are associated with Type A sensitivities, and uncertainties that are not correlated between years are associated with Type B sensitivities. Once the uncertainties introduced into the national inventory by Type A and Type B sensitivities have been calculated, they are summed using equation 3.1 (IPCC, 2006) to give the overall uncertainty in the trend.

In tables A2.1.1 and A2.1.2, the figure labelled ‘Uncertainty in the trend’ is an estimate of the total uncertainty in the trend in emissions since the base year. This is expressed as the number of percentage points in the 95 per cent confidence interval in the per cent change in emissions since the base year. The values for individual categories are an estimate of the uncertainty introduced into the trend by the category in question.

Table A2.1.1 	Uncertainty calculation (including LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2019 (IPCC, 2006, Approach 1)

| IPCC source category | Gas | 1990 emissions or absolute value of removals (kt CO₂-e) | 2019 emissions or absolute value of removals (kt CO₂-e) | Activity data uncertainty (%) | Emission or removal factor uncertainty (%) | Combined uncertainty (%) | Combined uncertainty as a per cent of the national total in 1990 (%) | Combined uncertainty as a per cent of the national total in 2019 (%) | Type A sensitivity (%) | Type B sensitivity (%) | Uncertainty in the trend in national total introduced by emission or removal factor uncertainty (%) | Uncertainty in trend in national total introduced by activity data uncertainty (%) | Combined uncertainty of the national total in 1990 ((%)2) | Combined uncertainty of the national total in 2019 ((%)2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Energy – Liquid fuels | CO₂ | 11803.54 | 20235.22 | 1.5 | 0.5 | 1.5 | 0.1980 | 0.2609 | 0.0530 | 0.2201 | 0.0265 | 0.4541 | 0.0392 | 0.0681 |
| Energy – Solid fuels | CO₂ | 3211.03 | 4155.77 | 4.6 | 2.2 | 5.1 | 0.1776 | 0.1767 | 0.0002 | 0.0452 | 0.0005 | 0.2940 | 0.0316 | 0.0312 |
| Energy – Gaseous fuels | CO₂ | 7095.50 | 7787.32 | 9.3 | 2.4 | 9.6 | 0.7390 | 0.6235 | 0.0157 | 0.0847 | 0.0378 | 1.1100 | 0.5461 | 0.3887 |
| Energy – Fugitive – geothermal | CO₂ | 228.58 | 585.06 | 5.0 | 5.0 | 7.1 | 0.0176 | 0.0346 | 0.0031 | 0.0064 | 0.0156 | 0.0450 | 0.0003 | 0.0012 |
| Energy – Fugitive – venting/ flaring | CO₂ | 229.84 | 434.30 | 9.3 | 2.4 | 9.6 | 0.0239 | 0.0348 | 0.0015 | 0.0047 | 0.0035 | 0.0619 | 0.0006 | 0.0012 |
| Energy – Fugitive – oil and gas activities | CO₂ | 0.08 | 0.20 | 5.0 | 100.0 | 100.1 | 0.0001 | 0.0002 | 0.0000 | 0.0000 | 0.0001 | 0.0000 | 0.0000 | 0.0000 |
| Energy – Fugitive – transmission and distribution | CO₂ | 1.46 | 1.31 | 9.3 | 100.0 | 100.4 | 0.0016 | 0.0011 | 0.0000 | 0.0000 | 0.0007 | 0.0002 | 0.0000 | 0.0000 |
| IPPU – Cement production | CO₂ | 448.75 | 410.94 | 1.0 | 1.0 | 1.4 | 0.0069 | 0.0049 | 0.0019 | 0.0045 | 0.0019 | 0.0063 | 0.0000 | 0.0000 |
| IPPU – Lime production | CO₂ | 82.60 | 108.04 | 2.0 | 2.0 | 2.8 | 0.0025 | 0.0026 | 0.0000 | 0.0012 | 0.0000 | 0.0033 | 0.0000 | 0.0000 |
| IPPU – Ceramics | CO₂ | 0.01 | 0.01 | 50.0 | 20.0 | 53.9 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Other uses of soda ash | CO₂ | 5.87 | 6.63 | 3.0 | 2.0 | 3.6 | 0.0002 | 0.0002 | 0.0000 | 0.0001 | 0.0000 | 0.0003 | 0.0000 | 0.0000 |
| IPPU – Other uses of limestone | CO₂ | 24.63 | 92.45 | 3.0 | 2.0 | 3.6 | 0.0010 | 0.0028 | 0.0007 | 0.0010 | 0.0013 | 0.0043 | 0.0000 | 0.0000 |
| IPPU – Ammonia production | CO₂ | 21.68 | 22.83 | 2.0 | 6.0 | 6.3 | 0.0015 | 0.0012 | 0.0001 | 0.0002 | 0.0004 | 0.0007 | 0.0000 | 0.0000 |
| IPPU – Calcium carbide | CO₂ | 1.43 | 1.43 | 50.0 | 50.0 | 70.7 | 0.0011 | 0.0008 | 0.0000 | 0.0000 | 0.0002 | 0.0011 | 0.0000 | 0.0000 |
| IPPU – Hydrogen production | CO₂ | 152.29 | 159.04 | 2.0 | 6.0 | 6.3 | 0.0105 | 0.0084 | 0.0004 | 0.0017 | 0.0025 | 0.0049 | 0.0001 | 0.0001 |
| IPPU – Iron and steel | CO₂ | 1306.73 | 1661.61 | 5.0 | 7.0 | 8.6 | 0.1223 | 0.1195 | 0.0004 | 0.0181 | 0.0029 | 0.1278 | 0.0150 | 0.0143 |
| IPPU – Aluminium | CO₂ | 448.98 | 574.50 | 5.0 | 2.0 | 5.4 | 0.0263 | 0.0259 | 0.0001 | 0.0062 | 0.0002 | 0.0442 | 0.0007 | 0.0007 |
| IPPU – Secondary lead production | CO₂ | 1.80 | 0.00 | 50.0 | 50.0 | 70.7 | 0.0014 | 0.0000 | 0.0000 | 0.0000 | 0.0013 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Lubricant use | CO₂ | 22.83 | 42.53 | 20.0 | 50.0 | 53.9 | 0.0134 | 0.0192 | 0.0001 | 0.0005 | 0.0070 | 0.0131 | 0.0002 | 0.0004 |
| IPPU – Paraffin wax | CO₂ | 2.35 | 2.35 | 20.0 | 100.0 | 102.0 | 0.0026 | 0.0020 | 0.0000 | 0.0000 | 0.0008 | 0.0007 | 0.0000 | 0.0000 |
| IPPU – Other: Urea catalyst in road transport | CO₂ | 0.00 | 3.16 | 50.0 | 10.0 | 51.0 | 0.0000 | 0.0013 | 0.0000 | 0.0000 | 0.0003 | 0.0024 | 0.0000 | 0.0000 |
| Agriculture – Liming | CO₂ | 360.06 | 546.14 | 3.4 | 50.0 | 50.1 | 0.1963 | 0.2289 | 0.0008 | 0.0059 | 0.0423 | 0.0286 | 0.0385 | 0.0524 |
| Agriculture – Urea application | CO₂ | 39.19 | 570.72 | 10.0 | 50.0 | 51.0 | 0.0217 | 0.2433 | 0.0057 | 0.0062 | 0.2827 | 0.0878 | 0.0005 | 0.0592 |
| LULUCF – Forest land | CO₂ | 23325.33 | 22280.41 | 6.4 | 78.9 | 79.2 | 20.0958 | 14.7565 | 0.0875 | 0.2424 | 6.9054 | 2.1888 | 403.8417 | 217.7538 |
| LULUCF – Cropland | CO₂ | 468.49 | 379.36 | 8.0 | 69.0 | 69.4 | 0.3538 | 0.2202 | 0.0025 | 0.0041 | 0.1726 | 0.0467 | 0.1251 | 0.0485 |
| LULUCF – Grassland | CO₂ | 531.37 | 4181.18 | 8.4 | 27.9 | 29.1 | 0.1683 | 1.0182 | 0.0380 | 0.0455 | 1.0584 | 0.5404 | 0.0283 | 1.0367 |
| LULUCF – Wetlands | CO₂ | 10.70 | 13.33 | 33.0 | 124.8 | 129.1 | 0.0150 | 0.0144 | 0.0000 | 0.0001 | 0.0008 | 0.0068 | 0.0002 | 0.0002 |
| LULUCF – Settlements | CO₂ | 74.44 | 112.66 | 22.0 | 64.7 | 68.3 | 0.0553 | 0.0644 | 0.0002 | 0.0012 | 0.0111 | 0.0381 | 0.0031 | 0.0041 |
| LULUCF – Other land | CO₂ | 12.65 | 48.64 | 8.0 | 27.1 | 28.3 | 0.0039 | 0.0115 | 0.0004 | 0.0005 | 0.0095 | 0.0060 | 0.0000 | 0.0001 |
| LULUCF – Harvested wood products | CO₂ | 2072.91 | 10067.30 | 15.0 | 67.4 | 69.0 | 1.5560 | 5.8095 | 0.0802 | 0.1095 | 5.3995 | 2.3230 | 2.4213 | 33.7500 |
| Waste – Incineration and open burning of waste | CO₂ | 158.78 | 90.20 | 50.0 | 40.0 | 64.0 | 0.1106 | 0.0483 | 0.0013 | 0.0010 | 0.0506 | 0.0694 | 0.0122 | 0.0023 |
| Tokelau Energy industries – sectoral approach – liquid | CO₂ | 0.23 | 0.25 | 10.0 | 7.0 | 12.2 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Gas Diesel Oil – Sectoral approach – liquid | CO₂ | 0.90 | 2.18 | 50.0 | 1.5 | 50.0 | 0.0005 | 0.0009 | 0.0000 | 0.0000 | 0.0000 | 0.0017 | 0.0000 | 0.0000 |
| Tokelau Other/Residential – Sectoral approach – liquid | CO₂ | 0.12 | 0.07 | 20.0 | 7.0 | 21.2 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – Incineration and open burning of waste | CO₂ | 0.05 | 0.04 | 50.0 | 40.0 | 64.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Energy – Liquid fuels | CH₄ | 82.86 | 26.73 | 1.5 | 50.0 | 50.0 | 0.0451 | 0.0112 | 0.0009 | 0.0003 | 0.0441 | 0.0006 | 0.0020 | 0.0001 |
| Energy – Solid fuels | CH₄ | 36.38 | 23.97 | 4.6 | 50.0 | 50.2 | 0.0199 | 0.0101 | 0.0003 | 0.0003 | 0.0127 | 0.0017 | 0.0004 | 0.0001 |
| Energy – Gaseous fuels | CH₄ | 9.07 | 4.63 | 9.3 | 50.0 | 50.9 | 0.0050 | 0.0020 | 0.0001 | 0.0001 | 0.0039 | 0.0007 | 0.0000 | 0.0000 |
| Energy – Biomass | CH₄ | 66.60 | 74.23 | 50.0 | 50.0 | 70.7 | 0.0512 | 0.0439 | 0.0001 | 0.0008 | 0.0068 | 0.0571 | 0.0026 | 0.0019 |
| Energy – Fugitive – geothermal | CH₄ | 54.79 | 156.88 | 5.0 | 5.0 | 7.1 | 0.0042 | 0.0093 | 0.0009 | 0.0017 | 0.0047 | 0.0121 | 0.0000 | 0.0001 |
| Energy – Fugitive – venting/ processing | CH₄ | 66.06 | 55.61 | 9.3 | 50.0 | 50.9 | 0.0365 | 0.0236 | 0.0003 | 0.0006 | 0.0165 | 0.0079 | 0.0013 | 0.0006 |
| Energy – Fugitive – Coal mining | CH₄ | 328.23 | 135.35 | 4.6 | 50.0 | 50.2 | 0.1793 | 0.0568 | 0.0032 | 0.0015 | 0.1586 | 0.0096 | 0.0321 | 0.0032 |
| Energy – Fugitive – transmission and distribution | CH₄ | 279.97 | 215.87 | 9.3 | 100.0 | 100.4 | 0.3058 | 0.1813 | 0.0016 | 0.0023 | 0.1613 | 0.0308 | 0.0935 | 0.0329 |
| Energy – fugitive – oil and gas activities | CH₄ | 50.43 | 135.03 | 9.3 | 100.0 | 100.4 | 0.0551 | 0.1134 | 0.0008 | 0.0015 | 0.0755 | 0.0192 | 0.0030 | 0.0129 |
| Energy – fugitive – oil transportation and storage | CH₄ | 1.67 | 4.75 | 5.0 | 50.0 | 50.2 | 0.0009 | 0.0020 | 0.0000 | 0.0001 | 0.0014 | 0.0004 | 0.0000 | 0.0000 |
| IPPU – chemical industry | CH₄ | 27.60 | 107.25 | 2.0 | 80.0 | 80.0 | 0.0240 | 0.0718 | 0.0008 | 0.0012 | 0.0621 | 0.0033 | 0.0006 | 0.0052 |
| Agriculture – enteric fermentation | CH₄ | 27350.36 | 28974.60 | 3.9 | 15.5 | 16.0 | 4.7601 | 3.8766 | 0.0716 | 0.3152 | 1.1113 | 1.7383 | 22.6583 | 15.0279 |
| Agriculture – manure management | CH₄ | 727.81 | 1622.56 | 5.0 | 20.0 | 20.6 | 0.1632 | 0.2797 | 0.0074 | 0.0176 | 0.1470 | 0.1248 | 0.0266 | 0.0782 |
| Agriculture – burning of residues | CH₄ | 22.62 | 20.03 | 6.0 | 20.0 | 20.9 | 0.0051 | 0.0035 | 0.0001 | 0.0002 | 0.0020 | 0.0018 | 0.0000 | 0.0000 |
| LULUCF | CH₄ | 93.51 | 76.56 | 30.0 | 48.3 | 56.9 | 0.0578 | 0.0364 | 0.0005 | 0.0008 | 0.0237 | 0.0353 | 0.0033 | 0.0013 |
| Waste – solid waste disposal | CH₄ | 3337.79 | 2692.93 | 92.0 | 40.0 | 100.3 | 3.6420 | 2.2589 | 0.0179 | 0.0293 | 0.7172 | 3.8108 | 13.2642 | 5.1024 |
| Waste – wastewater treatment and discharge | CH₄ | 221.17 | 253.58 | 10.0 | 40.0 | 41.2 | 0.0992 | 0.0874 | 0.0004 | 0.0028 | 0.0148 | 0.0390 | 0.0098 | 0.0076 |
| Waste – Biological treatment of solid waste | CH₄ | 2.74 | 38.02 | 100.0 | 100.0 | 141.4 | 0.0042 | 0.0450 | 0.0004 | 0.0004 | 0.0375 | 0.0585 | 0.0000 | 0.0020 |
| Waste – incineration and open burning of waste | CH₄ | 127.32 | 77.82 | 50.0 | 100.0 | 111.8 | 0.1548 | 0.0728 | 0.0010 | 0.0008 | 0.0955 | 0.0599 | 0.0240 | 0.0053 |
| Tokelau Energy industries – Sectoral approach – liquid | CH₄ | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Gas Diesel Oil – Sectoral approach – liquid | CH₄ | 0.00 | 0.01 | 50.0 | 50.0 | 70.7 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Other/ Residential – Sectoral approach – liquid | CH₄ | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Agriculture – enteric fermentation | CH₄ | 0.09 | 0.06 | 20.0 | 50.0 | 53.9 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Agriculture – manure management | CH₄ | 1.06 | 0.76 | 20.0 | 30.0 | 36.1 | 0.0004 | 0.0002 | 0.0000 | 0.0000 | 0.0002 | 0.0002 | 0.0000 | 0.0000 |
| Tokelau Waste – solid waste disposal | CH₄ | 0.39 | 0.31 | 140.0 | 40.0 | 145.6 | 0.0006 | 0.0004 | 0.0000 | 0.0000 | 0.0001 | 0.0007 | 0.0000 | 0.0000 |
| Tokelau Waste – wastewater treatment and discharge | CH₄ | 0.15 | 0.26 | 10.0 | 40.0 | 41.2 | 0.0001 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – incineration and open burning of waste | CH₄ | 0.09 | 0.07 | 50.0 | 100.0 | 111.8 | 0.0001 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0001 | 0.0000 | 0.0000 |
| Energy – liquid fuels | N₂O | 141.23 | 154.20 | 1.5 | 50.0 | 50.0 | 0.0768 | 0.0645 | 0.0003 | 0.0017 | 0.0161 | 0.0035 | 0.0059 | 0.0042 |
| Energy – solid fuels | N₂O | 14.96 | 19.16 | 4.6 | 50.0 | 50.2 | 0.0082 | 0.0080 | 0.0000 | 0.0002 | 0.0002 | 0.0014 | 0.0001 | 0.0001 |
| Energy – gaseous fuels | N₂O | 5.55 | 3.87 | 9.3 | 50.0 | 50.9 | 0.0031 | 0.0016 | 0.0000 | 0.0000 | 0.0018 | 0.0006 | 0.0000 | 0.0000 |
| Energy – biomass | N₂O | 36.59 | 53.33 | 50.0 | 50.0 | 70.7 | 0.0281 | 0.0315 | 0.0001 | 0.0006 | 0.0031 | 0.0410 | 0.0008 | 0.0010 |
| Energy – fugitive – venting/ flaring | N₂O | 0.06 | 0.08 | 5.0 | 100.0 | 100.1 | 0.0001 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Other product manufacture and use | N₂O | 102.45 | 84.48 | 15.0 | 0.0 | 15.0 | 0.0167 | 0.0106 | 0.0005 | 0.0009 | 0.0000 | 0.0195 | 0.0003 | 0.0001 |
| Agriculture – agricultural soils | N₂O | 5284.78 | 7763.12 | 11.4 | 54.1 | 55.3 | 3.1791 | 3.5900 | 0.0097 | 0.0844 | 0.5228 | 1.3571 | 10.1064 | 12.8878 |
| Agriculture – manure management | N₂O | 50.69 | 116.38 | 5.0 | 100.0 | 100.1 | 0.0552 | 0.0974 | 0.0005 | 0.0013 | 0.0549 | 0.0090 | 0.0030 | 0.0095 |
| Agriculture – burning of residues | N₂O | 4.77 | 4.15 | 6.0 | 20.0 | 20.9 | 0.0011 | 0.0007 | 0.0000 | 0.0000 | 0.0004 | 0.0004 | 0.0000 | 0.0000 |
| LULUCF | N₂O | 214.02 | 110.88 | 30.0 | 48.3 | 56.9 | 0.1324 | 0.0527 | 0.0018 | 0.0012 | 0.0880 | 0.0512 | 0.0175 | 0.0028 |
| Waste – wastewater treatment and discharge | N₂O | 82.11 | 118.90 | 10.0 | 90.0 | 90.6 | 0.0809 | 0.0900 | 0.0001 | 0.0013 | 0.0118 | 0.0183 | 0.0065 | 0.0081 |
| Waste – Incineration and open burning of waste | N₂O | 29.45 | 18.27 | 50.0 | 100.0 | 111.8 | 0.0358 | 0.0171 | 0.0002 | 0.0002 | 0.0218 | 0.0141 | 0.0013 | 0.0003 |
| Waste – Biological treatment of solid waste | N₂O | 1.96 | 27.19 | 100.0 | 150.0 | 180.3 | 0.0038 | 0.0410 | 0.0003 | 0.0003 | 0.0402 | 0.0418 | 0.0000 | 0.0017 |
| Tokelau Energy industries – Sectoral approach – liquid | N₂O | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Gas Diesel Oil – Sectoral approach – liquid | N₂O | 0.01 | 0.02 | 50.0 | 50.0 | 70.7 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Other/Residential – Sectoral approach – liquid | N₂O | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau IPPU – Other product manufacture and use | N₂O | 0.05 | 0.02 | 15.0 | 0.0 | 15.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – Wastewater treatment and discharge | N₂O | 0.02 | 0.00 | 10.0 | 90.0 | 90.6 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – Incineration and open burning of waste | N₂O | 0.01 | 0.01 | 50.0 | 100.0 | 111.8 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Refrigeration and air conditioning | HFCs | 0.00 | 1637.74 | 25.0 | 0.0 | 25.0 | 0.0000 | 0.3424 | 0.0178 | 0.0178 | 0.0000 | 0.6298 | 0.0000 | 0.1172 |
| IPPU – Foam blowing agents | HFCs | 0.00 | 6.71 | 12.0 | 50.0 | 51.4 | 0.0000 | 0.0029 | 0.0001 | 0.0001 | 0.0036 | 0.0012 | 0.0000 | 0.0000 |
| IPPU – Fire protection | HFCs | 0.00 | 2.22 | 10.0 | 41.0 | 42.2 | 0.0000 | 0.0008 | 0.0000 | 0.0000 | 0.0010 | 0.0003 | 0.0000 | 0.0000 |
| IPPU – Aerosols | HFCs | 0.00 | 87.16 | 25.0 | 10.0 | 26.9 | 0.0000 | 0.0196 | 0.0009 | 0.0009 | 0.0095 | 0.0335 | 0.0000 | 0.0004 |
| IPPU – Aluminium | PFCs | 909.95 | 89.12 | 5.0 | 30.0 | 30.4 | 0.3010 | 0.0227 | 0.0119 | 0.0010 | 0.3571 | 0.0069 | 0.0906 | 0.0005 |
| IPPU – Refrigeration and air conditioning | PFCs | 0.00 | 0.00 | 25.0 | 0.0 | 25.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Other product use | PFCs | 0.00 | 0.01 | 80.0 | 0.0 | 80.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Magnesium production | SF6 | 2.74 | 0.00 | 100.0 | 0.0 | 100.0 | 0.0030 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Electrical equipment | SF6 | 14.50 | 12.97 | 20.0 | 30.0 | 36.1 | 0.0057 | 0.0039 | 0.0001 | 0.0001 | 0.0019 | 0.0040 | 0.0000 | 0.0000 |
| IPPU – Other product use | SF6 | 2.74 | 2.74 | 80.0 | 0.0 | 80.0 | 0.0024 | 0.0018 | 0.0000 | 0.0000 | 0.0000 | 0.0034 | 0.0000 | 0.0000 |
| Tokelau IPPU – Product uses as substitutes for ODS | HFCs | 0.00 | 0.23 | 32.0 | 0.0 | 32.0 | 0.0000 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0001 | 0.0000 | 0.0000 |
| Total emissions removals |  | 91.932.6 | 119,588.0 |  | Uncertainty in the base year | 21.3 |  | Uncertainty in the final year |  | 16.9 |  | Uncertainty in the trend | 10.6 |  |  |

Table A2.1.2 	Uncertainty calculation (excluding LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2019 (2006 IPCC, Approach 1)

| IPCC source category | Gas | 1990 emissions Kt CO₂-e | 2019 emissions, kt CO₂-e | Activity data uncertainty (%) | Emission factor uncertainty (%) | Combined uncertainty (%) | Combined uncertainty as a per cent of the national total in 1990 (%) | Combined uncertainty as a per cent of the national total in 2018 (%) | Type A sensitivity (%) | Type B sensitivity (%) | Uncertainty in the trend in national total introduced by emission factor uncertainty (%) | Uncertainty in trend in national total introduced by activity data uncertainty (%) | Combined uncertainty of the national total in 1990 ((%)2) | Combined uncertainty of the national total in 2019 ((%)2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Energy – liquid fuels | CO₂ | 11803.54 | 20235.22 | 1.5 | 0.5 | 1.5 | 0.2795 | 0.3791 | 0.0815 | 0.3107 | 0.0407 | 0.6410 | 0.0781 | 0.1437 |
| Energy – solid fuels | CO₂ | 3211.03 | 4155.77 | 4.6 | 2.2 | 5.1 | 0.2508 | 0.2568 | 0.0015 | 0.0638 | 0.0032 | 0.4149 | 0.0629 | 0.0659 |
| Energy – gaseous fuels | CO₂ | 7095.50 | 7787.32 | 9.3 | 2.4 | 9.6 | 1.0431 | 0.9057 | 0.0181 | 0.1196 | 0.0436 | 1.5668 | 1.0880 | 0.8204 |
| Energy – fugitive – geothermal | CO₂ | 228.58 | 585.06 | 5.0 | 5.0 | 7.1 | 0.0248 | 0.0503 | 0.0045 | 0.0090 | 0.0227 | 0.0635 | 0.0006 | 0.0025 |
| Energy – fugitive – venting/ flaring | CO₂ | 229.84 | 434.30 | 9.3 | 2.4 | 9.6 | 0.0338 | 0.0505 | 0.0022 | 0.0067 | 0.0053 | 0.0874 | 0.0011 | 0.0026 |
| Energy – fugitive – oil and gas activities | CO₂ | 0.08 | 0.20 | 5.0 | 100.0 | 100.1 | 0.0001 | 0.0002 | 0.0000 | 0.0000 | 0.0002 | 0.0000 | 0.0000 | 0.0000 |
| Energy – fugitive – transmission and distribution | CO₂ | 1.46 | 1.31 | 9.3 | 100.0 | 100.4 | 0.0023 | 0.0016 | 0.0000 | 0.0000 | 0.0008 | 0.0003 | 0.0000 | 0.0000 |
| IPPU – Cement production | CO₂ | 448.75 | 410.94 | 1.0 | 1.0 | 1.4 | 0.0097 | 0.0071 | 0.0024 | 0.0063 | 0.0024 | 0.0089 | 0.0001 | 0.0000 |
| IPPU – Lime production | CO₂ | 82.60 | 108.04 | 2.0 | 2.0 | 2.8 | 0.0036 | 0.0037 | 0.0001 | 0.0017 | 0.0001 | 0.0047 | 0.0000 | 0.0000 |
| IPPU – Ceramics | CO₂ | 0.01 | 0.01 | 50.0 | 20.0 | 53.9 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Other uses of soda ash | CO₂ | 5.87 | 6.63 | 3.0 | 2.0 | 3.6 | 0.0003 | 0.0003 | 0.0000 | 0.0001 | 0.0000 | 0.0004 | 0.0000 | 0.0000 |
| IPPU – Other uses of limestone | CO₂ | 24.63 | 92.45 | 3.0 | 2.0 | 3.6 | 0.0014 | 0.0040 | 0.0009 | 0.0014 | 0.0019 | 0.0060 | 0.0000 | 0.0000 |
| IPPU – Ammonia production | CO₂ | 21.68 | 22.83 | 2.0 | 6.0 | 6.3 | 0.0021 | 0.0018 | 0.0001 | 0.0004 | 0.0004 | 0.0010 | 0.0000 | 0.0000 |
| IPPU – Calcium carbide | CO₂ | 1.43 | 1.43 | 50.0 | 50.0 | 70.7 | 0.0016 | 0.0012 | 0.0000 | 0.0000 | 0.0003 | 0.0016 | 0.0000 | 0.0000 |
| IPPU – Hydrogen production | CO₂ | 152.29 | 159.04 | 2.0 | 6.0 | 6.3 | 0.0148 | 0.0122 | 0.0005 | 0.0024 | 0.0031 | 0.0069 | 0.0002 | 0.0001 |
| IPPU – Iron and steel | CO₂ | 1306.73 | 1661.61 | 5.0 | 7.0 | 8.6 | 0.1726 | 0.1736 | 0.0002 | 0.0255 | 0.0011 | 0.1804 | 0.0298 | 0.0302 |
| IPPU – Aluminium | CO₂ | 448.98 | 574.50 | 5.0 | 2.0 | 5.4 | 0.0371 | 0.0376 | 0.0001 | 0.0088 | 0.0002 | 0.0624 | 0.0014 | 0.0014 |
| IPPU – Secondary lead production | CO₂ | 1.80 | 0.00 | 50.0 | 50.0 | 70.7 | 0.0020 | 0.0000 | 0.0000 | 0.0000 | 0.0017 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Lubricant use | CO₂ | 22.83 | 42.53 | 20.0 | 50.0 | 53.9 | 0.0189 | 0.0278 | 0.0002 | 0.0007 | 0.0105 | 0.0185 | 0.0004 | 0.0008 |
| IPPU – Paraffin wax | CO₂ | 2.35 | 2.35 | 20.0 | 100.0 | 102.0 | 0.0037 | 0.0029 | 0.0000 | 0.0000 | 0.0010 | 0.0010 | 0.0000 | 0.0000 |
| IPPU – Other: Urea catalyst in road transport | CO₂ | 0.00 | 3.16 | 50.0 | 10.0 | 51.0 | 0.0000 | 0.0020 | 0.0000 | 0.0000 | 0.0005 | 0.0034 | 0.0000 | 0.0000 |
| Agriculture – Liming | CO₂ | 360.06 | 546.14 | 3.4 | 50.0 | 50.1 | 0.2771 | 0.3325 | 0.0014 | 0.0084 | 0.0699 | 0.0403 | 0.0768 | 0.1106 |
| Agriculture – Urea application | CO₂ | 39.19 | 570.72 | 10.0 | 50.0 | 51.0 | 0.0307 | 0.3535 | 0.0080 | 0.0088 | 0.4001 | 0.1239 | 0.0009 | 0.1250 |
| Waste – Incineration and open burning of waste | CO₂ | 158.78 | 90.20 | 50.0 | 40.0 | 64.0 | 0.1561 | 0.0702 | 0.0017 | 0.0014 | 0.0679 | 0.0979 | 0.0244 | 0.0049 |
| Tokelau Energy industries – Sectoral approach – liquid | CO₂ | 0.23 | 0.25 | 10.0 | 7.0 | 12.2 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0001 | 0.0000 | 0.0000 |
| Tokelau Gas Diesel Oil – Sectoral approach – liquid | CO₂ | 0.90 | 2.18 | 50.0 | 1.5 | 50.0 | 0.0007 | 0.0013 | 0.0000 | 0.0000 | 0.0000 | 0.0024 | 0.0000 | 0.0000 |
| Tokelau Other/Residential – Sectoral approach – liquid | CO₂ | 0.12 | 0.07 | 20.0 | 7.0 | 21.2 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – Incineration and open burning of waste | CO₂ | 0.05 | 0.04 | 50.0 | 40.0 | 64.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Energy – Liquid fuels | CH₄ | 82.86 | 26.73 | 1.5 | 50.0 | 50.0 | 0.0636 | 0.0162 | 0.0012 | 0.0004 | 0.0599 | 0.0008 | 0.0040 | 0.0003 |
| Energy – Solid fuels | CH₄ | 36.38 | 23.97 | 4.6 | 50.0 | 50.2 | 0.0280 | 0.0146 | 0.0003 | 0.0004 | 0.0169 | 0.0024 | 0.0008 | 0.0002 |
| Energy – Gaseous fuels | CH₄ | 9.07 | 4.63 | 9.3 | 50.0 | 50.9 | 0.0071 | 0.0029 | 0.0001 | 0.0001 | 0.0052 | 0.0009 | 0.0001 | 0.0000 |
| Energy – Biomass | CH₄ | 66.60 | 74.23 | 50.0 | 50.0 | 70.7 | 0.0723 | 0.0638 | 0.0002 | 0.0011 | 0.0076 | 0.0806 | 0.0052 | 0.0041 |
| Energy – Fugitive – geothermal | CH₄ | 54.79 | 156.88 | 5.0 | 5.0 | 7.1 | 0.0059 | 0.0135 | 0.0013 | 0.0024 | 0.0067 | 0.0170 | 0.0000 | 0.0002 |
| Energy – Fugitive – venting/ processing | CH₄ | 66.06 | 55.61 | 9.3 | 50.0 | 50.9 | 0.0516 | 0.0344 | 0.0004 | 0.0009 | 0.0214 | 0.0112 | 0.0027 | 0.0012 |
| Energy – Fugitive – coal mining | CH₄ | 328.23 | 135.35 | 4.6 | 50.0 | 50.2 | 0.2531 | 0.0826 | 0.0043 | 0.0021 | 0.2146 | 0.0135 | 0.0640 | 0.0068 |
| Energy – Fugitive – transmission and distribution | CH₄ | 279.97 | 215.87 | 9.3 | 100.0 | 100.4 | 0.4317 | 0.2634 | 0.0021 | 0.0033 | 0.2119 | 0.0434 | 0.1864 | 0.0694 |
| Energy – Fugitive – oil and gas activities | CH₄ | 50.43 | 135.03 | 9.3 | 100.0 | 100.4 | 0.0778 | 0.1647 | 0.0011 | 0.0021 | 0.1095 | 0.0272 | 0.0060 | 0.0271 |
| Energy – Fugitive – oil transportation and storage | CH₄ | 1.67 | 4.75 | 5.0 | 50.0 | 50.2 | 0.0013 | 0.0029 | 0.0000 | 0.0001 | 0.0020 | 0.0005 | 0.0000 | 0.0000 |
| IPPU – Chemical industry | CH₄ | 27.60 | 107.25 | 2.0 | 80.0 | 80.0 | 0.0339 | 0.1043 | 0.0011 | 0.0016 | 0.0889 | 0.0047 | 0.0012 | 0.0109 |
| Agriculture – Enteric fermentation | CH₄ | 27350.36 | 28974.60 | 3.9 | 15.5 | 16.0 | 6.7190 | 5.6318 | 0.0855 | 0.4449 | 1.3272 | 2.4537 | 45.1455 | 31.7167 |
| Agriculture – Manure management | CH₄ | 727.81 | 1622.56 | 5.0 | 20.0 | 20.6 | 0.2304 | 0.4064 | 0.0108 | 0.0249 | 0.2158 | 0.1762 | 0.0531 | 0.1651 |
| Agriculture – Burning of residues | CH₄ | 22.62 | 20.03 | 6.0 | 20.0 | 20.9 | 0.0073 | 0.0051 | 0.0001 | 0.0003 | 0.0026 | 0.0026 | 0.0001 | 0.0000 |
| Waste – Solid waste disposal | CH₄ | 3337.79 | 2692.93 | 92.0 | 40.0 | 100.3 | 5.1409 | 3.2816 | 0.0234 | 0.0413 | 0.9366 | 5.3791 | 26.4284 | 10.7688 |
| Waste – Wastewater treatment and discharge | CH₄ | 221.17 | 253.58 | 10.0 | 40.0 | 41.2 | 0.1400 | 0.1270 | 0.0004 | 0.0039 | 0.0159 | 0.0551 | 0.0196 | 0.0161 |
| Waste – Biological treatment of solid waste | CH₄ | 2.74 | 38.02 | 100.0 | 100.0 | 141.4 | 0.0059 | 0.0653 | 0.0005 | 0.0006 | 0.0531 | 0.0825 | 0.0000 | 0.0043 |
| Waste – Incineration and open burning of waste | CH₄ | 127.32 | 77.82 | 50.0 | 100.0 | 111.8 | 0.2186 | 0.1057 | 0.0013 | 0.0012 | 0.1276 | 0.0845 | 0.0478 | 0.0112 |
| Tokelau Energy industries – Sectoral approach – liquid | CH₄ | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Gas Diesel Oil – Sectoral approach – liquid | CH₄ | 0.00 | 0.01 | 50.0 | 50.0 | 70.7 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Other/Residential – Sectoral approach – liquid | CH₄ | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Agriculture – enteric fermentation | CH₄ | 0.09 | 0.06 | 20.0 | 50.0 | 53.9 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Agriculture – manure management | CH₄ | 1.06 | 0.76 | 20.0 | 30.0 | 36.1 | 0.0006 | 0.0003 | 0.0000 | 0.0000 | 0.0003 | 0.0003 | 0.0000 | 0.0000 |
| Tokelau Waste – solid waste disposal | CH₄ | 0.39 | 0.31 | 140.0 | 40.0 | 145.6 | 0.0009 | 0.0005 | 0.0000 | 0.0000 | 0.0001 | 0.0009 | 0.0000 | 0.0000 |
| Tokelau Waste – wastewater treatment and discharge | CH₄ | 0.15 | 0.26 | 10.0 | 40.0 | 41.2 | 0.0001 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0001 | 0.0000 | 0.0000 |
| Tokelau Waste – incineration and open burning of waste | CH₄ | 0.09 | 0.07 | 50.0 | 100.0 | 111.8 | 0.0002 | 0.0001 | 0.0000 | 0.0000 | 0.0001 | 0.0001 | 0.0000 | 0.0000 |
| Energy – Liquid fuels | N₂O | 141.23 | 154.20 | 1.5 | 50.0 | 50.0 | 0.1085 | 0.0937 | 0.0004 | 0.0024 | 0.0187 | 0.0049 | 0.0118 | 0.0088 |
| Energy – Solid fuels | N₂O | 14.96 | 19.16 | 4.6 | 50.0 | 50.2 | 0.0115 | 0.0117 | 0.0000 | 0.0003 | 0.0002 | 0.0019 | 0.0001 | 0.0001 |
| Energy – Gaseous fuels | N₂O | 5.55 | 3.87 | 9.3 | 50.0 | 50.9 | 0.0043 | 0.0024 | 0.0000 | 0.0001 | 0.0024 | 0.0008 | 0.0000 | 0.0000 |
| Energy – Biomass | N₂O | 36.59 | 53.33 | 50.0 | 50.0 | 70.7 | 0.0397 | 0.0458 | 0.0001 | 0.0008 | 0.0054 | 0.0579 | 0.0016 | 0.0021 |
| Energy – Fugitive – venting/ flaring | N₂O | 0.06 | 0.08 | 5.0 | 100.0 | 100.1 | 0.0001 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Other product manufacture and use | N₂O | 102.45 | 84.48 | 15.0 | 0.0 | 15.0 | 0.0236 | 0.0154 | 0.0007 | 0.0013 | 0.0000 | 0.0275 | 0.0006 | 0.0002 |
| Agriculture – Agricultural soils | N₂O | 5284.78 | 7763.12 | 11.4 | 54.1 | 55.3 | 4.4874 | 5.2154 | 0.0166 | 0.1192 | 0.8997 | 1.9156 | 20.1365 | 27.1999 |
| Agriculture – Manure management | N₂O | 50.69 | 116.38 | 5.0 | 100.0 | 100.1 | 0.0779 | 0.1416 | 0.0008 | 0.0018 | 0.0803 | 0.0126 | 0.0061 | 0.0200 |
| Agriculture – Burning of residues | N₂O | 4.77 | 4.15 | 6.0 | 20.0 | 20.9 | 0.0015 | 0.0011 | 0.0000 | 0.0001 | 0.0006 | 0.0005 | 0.0000 | 0.0000 |
| Waste – Wastewater treatment and discharge | N₂O | 82.11 | 118.90 | 10.0 | 90.0 | 90.6 | 0.1142 | 0.1308 | 0.0002 | 0.0018 | 0.0209 | 0.0258 | 0.0130 | 0.0171 |
| Waste – Incineration and open burning of waste | N₂O | 29.45 | 18.27 | 50.0 | 100.0 | 111.8 | 0.0506 | 0.0248 | 0.0003 | 0.0003 | 0.0291 | 0.0198 | 0.0026 | 0.0006 |
| Waste – Biological treatment of solid waste | N₂O | 1.96 | 27.19 | 100.0 | 150.0 | 180.3 | 0.0054 | 0.0595 | 0.0004 | 0.0004 | 0.0569 | 0.0590 | 0.0000 | 0.0035 |
| Tokelau Energy industries – Sectoral approach – liquid | N₂O | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Gas Diesel Oil – Sectoral approach – liquid | N₂O | 0.01 | 0.02 | 50.0 | 50.0 | 70.7 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Other/Residential – Sectoral approach – liquid | N₂O | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau IPPU – Other product manufacture and use | N₂O | 0.05 | 0.02 | 15.0 | 0.0 | 15.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – wastewater treatment and discharge | N₂O | 0.02 | 0.00 | 10.0 | 90.0 | 90.6 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Tokelau Waste – Incineration and open burning of waste | N₂O | 0.01 | 0.01 | 50.0 | 100.0 | 111.8 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Refrigeration and air conditioning | HFCs | 0.00 | 1637.74 | 25.0 | 0.0 | 25.0 | 0.0000 | 0.4974 | 0.0251 | 0.0251 | 0.0000 | 0.8890 | 0.0000 | 0.2474 |
| IPPU – Foam blowing agents | HFCs | 0.00 | 6.71 | 12.0 | 50.0 | 51.4 | 0.0000 | 0.0042 | 0.0001 | 0.0001 | 0.0051 | 0.0017 | 0.0000 | 0.0000 |
| IPPU – Fire protection | HFCs | 0.00 | 2.22 | 10.0 | 41.0 | 42.2 | 0.0000 | 0.0011 | 0.0000 | 0.0000 | 0.0014 | 0.0005 | 0.0000 | 0.0000 |
| IPPU – Aerosols | HFCs | 0.00 | 87.16 | 25.0 | 10.0 | 26.9 | 0.0000 | 0.0285 | 0.0013 | 0.0013 | 0.0134 | 0.0473 | 0.0000 | 0.0008 |
| IPPU – Aluminium | PFCs | 909.95 | 89.12 | 5.0 | 30.0 | 30.4 | 0.4249 | 0.0329 | 0.0163 | 0.0014 | 0.4886 | 0.0097 | 0.1806 | 0.0011 |
| IPPU – Refrigeration and air conditioning | PFCs | 0.00 | 0.00 | 25.0 | 0.0 | 25.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Other product use | PFCs | 0.00 | 0.01 | 80.0 | 0.0 | 80.0 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Magnesium production | SF6 | 2.74 | 0.00 | 100.0 | 0.0 | 100.0 | 0.0042 | 0.0000 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| IPPU – Electrical equipment | SF6 | 14.50 | 12.97 | 20.0 | 30.0 | 36.1 | 0.0080 | 0.0057 | 0.0001 | 0.0002 | 0.0025 | 0.0056 | 0.0001 | 0.0000 |
| IPPU – Other product use | SF6 | 2.74 | 2.74 | 80.0 | 0.0 | 80.0 | 0.0034 | 0.0027 | 0.0000 | 0.0000 | 0.0000 | 0.0048 | 0.0000 | 0.0000 |
| Tokelau IPPU – Product uses as substitutes for ODS | HFCs | 0.00 | 0.23 | 32.0 | 0.0 | 32.0 | 0.0000 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0002 | 0.0000 | 0.0000 |
| Total emissions |  | 65,129.2 | 82,317.7 |  | Uncertainty in the base year |  | 9.7% | Uncertainty in the final year |  | 8.5% |  | Uncertainty in the trend | 6.8% |  |

## Annex 2: References

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 1. General Guidance and Reporting. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

UNFCCC. 2013. FCCC/CP/2013/Add.3. Guidelines for the preparation of national communications by Parties included in Annex I to the Convention, Part I: UNFCCC reporting guidelines on annual greenhouse gas inventories (addendum to Decision 24/CP.19).

# Annex 3: Detailed methodological information for other sectors

## A3.1	Supplementary information for the Agriculture sector

### A3.1.1	Livestock population data

#### 2017 Agricultural Production Census and 2019 Agricultural Production Survey

Details of the Agricultural Production Census (APC) and Agricultural Production Survey (APS) are included to provide an understanding of the livestock statistics process and uncertainty values. The information here is provided by Stats NZ, with full details available from the Stats NZ website (www.stats.govt.nz/information-releases/agricultural-production-statistics-june-2019-final).

Stats NZ holds the APC every five years, with the most recent census held in 2017. In all other years, Stats NZ holds the APS, which applies a very similar method to the APC, but targets only about half of the businesses involved in agriculture or forestry production. The National Inventory Report is compiled with data from the APC and APS.

The 2019 APS used a stratified sample design to select a sample from the target population (all businesses that were engaged in agricultural production activity (including livestock, cropping, horticulture and forestry) or that owned land that was intended for agricultural activity during the year ended 30 June 2019). The response rate, or the estimated proportion of eligible businesses that responded to the 2019 APS, was 83 per cent.

The imputation levels of the 2017 APC and the 2019 APS are provided in table A3.1.1. Full details on APC and APS data collection methodology can be found on the Stats NZ website (datainfoplus.stats.govt.nz).

Sampling error arises in the APS from selecting a sample of businesses and weighting the results rather than taking a complete enumeration (i.e., census). Non-sampling error arises from biases in the patterns of response and non-response, inaccuracies in reporting by respondents and errors in the recording and classification of data. Stats NZ adopts procedures to detect and minimise these types of errors, but they may still occur and are not easy to quantify.

Table A3.1.1	Imputation levels and sampling errors for recent Agricultural Production surveys

| Statistic | Proportion of total estimate imputed (%) |  |  | Relative sampling errors at 95% confidence interval (%) |  |
| --- | --- | --- | --- | --- | --- |
| Survey year | 2018 |  | 2019 | 2018 | 2019 |
| Ewe hoggets put to ram | 17 |  | 17 | 4 | 5 |
| Breeding ewes, two tooth and over | 17 |  | 16 | 2 | 3 |
| Total number of sheep | 17 |  | 16 | 3 | 3 |
| Lambs born to ewe hoggets | 18 |  | 16 | 4 | 5 |
| Lambs born to ewes | 17 |  | 16 | 2 | 3 |
| Total number of lambs | 17 |  | 16 | 2 | 3 |
| Calves born alive to dairy heifers and/or cows | 29 |  | 28 | 4 | 4 |
| Dairy cows and heifers, in milk or calf | 28 |  | 25 | 6 | 4 |
| Total number of dairy cattle | 27 |  | 24 | 6 | 3 |
| Calves born alive to beef heifers and/or cows | 18 |  | 18 | 2 | 4 |
| Beef cows and heifers (in calf) one to two years | 17 |  | 18 | 3 | 13 |
| Beef cows and heifers (in calf) two years and over | 17 |  | 18 | 2 | 3 |
| Total number of beef cattle | 18 |  | 19 | 2 | 3 |
| Female deer mated | 13 |  | 8 | 5 | 5 |
| Total number of deer | 13 | 8 |  | 5 | 5 |
| Fawns born on farm and alive at four months | 13 | 8 |  | 5 | 5 |
| Total pigs | 9 |  | 3 | 1 | 1 |
| Area of wheat harvested | 15 |  | 14 | 4 | 5 |
| Area of barley harvested | 20 |  | 17 | 7 | 8 |
| Area of oat grain harvested | 14 |  | 18 | 13 | 22 |
| Area of maize grain harvested | 11 |  | 15 | 12 | 10 |

#### Livestock characterisation in New Zealand’s Tier 2 modelling

The delineation of the major livestock categories in New Zealand’s Tier 2 livestock nutritional and energy requirements modelling (see table A3.1.2) are taken from population data collected by the APC and APS and Ministry for Primary Industries slaughter statistics.

Table A3.1.2	Characterisation of major livestock subcategories (dairy cattle, beef cattle,
sheep and deer) in New Zealand’s Tier 2 livestock modelling

| Livestock category |  | Subcategory |
| --- | --- | --- |
| Dairy cattle | Milking cows and heifers Growing females less than one year Growing females one to two years Breeding bulls | Milking cows and heifers |
|  |  | Growing females less than one year |
|  |  | Growing females one to two years |
|  |  | Breeding bulls |
|  | Northland Auckland Waikato Bay of Plenty Gisborne Hawke's Bay Taranaki Manawatu–Whanganui Wellington Tasman Nelson Marlborough West Coast Canterbury Otago Southland | Northland |
|  |  | Auckland |
|  |  | Waikato |
|  |  | Bay of Plenty |
|  |  | Gisborne |
|  |  | Hawke's Bay |
|  |  | Taranaki |
|  |  | Manawatu–Whanganui |
|  |  | Wellington |
|  |  | Tasman |
|  |  | Nelson |
|  |  | Marlborough |
|  |  | West Coast |
|  |  | Canterbury |
|  |  | Otago |
|  |  | Southland |
| Beef cattle categories |  | Breeding growing cows less than one year |
|  |  | Breeding growing cows one to two years |
|  |  | Breeding growing cows two to three years |
|  |  | Breeding mature cows |
|  |  | Breeding bulls – mixed age |
|  |  | Slaughter heifers less than one year |
|  |  | Slaughter heifers one to two years |
|  |  | Slaughter steers less than one year |
|  |  | Slaughter steers one to two years |
|  |  | Slaughter bulls less than one year |
|  |  | Slaughter bulls one to two years |
| Sheep categories |  | Dry ewes |
|  |  | Mature breeding ewes |
|  |  | Growing breeding sheep |
|  |  | Growing non-breeding sheep |
|  |  | Wethers |
|  |  | Lambs |
|  |  | Rams |
| Deer categories |  | Breeding hinds |
|  |  | Hinds less than one year |
|  |  | Hinds one to two years |
|  |  | Stags less than one year |
|  |  | Stags one to two years |
|  |  | Stags two to three years |
|  |  | Mixed age and breeding stags |

### A3.1.2	Key parameters and emission factors used in the 
Agriculture sector

For the major livestock categories, milk yield varies over the course of a year, which affects energy requirements, feed intake and greenhouse gas emissions. Table A3.1.3 shows the proportions that are used to calculate milk yield for different months over the course of a year. Table A3.1.4 shows the emission factors used to calculate methane emissions from minor livestock species, while tables A3.1.5 and A3.1.6 show the emission factors used to calculate nitrous oxide emissions from agriculture. Table A3.1.7 shows some of the parameter values used to calculate nitrous oxide emissions.

Table A3.1.3	Proportion of annual milk yield each month for major livestock categories

| Month | Dairy cattle | Beef cattle | Sheep | Deer |
| --- | --- | --- | --- | --- |
| July | 0.0088 | 0.0000 | 0.0000 | 0.0000 |
| August | 0.0578 | 0.0000 | 0.0000 | 0.0000 |
| September | 0.1213 | 0.1670 | 0.1639 | 0.0000 |
| October | 0.1503 | 0.1670 | 0.2541 | 0.0000 |
| November | 0.1425 | 0.1670 | 0.2459 | 0.1000 |
| December | 0.1282 | 0.1670 | 0.2541 | 0.2583 |
| January | 0.1109 | 0.1670 | 0.0820 | 0.2583 |
| February | 0.0900 | 0.1670 | 0.0000 | 0.2333 |
| March | 0.0851 | 0.0000 | 0.0000 | 0.1500 |
| April | 0.0654 | 0.0000 | 0.0000 | 0.0000 |
| May | 0.0335 | 0.0000 | 0.0000 | 0.0000 |
| June | 0.0061 | 0.0000 | 0.0000 | 0.0000 |

Source: Suttie (2012) and Pickering and Fick (2015)

Note:	All values presented in the table are rounded to four decimal places for presentation purposes and precise values are available upon request.

Table A3.1.4	Methane emission factors for Tier 1 enteric fermentation livestock and manure management

| Emission factor | Emission type | Source | Parameter value (kg CH4/head/yr) |
| --- | --- | --- | --- |
| EFGOATS | Enteric fermentation – goats | Lassey (2011) | 9.0 |
| EFHORSES | Enteric fermentation – horses | IPCC (2006), table 10.10 | 18.0 |
| EFMULES | Enteric fermentation – mules and asses | IPCC (2006), table 10.10 | 10.0 |
| EFSWINE | Enteric fermentation – swine | Hill (2012) | 1.06 |
| EFALPACA | Enteric fermentation – alpaca | IPCC (2006), table 10.10 | 8.0 |
| MMGOATS | Manure management – goats | IPCC (2006), table 10.15 | 0.20 |
| MMHORSES | Manure management – horses | IPCC (2006), table 10.15 | 2.34 |
| MMMULES | Manure management – mules and asses | IPCC (2006), table 10.15 | 1.1 |
| MMSWINE | Manure management – swine | Hill (2012); IPCC (2000) | 5.94 |
| MMBROILERS | Manure management – broilers | Fick et al. (2011) | 0.022 |
| MMLAYERS | Manure management – layer hens | Fick et al. (2011) | 0.016 |
| MMOTHER POULTRY | Manure management – other poultry | IPCC (1996), table 4.5 | 0.117 |
| MMALPACA | Manure management – alpaca | New Zealand 1990 sheep value | 0.103 |

Table A3.1.5	Emission factors for New Zealand’s agriculture nitrous oxide emissions

| Emission factor | Emissions | Source | Parameter value |
| --- | --- | --- | --- |
| EF1 (kg N2O-N/kg N) | Direct emissions from nitrogen input to soil | Kelliher and de Klein (unpublished) | 0.0100 |
| EF1-UREA (kg N2O-N/kg N) | Direct emissions from nitrogen input to soil from urea fertiliser | van der Weerden et al. (2016) | 0.0059 |
| EF1-DAIRY (kg N2O-N/kg N) | Direct emissions from nitrogen input to soil from dairy cattle manure | van der Weerden et al. (2016) | 0.0025 |
| EF2 (kg N2O-N/ha-yr) | Direct emissions from organic soil mineralisation due to cultivation | IPCC (2006), table 11.1 | 8.0000 |
| EF3SSD (kg N2O-N/kg N excreted) | Direct emissions from waste in the solid waste and dry lot animal waste management systems | IPCC (2000), table 4.12 | 0.0200 |
| EF3(PRP-MINOR) (kg N2O-N/kg N excreted) | Direct emissions from manure (dung and urine) from minor grazing animals (i.e., excluding cattle, sheep and deer) in pasture, range and paddock systems | Carran et al. (1995); Muller et al. (1995); de Klein et al. (2003) | 0.0100 |
| EF3(PRP DUNG) (kg N2O-N/kg N excreted) | Direct emissions from dung in pasture, range and paddock systems for cattle, sheep and deer (direct emission factors for dung are reported in table A3.1.6) | van der Weerden et al. (2019) | 0.0012 |
| EF3OTHER (kg N2O-N/kg N excreted) | Direct emissions from waste in other animal waste management systems | IPCC (2000), table 4.13 | 0.0050 |
| EF3POULTRY (kg N2O-N/kg N excreted) | Direct emissions from waste in other animal waste management systems – poultry specific | Fick et al. (2011) | 0.0010 |
| EF4 (kg N2O-N/kg NHx-N) | Indirect emissions from volatising nitrogen | IPCC (2006), table 11.3 | 0.0100 |
| EF5 (kg N2O-N/kg N leached and run-off) | Indirect emissions from leaching nitrogen | IPCC (2006), table 11.3 | 0.0075 |

Table A3.1.6	Direct nitrous oxide emission factors for urine deposited by cattle, sheep and deer,
by livestock type and slope

|  | Emission factor by topography (kg N2O–N/kg N excreted) |  |
| --- | --- | --- |
| Livestock type | Flat and low sloped land (less than 12° gradient) EF3(PRP-FLAT) | Medium and steep sloped land (greater than 12° gradient) EF3(PRP-STEEP) |
| All cattle (includes dairy and non-dairy) | 0.0098 | 0.0033 |
| Deer | 0.0074 | 0.0020 |
| Sheep | 0.0050 | 0.0008 |

Source: Values used as calculated by van der Weerden et al. (2019)

Note:	N = nitrogen; N2O = nitrous oxide.

Table A3.1.7	Parameter values for New Zealand’s agriculture nitrous oxide emissions

| Parameter (fraction) | Fraction of the parameter | Source | Parameter value |
| --- | --- | --- | --- |
| FracGASF (kg NH3-N + NOx-N/kg of synthetic fertiliser N applied) | Total synthetic fertiliser emitted as NOx or NH3 | IPCC (2006) verified by Sherlock et al. (2008) | 0.1 |
| FracGASM (kg NH3-N + NOx-N/kg of N excreted by livestock) | Total nitrogen emitted as NOx or NH3 | Sherlock et al. (2008) | 0.1 |
| FracLEACH(-H) (kg N/kg fertiliser or manure N) | Nitrogen input to soils that is lost through leaching and run-off | Thomas et al. (unpublished, 2005) | 0.07 |
| FracBURN (kg N/kg crop-N) | Crop residue burned in fields | Thomas et al. (2008), table 14 | Crop specific survey data |
| FracBURNL (kg N/kg legume-N) | Legume crop residue burned in fields | Thomas et al. (2008) Practice does not occur in New Zealand | 0 |
| FracRENEW | Fraction of land undergoing pasture renewal | Thomas et al. (2014) | Year specific |
| FracREMOVE | Fraction of nitrogen in above-ground residues removed for bedding, feed or construction | Thomas et al. (2014) Practice does not occur in New Zealand | 0 |
| FracFUEL (N/kg N excreted) | Livestock nitrogen excretion in excrements burned for fuel | Practice does not occur in New Zealand | 0 |

Some of the parameters used to calculate Nitrous oxide emissions from crop residue returned to soil and emissions from Field burning of agricultural residues are summarised in table A3.1.8. These values are taken from research conducted by Thomas et al. (2008, 2011).

Table A3.1.8	Parameter values for New Zealand’s cropping emissions

| Crop | HI | dmf | AGN | Root Shoot ratio RBG | BGN |
| --- | --- | --- | --- | --- | --- |
| Wheat | 0.41 | 0.86 | 0.005 | 0.1 | 0.009 |
| Barley | 0.46 | 0.86 | 0.005 | 0.1 | 0.009 |
| Oats | 0.30 | 0.86 | 0.005 | 0.1 | 0.009 |
| Maize grain | 0.50 | 0.86 | 0.007 | 0.1 | 0.007 |
| Field seed peas | 0.50 | 0.21 | 0.02 | 0.1 | 0.015 |
| Lentils | 0.50 | 0.86 | 0.02 | 0.1 | 0.015 |
| Peas fresh and processed | 0.45 | 0.86 | 0.03 | 0.1 | 0.015 |
| Potatoes | 0.90 | 0.22 | 0.02 | 0.1 | 0.01 |
| Onions | 0.80 | 0.11 | 0.02 | 0.1 | 0.01 |
| Sweet corn | 0.55 | 0.24 | 0.009 | 0.1 | 0.007 |
| Squash | 0.80 | 0.20 | 0.02 | 0.1 | 0.01 |
| Herbage seeds | 0.11 | 0.85 | 0.015 | 0.1 | 0.01 |
| Legume seeds | 0.09 | 0.85 | 0.04 | 0.1 | 0.01 |
| Brassica seeds | 0.20 | 0.85 | 0.01 | 0.1 | 0.008 |

Source: Thomas et al. (2008, 2011)

Note: 	AGN = above-ground nitrogen residue; BGN = below-ground nitrogen residue; dmf = dry-matter conversion factor; HI = harvest index; RBG = ratio of below-ground residues to the harvest yield.

### A3.1.3	Methodology and data used to allocate livestock excreta
to different hill slopes, for cattle, sheep and deer

The emission factors used to calculate direct N2O emissions from all cattle, sheep and deer were described in detail in chapter 5, section 5.5.2. These pages explained the research behind the revised emission factors and how they were applied to estimate emissions from cattle, sheep and deer.

These revised emission factors are disaggregated by slope (as well as livestock type), and a methodology is used to calculate the amount of nitrogen (in the form of urine or dung) deposited on these different slopes. The steps described below are used to do this.

The nutrient transfer model outlined by Saggar et al. (2015) is used to allocate total dung and urine (calculated elsewhere in the Inventory model) between low, medium and steep slopes. The nutrient transfer model was discussed by the Agriculture Inventory Panel in 2015, which agreed that the methodology used in the nutrient transfer model was appropriate. Beef + Lamb New Zealand provides data (on the topography and number of animals on different farm types) used in the nutrient transfer model.

Dairy excreta is not allocated to different slope types because the Inventory assumes that all dairy cattle graze on flatland. The flatland/low slope emission factor for cattle urine (EF3(PRP FLAT) = 0.0098) is applied to all dairy cattle urine.

##### Step 1: Calculations of total nitrogen excretion rates for each animal category

Total nitrogen excretion rates (Nex) for each animal category are calculated using the methods described in chapter 5, section 5.3.2 of the National Inventory Report (Nitrogen excretion rates for the major livestock categories), and in chapter 5 of Pickering and Gibbs (2019).

##### Step 2: Split of nitrogen between urine and dung

The total Nex calculated in step 1 is split into urine and dung using the method described by Pacheco et al. (unpublished), and section 5.2.4 (beef cattle), section 5.3.5 (sheep) and section 5.4.5 (deer) of Pickering and Gibbs (2019).

##### Step 3: Allocating urine excreta to different hill slopes

The nutrient transfer model (described by Saggar et al. (2015)) uses Beef + Lamb New Zealand data on the proportion of sheep and beef farmland on different hill slopes to allocate urine excreta to different hill slopes. The nutrient transfer model takes into account the preference for animals to spend more time on flatter slopes. Using this model, the proportion of excreta deposited on low slopes is greater than the proportion of low slope land area, because animals spend more time on flatter land.

The equations and variables needed to allocate excreta to different slopes are outlined in table A3.1.9 and figures A3.1.1 and A3.1.2. For example, an area with 60 per cent low slopes and 25 per cent steep slopes will have 72 per cent of livestock urine deposited on low slope land (0.45*60 per cent + 0.45 = 72 per cent), 14 per cent of livestock urine deposited on steep slope land. After the allocation of excreta to low and high slope areas, the remainder (14 per cent) is assumed to be deposited onto medium sloped land.

Because a single dung emission factor (EF3(PRP-DUNG) = 0.0012) is used across all slope categories for cattle, sheep and deer, dung excreta does not need to be allocated to different slopes.

Table A3.1.9 	Allocation of urine deposition to low slope (0–12 degrees) and steep slope (more than 24 degrees), split by the percentage of low slope and steep slope land available

| Allocation to flat land |  |
| --- | --- |
| Percentage of low land area | Fraction urine deposition |
| Less than 1% | 27x |
| 1–5% | 0.27 |
| 5–9% | 0.405 |
| 9–35% | 0.55 |
| 35–85% | (0.45x + 0.45) |
| Greater than 85% | (0.5x + 0.5) |
| Allocation to steep land |  |
| Percentage of steep land area | Fraction urine deposition |
| Less than 1% | 10x |
| 1–20% | 0.10 |
| 20–40% | 0.14 |
| 40–60% | 0.21 |
| 60–85% | 0.28 |
| Greater than 85% | 4.8x – 3.8 |

Figure A3.1.1	Proportion of urine nitrogen (N) applied to low (0–12 degree) slopes using nutrient transfer model (equal proportion line shown in grey for comparison)

Figure A3.1.2	Proportion of urine nitrogen (N) applied to steep (more than 24 degree) slopes using nutrient transfer model (equal proportion line shown in grey for comparison)

Tables A3.1.10, A3.1.11, A3.1.12 and figure A3.1.3 provide examples of how this nutrient allocation methodology uses Beef + Lamb New Zealand data to allocate urine nitrogen (N) to different hill slopes. First, data on the number of sheep, beef cattle and deer in each farm class are used to allocate total urine N (calculated using the methods described in chapter 5, section 5.3.2 of the National Inventory Report) to these different farm classes (tables A.3.1.11 and A.3.1.12).

Table A3.1.10	Share of livestock population, and amount of urine nitrogen (N) deposition in 2019,
by Beef + Lamb New Zealand farm class

| Farm class | Percentage of sheep population on farm class (%) | Amount of sheep urine N on farm class (kg N) | Percentage of beef cattle population on farm class (%) | Amount of beef cattle urine N on farm class (kg N) | Percentage of deer population on farm class (%) | Amount of deer urine N on farm class (kg N) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. South Island High Country | 7.4 | 26,550,296 | 3.5 | 8,048,380 | 14.3 | 2,831,091 |
| 2. South Island Hill Country | 12.1 | 43,132,632 | 6.7 | 15,618,726 | 7.8 | 1,544,421 |
| 3. North Island Hard Hill Country | 17.2 | 61,282,608 | 15.7 | 36,654,059 | 7.7 | 1,534,984 |
| 4. North Island Hill Country | 25.4 | 90,627,947 | 41.3 | 96,240,410 | 32.0 | 6,338,045 |
| 5. North Island Intensive Finishing | 6.8 | 24,437,161 | 11.1 | 25,806,893 | 2.2 | 437,992 |
| 6. South Island Finishing Breeding | 19.1 | 68,037,424 | 14.0 | 32,686,935 | 27.5 | 5,457,651 |
| 7. South Island Intensive Finishing | 10.1 | 36,093,050 | 3.4 | 7,890,935 | 8.5 | 1,685,597 |
| 8. South Island Mixed Finishing | 1.9 | 6,933,601 | 4.2 | 9,858,498 | 0.0 | 0 |
| Total |  | 357,094,719 |  | 232,894,836 |  | 19,829,780 |

Each farm class has a different proportion of land in low, medium and steep slopes, as shown in table A3.1.11. These data are combined with the nutrient transfer methodology to calculate total urine N that is estimated to be deposited on different hill slopes for different animal categories. From this point, direct N2O emissions can be calculated using the emission factors in chapter 5, table 5.5.3.

Table A3.1.11	Proportion of total sheep, beef and deer land on different hill slopes,
by Beef + Lamb New Zealand farm class, for 2018/19 year

| Farm class | Land type by slope |  |  |
| --- | --- | --- | --- |
|  | Flat/low (0–12o slope) (%) | Rolling/medium (12–24o slope) (%) | Steep (>24o slope) (%) |
| 1. South Island High Country | 8.3 | 26.4 | 65.2 |
| 2. South Island Hill Country | 16.2 | 25.8 | 58.0 |
| 3. North Island Hard Hill Country | 8.1 | 35.7 | 56.1 |
| 4. North Island Hill Country | 15.1 | 54.4 | 30.5 |
| 5. North Island Intensive Finishing | 44.2 | 50.7 | 5.1 |
| 6. South Island Finishing Breeding | 36.3 | 47.3 | 16.4 |
| 7. South Island Intensive Finishing | 58.7 | 41.3 | 0.0 |
| 8. South Island Mixed Finishing | 87.1 | 12.9 | 0.0 |
| Total sheep, beef and deer land | 20.8 | 38.4 | 40.8 |

Note:	The percentages may not add up to 100 per cent due to rounding.

Table A3.1.12	Proportion of total sheep, beef and deer urine nitrogen deposited on different hill slopes,
by Beef + Lamb New Zealand farm class, for 2019

| Farm class | Flat/low | Rolling/medium | Steep |
| --- | --- | --- | --- |
| 1. South Island High Country | 0.41 | 0.32 | 0.28 |
| 2. South Island Hill Country | 0.55 | 0.24 | 0.21 |
| 3. North Island Hard Hill Country | 0.41 | 0.39 | 0.21 |
| 4. North Island Hill Country | 0.55 | 0.31 | 0.14 |
| 5. North Island Intensive Finishing | 0.65 | 0.25 | 0.10 |
| 6. South Island Finishing Breeding | 0.61 | 0.29 | 0.10 |
| 7. South Island Intensive Finishing | 0.71 | 0.29 | 0.00 |
| 8. South Island Mixed Finishing | 0.94 | 0.06 | 0.00 |
| Total sheep urine | 0.56 | 0.30 | 0.14 |
| Total beef urine | 0.56 | 0.30 | 0.14 |
| Total deer urine | 0.55 | 0.30 | 0.15 |
| Total sheep, beef and deer urine | 0.56 | 0.30 | 0.14 |

Note:	The proportions may not add up to 1 due to rounding.

Figure A3.1.3	Proportion of land area, excretal nitrogen (N) and nitrous oxide (N2O) emissions
by hill slope category for sheep, beef cattle and deer farms in 2019

## A3.2	Supplementary information for the LULUCF sector

### A3.2.1	Annual land-use change summary

This section contains a summary of the annual land-use change from 1990 to 2019 (see table A3.2.1).

Table A3.2.1 	Annual land-use changes (units in thousand hectares)

### A3.2.2	Natural forest yield tables

This section contains the natural forest yield tables used for this submission.

Table A3.2.2	Pre-1990 natural forest – tall forest yield table (tonnes C ha–1)1

| Year | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 1990 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1991 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1992 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1993 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1994 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1995 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1996 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1997 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1998 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 1999 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2000 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2001 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2002 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2003 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2004 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2005 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2006 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2007 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2008 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2009 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2010 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2011 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2012 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2013 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2014 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2015 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2016 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2017 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2018 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |
| 2019 | 148.6 | 34.9 | 45.9 | 22.7 | 252.1 |

1 Yield table data source: Paul et al. (2020, submitted).

Table A3.2.3	Pre-1990 natural forest – regenerating forest yield table (tonnes C ha–1)1

| Year | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 1990 | 26.1 | 6.2 | 3.1 | 8.8 | 44.1 |
| 1991 | 26.5 | 6.3 | 3.1 | 8.8 | 44.7 |
| 1992 | 27.0 | 6.4 | 3.2 | 8.8 | 45.4 |
| 1993 | 27.4 | 6.5 | 3.3 | 8.8 | 46.0 |
| 1994 | 27.9 | 6.6 | 3.3 | 8.8 | 46.6 |
| 1995 | 28.3 | 6.7 | 3.4 | 8.8 | 47.2 |
| 1996 | 28.8 | 6.8 | 3.4 | 8.8 | 47.9 |
| 1997 | 29.3 | 7.0 | 3.5 | 8.8 | 48.5 |
| 1998 | 29.7 | 7.1 | 3.6 | 8.8 | 49.1 |
| 1999 | 30.2 | 7.2 | 3.6 | 8.8 | 49.7 |
| 2000 | 30.6 | 7.3 | 3.7 | 8.8 | 50.4 |
| 2001 | 31.1 | 7.4 | 3.7 | 8.8 | 51.0 |
| 2002 | 31.5 | 7.5 | 3.8 | 8.8 | 51.6 |
| 2003 | 32.0 | 7.6 | 3.9 | 8.8 | 52.3 |
| 2004 | 32.4 | 7.7 | 3.9 | 8.8 | 52.9 |
| 2005 | 32.9 | 7.8 | 4.0 | 8.8 | 53.5 |
| 2006 | 33.3 | 8.0 | 4.1 | 8.8 | 54.1 |
| 2007 | 33.8 | 8.1 | 4.1 | 8.8 | 54.8 |
| 2008 | 34.2 | 8.2 | 4.2 | 8.8 | 55.4 |
| 2009 | 34.7 | 8.3 | 4.2 | 8.8 | 56.0 |
| 2010 | 35.2 | 8.4 | 4.3 | 8.8 | 56.6 |
| 2011 | 35.6 | 8.5 | 4.4 | 8.8 | 57.3 |
| 2012 | 36.1 | 8.6 | 4.4 | 8.8 | 57.9 |
| 2013 | 36.5 | 8.7 | 4.5 | 8.8 | 58.5 |
| 2014 | 37.0 | 8.8 | 4.5 | 8.8 | 59.1 |
| 2015 | 37.4 | 8.9 | 4.6 | 8.8 | 59.8 |
| 2016 | 37.9 | 9.1 | 4.7 | 8.8 | 60.4 |
| 2017 | 38.3 | 9.2 | 4.7 | 8.8 | 61.0 |
| 2018 | 38.8 | 9.3 | 4.8 | 8.8 | 61.6 |
| 2019 | 39.2 | 9.4 | 4.9 | 8.8 | 62.3 |

1 Yield table data source: Paul et al. (2020, submitted).

Table A3.2.4	Post-1989 natural forest yield table (tonnes C ha–1)1

| Age | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.6 | 0.4 | 0.0 | 0.0 | 2.0 |
| 1 | 2.5 | 0.6 | 0.0 | 0.0 | 3.2 |
| 2 | 3.7 | 0.9 | 0.0 | 0.1 | 4.7 |
| 3 | 5.1 | 1.3 | 0.0 | 0.1 | 6.5 |
| 4 | 6.6 | 1.7 | 0.0 | 0.1 | 8.4 |
| 5 | 8.3 | 2.1 | 0.0 | 0.1 | 10.6 |
| 6 | 10.2 | 2.6 | 0.1 | 0.2 | 13.0 |
| 7 | 12.2 | 3.1 | 0.1 | 0.2 | 15.5 |
| 8 | 14.4 | 3.6 | 0.1 | 0.2 | 18.2 |
| 9 | 16.6 | 4.2 | 0.1 | 0.2 | 21.1 |
| 10 | 18.9 | 4.7 | 0.1 | 0.3 | 24.0 |
| 11 | 21.4 | 5.3 | 0.1 | 0.3 | 27.1 |
| 12 | 23.9 | 6.0 | 0.1 | 0.3 | 30.2 |
| 13 | 26.4 | 6.6 | 0.1 | 0.4 | 33.5 |
| 14 | 29.0 | 7.2 | 0.1 | 0.4 | 36.7 |
| 15 | 31.6 | 7.9 | 0.1 | 0.4 | 40.0 |
| 16 | 34.2 | 8.6 | 0.1 | 0.4 | 43.3 |
| 17 | 36.8 | 9.2 | 0.1 | 0.5 | 46.7 |
| 18 | 39.4 | 9.9 | 0.1 | 0.5 | 49.9 |
| 19 | 42.0 | 10.5 | 0.1 | 0.5 | 53.2 |
| 20 | 44.5 | 11.1 | 0.1 | 0.6 | 56.4 |
| 21 | 47.0 | 11.8 | 0.2 | 0.6 | 59.5 |
| 22 | 49.4 | 12.3 | 0.2 | 0.6 | 62.5 |
| 23 | 51.7 | 12.9 | 0.2 | 0.6 | 65.4 |
| 24 | 53.9 | 13.5 | 0.2 | 0.7 | 68.2 |
| 25 | 56.0 | 14.0 | 0.2 | 0.7 | 70.9 |
| 26 | 57.9 | 14.5 | 0.2 | 0.7 | 73.3 |
| 27 | 59.7 | 14.9 | 0.2 | 0.8 | 75.6 |
| 28 | 61.4 | 15.3 | 0.2 | 0.8 | 77.7 |
| 29 | 62.9 | 15.7 | 0.2 | 0.8 | 79.6 |
| 30 | 64.1 | 16.0 | 0.2 | 0.9 | 81.3 |

1 Yield table source Paul et al. (unpublished(a)).

### A3.2.3	Planted forest yield tables

This section contains the planted forest yield tables used for this submission.

Table A3.2.5	Pre-1990 ‘planted before 1990’ planted forest yield table (tonnes C ha–1)1

| Age | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 0 | 3.53 | 0.81 | 0.30 | 0.00 | 4.64 |
| 1 | 3.67 | 0.86 | 0.30 | 0.01 | 4.83 |
| 2 | 4.08 | 0.99 | 0.30 | 0.03 | 5.40 |
| 3 | 5.34 | 1.41 | 0.30 | 0.14 | 7.18 |
| 4 | 8.11 | 2.18 | 0.31 | 0.43 | 11.02 |
| 5 | 12.31 | 3.21 | 0.39 | 1.21 | 17.11 |
| 6 | 17.56 | 4.40 | 0.67 | 2.54 | 25.17 |
| 7 | 22.69 | 5.48 | 1.74 | 4.49 | 34.39 |
| 8 | 26.95 | 6.37 | 3.60 | 6.81 | 43.72 |
| 9 | 32.36 | 7.51 | 4.54 | 8.14 | 52.55 |
| 10 | 39.58 | 9.02 | 4.39 | 8.50 | 61.47 |
| 11 | 45.91 | 10.31 | 5.54 | 9.21 | 70.96 |
| 12 | 52.44 | 11.64 | 6.66 | 9.69 | 80.43 |
| 13 | 60.55 | 13.30 | 6.48 | 9.53 | 89.86 |
| 14 | 68.56 | 14.94 | 6.72 | 9.48 | 99.70 |
| 15 | 76.47 | 16.56 | 7.24 | 9.48 | 109.74 |
| 16 | 83.42 | 17.96 | 8.81 | 9.64 | 119.82 |
| 17 | 90.81 | 19.46 | 9.77 | 9.61 | 129.65 |
| 18 | 99.10 | 21.17 | 9.33 | 9.36 | 138.96 |
| 19 | 107.36 | 22.87 | 9.00 | 9.15 | 148.37 |
| 20 | 116.22 | 24.72 | 8.51 | 8.81 | 158.24 |
| 21 | 124.96 | 26.56 | 8.24 | 8.54 | 168.29 |
| 22 | 133.67 | 28.42 | 8.04 | 8.30 | 178.42 |
| 23 | 142.46 | 30.29 | 7.80 | 8.06 | 188.61 |
| 24 | 151.03 | 32.14 | 7.78 | 7.89 | 198.84 |
| 25 | 159.31 | 33.93 | 7.89 | 7.77 | 208.89 |
| 26 | 167.37 | 35.69 | 7.98 | 7.65 | 218.67 |
| 27 | 175.31 | 37.42 | 7.99 | 7.54 | 228.25 |
| 28 | 182.78 | 39.04 | 8.32 | 7.51 | 237.65 |
| 29 | 190.20 | 40.66 | 8.61 | 7.48 | 246.94 |
| 30 | 198.14 | 42.40 | 8.40 | 7.35 | 256.28 |
| 31 | 206.16 | 44.18 | 8.20 | 7.22 | 265.76 |
| 32 | 214.00 | 45.92 | 8.03 | 7.11 | 275.05 |
| 33 | 221.72 | 47.62 | 7.87 | 7.00 | 284.21 |
| 34 | 229.45 | 49.37 | 7.76 | 6.90 | 293.47 |
| 35 | 237.07 | 51.09 | 7.72 | 6.80 | 302.67 |
| 36 | 244.54 | 52.80 | 7.74 | 6.70 | 311.77 |
| 37 | 251.88 | 54.50 | 7.81 | 6.61 | 320.79 |
| 38 | 259.13 | 56.18 | 7.92 | 6.53 | 329.75 |
| 39 | 266.27 | 57.85 | 8.08 | 6.45 | 338.63 |
| 40 | 273.30 | 59.51 | 8.26 | 6.37 | 347.43 |
| 41 | 280.20 | 61.14 | 8.46 | 6.30 | 356.10 |
| 42 | 286.92 | 62.74 | 8.69 | 6.24 | 364.58 |
| 43 | 293.42 | 64.30 | 8.93 | 6.17 | 372.82 |
| 44 | 299.70 | 65.83 | 9.19 | 6.10 | 380.81 |
| 45 | 305.80 | 67.32 | 9.46 | 6.02 | 388.60 |
| 46 | 311.74 | 68.79 | 9.73 | 5.94 | 396.20 |
| 47 | 317.55 | 70.25 | 10.01 | 5.86 | 403.67 |
| 48 | 323.26 | 71.68 | 10.29 | 5.78 | 411.02 |
| 49 | 328.83 | 73.09 | 10.57 | 5.70 | 418.20 |
| 50 | 334.27 | 74.48 | 10.85 | 5.62 | 425.21 |
| 51 | 339.57 | 75.84 | 11.13 | 5.54 | 432.08 |
| 52 | 344.76 | 77.19 | 11.41 | 5.46 | 438.81 |
| 53 | 349.83 | 78.51 | 11.68 | 5.38 | 445.39 |
| 54 | 354.80 | 79.82 | 11.94 | 5.31 | 451.85 |
| 55 | 359.66 | 81.11 | 12.19 | 5.23 | 458.18 |
| 56 | 364.42 | 82.38 | 12.44 | 5.16 | 464.39 |
| 57 | 369.08 | 83.63 | 12.68 | 5.09 | 470.48 |
| 58 | 373.66 | 84.87 | 12.92 | 5.02 | 476.47 |
| 59 | 378.15 | 86.09 | 13.15 | 4.96 | 482.34 |
| 60 | 382.61 | 87.30 | 13.37 | 4.89 | 488.16 |

1 Yield table source: Paul and Wakelin (unpublished).

Table A3.2.6	Pre-1990 ‘planted between 1990 and 2009’ planted forest yield table (tonnes C ha–1)1

| Age | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 0 | 2.23 | 0.51 | 0.10 | 0.00 | 2.94 |
| 1 | 2.37 | 0.56 | 0.10 | 0.01 | 3.13 |
| 2 | 2.77 | 0.68 | 0.10 | 0.04 | 3.68 |
| 3 | 4.00 | 1.09 | 0.10 | 0.15 | 5.44 |
| 4 | 6.77 | 1.88 | 0.12 | 0.47 | 9.33 |
| 5 | 11.12 | 2.95 | 0.18 | 1.24 | 15.58 |
| 6 | 16.83 | 4.25 | 0.38 | 2.48 | 24.03 |
| 7 | 23.51 | 5.69 | 0.82 | 4.10 | 34.21 |
| 8 | 30.32 | 7.10 | 1.79 | 6.05 | 45.35 |
| 9 | 37.23 | 8.51 | 3.01 | 7.83 | 56.68 |
| 10 | 44.99 | 10.10 | 3.67 | 8.95 | 67.81 |
| 11 | 53.24 | 11.77 | 4.22 | 9.68 | 79.00 |
| 12 | 61.75 | 13.48 | 4.87 | 10.21 | 90.40 |
| 13 | 70.91 | 15.35 | 5.14 | 10.41 | 101.90 |
| 14 | 80.21 | 17.23 | 5.59 | 10.55 | 113.67 |
| 15 | 90.00 | 19.23 | 5.77 | 10.52 | 125.61 |
| 16 | 100.47 | 21.38 | 5.41 | 10.28 | 137.63 |
| 17 | 110.94 | 23.53 | 5.17 | 10.05 | 149.79 |
| 18 | 121.21 | 25.66 | 5.12 | 9.86 | 161.93 |
| 19 | 131.32 | 27.77 | 5.15 | 9.66 | 173.99 |
| 20 | 141.41 | 29.91 | 5.19 | 9.45 | 186.05 |
| 21 | 151.39 | 32.06 | 5.33 | 9.26 | 198.14 |
| 22 | 161.32 | 34.22 | 5.53 | 9.10 | 210.25 |
| 23 | 170.82 | 36.31 | 6.18 | 9.01 | 222.41 |
| 24 | 180.17 | 38.39 | 6.85 | 8.93 | 234.44 |
| 25 | 189.78 | 40.54 | 7.13 | 8.78 | 246.32 |
| 26 | 199.29 | 42.68 | 7.46 | 8.64 | 258.16 |
| 27 | 208.66 | 44.81 | 7.86 | 8.52 | 269.94 |
| 28 | 217.87 | 46.93 | 8.30 | 8.41 | 281.60 |
| 29 | 226.91 | 49.03 | 8.78 | 8.32 | 293.13 |
| 30 | 235.76 | 51.10 | 9.29 | 8.23 | 304.47 |
| 31 | 244.40 | 53.14 | 9.84 | 8.14 | 315.60 |
| 32 | 252.82 | 55.15 | 10.41 | 8.05 | 326.52 |
| 33 | 261.03 | 57.14 | 11.00 | 7.96 | 337.22 |
| 34 | 269.04 | 59.09 | 11.61 | 7.87 | 347.70 |
| 35 | 276.85 | 61.02 | 12.22 | 7.77 | 357.96 |
| 36 | 284.47 | 62.93 | 12.84 | 7.68 | 368.02 |
| 37 | 291.92 | 64.81 | 13.46 | 7.59 | 377.88 |
| 38 | 299.21 | 66.68 | 14.08 | 7.50 | 387.56 |
| 39 | 306.35 | 68.52 | 14.70 | 7.41 | 397.08 |
| 40 | 313.37 | 70.35 | 15.30 | 7.33 | 406.45 |
| 41 | 320.22 | 72.14 | 15.90 | 7.26 | 415.61 |
| 42 | 326.86 | 73.89 | 16.48 | 7.18 | 424.52 |
| 43 | 333.31 | 75.62 | 17.05 | 7.10 | 433.18 |
| 44 | 339.57 | 77.31 | 17.60 | 7.02 | 441.60 |
| 45 | 345.65 | 78.97 | 18.14 | 6.94 | 449.79 |
| 46 | 351.56 | 80.60 | 18.65 | 6.85 | 457.76 |
| 47 | 357.31 | 82.21 | 19.14 | 6.76 | 465.51 |
| 48 | 362.90 | 83.79 | 19.61 | 6.67 | 473.06 |
| 49 | 368.34 | 85.34 | 20.06 | 6.58 | 480.41 |
| 50 | 373.66 | 86.87 | 20.48 | 6.49 | 487.59 |
| 51 | 378.85 | 88.38 | 20.88 | 6.40 | 494.59 |
| 52 | 383.92 | 89.86 | 21.26 | 6.31 | 501.44 |
| 53 | 388.87 | 91.32 | 21.61 | 6.23 | 508.13 |
| 54 | 393.73 | 92.77 | 21.94 | 6.15 | 514.67 |
| 55 | 398.49 | 94.19 | 22.25 | 6.07 | 521.09 |
| 56 | 403.15 | 95.60 | 22.55 | 5.99 | 527.38 |
| 57 | 407.74 | 96.99 | 22.82 | 5.92 | 533.56 |
| 58 | 412.25 | 98.37 | 23.07 | 5.85 | 539.62 |
| 59 | 416.68 | 99.73 | 23.31 | 5.78 | 545.59 |
| 60 | 421.08 | 101.08 | 23.54 | 5.71 | 551.50 |

1 Yield table source: Paul and Wakelin, unpublished.

Table A3.2.7	Pre-1990 ‘planted between 2009 and 2019’ planted forest yield table (tonnes C ha–1)1

| Age | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.54 | 0.42 | 0.20 | 0.00 | 2.25 |
| 1 | 1.74 | 0.49 | 0.20 | 0.01 | 2.53 |
| 2 | 2.28 | 0.65 | 0.20 | 0.04 | 3.26 |
| 3 | 4.05 | 1.22 | 0.21 | 0.14 | 5.70 |
| 4 | 8.13 | 2.35 | 0.21 | 0.46 | 11.24 |
| 5 | 14.70 | 3.88 | 0.22 | 1.23 | 20.13 |
| 6 | 23.41 | 5.77 | 0.24 | 2.57 | 32.08 |
| 7 | 33.55 | 7.88 | 0.26 | 4.46 | 46.24 |
| 8 | 44.39 | 10.07 | 0.46 | 6.59 | 61.60 |
| 9 | 55.63 | 12.32 | 0.72 | 8.49 | 77.25 |
| 10 | 67.23 | 14.68 | 0.84 | 9.88 | 92.71 |
| 11 | 79.01 | 17.09 | 0.96 | 10.81 | 107.96 |
| 12 | 90.87 | 19.54 | 1.16 | 11.35 | 123.01 |
| 13 | 102.72 | 22.02 | 1.44 | 11.59 | 137.87 |
| 14 | 114.52 | 24.52 | 1.80 | 11.63 | 152.56 |
| 15 | 126.23 | 27.03 | 2.25 | 11.53 | 167.13 |
| 16 | 137.81 | 29.54 | 2.78 | 11.35 | 181.57 |
| 17 | 149.21 | 32.04 | 3.39 | 11.14 | 195.87 |
| 18 | 160.40 | 34.52 | 4.08 | 10.90 | 210.00 |
| 19 | 171.30 | 36.96 | 4.85 | 10.66 | 223.87 |
| 20 | 181.98 | 39.41 | 5.69 | 10.42 | 237.59 |
| 21 | 192.55 | 41.88 | 6.59 | 10.19 | 251.30 |
| 22 | 203.02 | 44.34 | 7.54 | 9.98 | 264.98 |
| 23 | 213.35 | 46.80 | 8.55 | 9.80 | 278.60 |
| 24 | 223.50 | 49.24 | 9.62 | 9.65 | 292.10 |
| 25 | 233.46 | 51.66 | 10.72 | 9.52 | 305.46 |
| 26 | 243.22 | 54.06 | 11.86 | 9.41 | 318.65 |
| 27 | 252.75 | 56.44 | 13.02 | 9.32 | 331.62 |
| 28 | 262.04 | 58.79 | 14.21 | 9.23 | 344.36 |
| 29 | 271.11 | 61.10 | 15.40 | 9.15 | 356.86 |
| 30 | 279.92 | 63.38 | 16.60 | 9.07 | 369.06 |
| 31 | 288.45 | 65.63 | 17.79 | 8.99 | 380.95 |
| 32 | 296.72 | 67.84 | 18.97 | 8.91 | 392.53 |
| 33 | 304.75 | 70.01 | 20.13 | 8.82 | 403.80 |
| 34 | 312.54 | 72.14 | 21.27 | 8.74 | 414.78 |
| 35 | 320.10 | 74.25 | 22.38 | 8.65 | 425.46 |
| 36 | 327.46 | 76.32 | 23.45 | 8.56 | 435.87 |
| 37 | 334.62 | 78.37 | 24.48 | 8.47 | 446.03 |
| 38 | 341.60 | 80.40 | 25.48 | 8.38 | 455.94 |
| 39 | 348.43 | 82.40 | 26.43 | 8.29 | 465.64 |
| 40 | 355.11 | 84.37 | 27.35 | 8.20 | 475.13 |
| 41 | 361.60 | 86.30 | 28.22 | 8.12 | 484.34 |
| 42 | 367.85 | 88.18 | 29.05 | 8.04 | 493.22 |
| 43 | 373.88 | 90.04 | 29.83 | 7.96 | 501.79 |
| 44 | 379.71 | 91.85 | 30.56 | 7.87 | 510.08 |
| 45 | 385.36 | 93.63 | 31.24 | 7.77 | 518.09 |
| 46 | 390.83 | 95.38 | 31.88 | 7.67 | 525.85 |
| 47 | 396.13 | 97.09 | 32.46 | 7.57 | 533.35 |
| 48 | 401.28 | 98.78 | 33.00 | 7.47 | 540.63 |
| 49 | 406.29 | 100.44 | 33.49 | 7.37 | 547.69 |
| 50 | 411.17 | 102.07 | 33.94 | 7.27 | 554.54 |
| 51 | 415.93 | 103.68 | 34.34 | 7.17 | 561.21 |
| 52 | 420.57 | 105.26 | 34.70 | 7.07 | 567.70 |
| 53 | 425.11 | 106.83 | 35.03 | 6.97 | 574.03 |
| 54 | 429.55 | 108.37 | 35.31 | 6.88 | 580.21 |
| 55 | 433.91 | 109.89 | 35.56 | 6.79 | 586.25 |
| 56 | 438.19 | 111.39 | 35.78 | 6.70 | 592.16 |
| 57 | 442.40 | 112.87 | 35.97 | 6.62 | 597.96 |
| 58 | 446.55 | 114.34 | 36.12 | 6.54 | 603.65 |
| 59 | 450.65 | 115.80 | 36.25 | 6.46 | 609.25 |
| 60 | 454.72 | 117.25 | 36.37 | 6.38 | 614.81 |

1 Yield table data source: Paul and Wakelin (unpublished).

Table A3.2.8	Post-1989 planted forest yield table (tonnes C ha–1) 1

| Year | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 0 | 3.43 | 0.81 | 0.40 | 0.00 | 4.74 |
| 1 | 3.61 | 0.88 | 0.40 | 0.01 | 4.99 |
| 2 | 4.23 | 1.05 | 0.40 | 0.04 | 5.91 |
| 3 | 5.30 | 1.59 | 0.41 | 0.16 | 6.96 |
| 4 | 7.68 | 2.81 | 0.43 | 0.48 | 9.27 |
| 5 | 12.98 | 4.14 | 0.46 | 1.26 | 14.30 |
| 6 | 21.05 | 5.43 | 1.48 | 2.88 | 27.18 |
| 7 | 28.69 | 6.80 | 2.52 | 5.21 | 42.70 |
| 8 | 34.33 | 7.92 | 3.74 | 7.71 | 53.89 |
| 9 | 38.71 | 8.76 | 6.56 | 10.08 | 64.33 |
| 10 | 43.12 | 9.66 | 9.05 | 11.63 | 73.69 |
| 11 | 49.27 | 10.95 | 10.04 | 12.05 | 82.55 |
| 12 | 57.05 | 12.56 | 10.03 | 11.89 | 91.74 |
| 13 | 65.86 | 14.37 | 9.65 | 11.54 | 101.56 |
| 14 | 75.11 | 16.26 | 9.29 | 11.22 | 111.94 |
| 15 | 84.65 | 18.19 | 8.91 | 10.89 | 122.64 |
| 16 | 94.37 | 20.17 | 8.52 | 10.57 | 133.57 |
| 17 | 104.19 | 22.18 | 8.17 | 10.26 | 144.71 |
| 18 | 113.92 | 24.18 | 7.97 | 9.96 | 155.91 |
| 19 | 123.48 | 26.16 | 7.96 | 9.69 | 167.06 |
| 20 | 133.00 | 28.15 | 8.09 | 9.40 | 178.09 |
| 21 | 142.41 | 30.14 | 7.23 | 9.09 | 189.03 |
| 22 | 151.82 | 32.15 | 7.10 | 8.80 | 200.02 |
| 23 | 161.31 | 34.19 | 7.23 | 8.55 | 211.17 |
| 24 | 170.77 | 36.24 | 5.70 | 8.29 | 222.38 |
| 25 | 179.96 | 38.25 | 7.23 | 8.29 | 233.59 |
| 26 | 189.02 | 40.24 | 9.54 | 8.13 | 244.76 |
| 27 | 198.12 | 42.26 | 7.49 | 7.75 | 255.84 |
| 28 | 207.12 | 44.27 | 4.83 | 7.56 | 266.87 |
| 29 | 216.01 | 46.27 | 4.54 | 7.36 | 277.83 |
| 30 | 224.72 | 48.25 | 4.81 | 7.24 | 288.65 |
| 31 | 233.23 | 50.20 | 4.75 | 7.12 | 299.29 |
| 32 | 241.54 | 52.12 | 4.72 | 7.00 | 309.74 |
| 33 | 249.65 | 54.01 | 6.38 | 6.95 | 319.99 |
| 34 | 257.56 | 55.88 | 8.30 | 6.95 | 330.03 |
| 35 | 265.29 | 57.72 | 8.77 | 6.92 | 339.87 |
| 36 | 272.82 | 59.53 | 9.21 | 6.84 | 349.51 |
| 37 | 280.17 | 61.32 | 9.63 | 6.76 | 358.94 |
| 38 | 287.35 | 63.07 | 10.06 | 6.68 | 368.19 |
| 39 | 294.37 | 64.81 | 10.49 | 6.59 | 377.27 |
| 40 | 301.32 | 66.54 | 10.92 | 6.50 | 386.26 |

1 Yield table source: Paul et al., unpublished(b)

### A3.2.4	Uncertainty analysis for the LULUCF sector

This section contains the disaggregated uncertainty analysis for the Land Use, Land-Use Change and Forestry (LULUCF) sector. This additional information has been provided as a result of the review of New Zealand’s 2012 inventory submission. One of the recommendations of the review was that New Zealand provides “a detailed disaggregated assessment of uncertainty, as well as the aggregated uncertainty associated with the LULUCF sector, consistent with the Intergovernmental Panel on Climate Change (IPCC) good practice guidance for LULUCF”. This information is now provided in table A3.2.9.

Table A3.2.9	Uncertainty analysis for the LULUCF sector

| IPCC category | Gas | 1990 emissions or removals (kt CO2-e) | 2019 emissions or removals (kt CO2-e) | Activity data uncertainty (%) | Emission factor / estimation parameter uncertainty (biomass) (%) | Emission factor / estimation parameter uncertainty (mineral soil) (%) | Combined uncertainty (%) | Contribution to variance by category in 2019 (%) | Type A sensitivity (%) | Type B sensitivity (%) | Uncertainty in trend in LULUCF emissions introduced by emission factor / estimation parameter uncertainty (%) | Uncertainty in trend in LULUCF emissions introduced by activity data uncertainty (%) | Uncertainty introduced into the trend in total LULUCF emissions (%) | Emission factor quality indicator | Activity data quality indicator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Post-1989 natural forest remaining post-1989 natural forest | CO2 | 0.0 | –156.7 | 8.0 | 44.8 | 10.4 | 45.5 | 0.3 | 0.7 | 0.7 | 0.3 | 0.1 | 0.3 | M | M |
| Land converted to post-89 natural forest | CO2 | 23.7 | –556.6 | 8.0 | 44.8 | 10.4 | 41.2 | 0.8 | 2.4 | 2.3 | 1.1 | 0.3 | 1.1 | M | M |
| Pre-1990 natural forest remaining pre-1990 natural forest | CO2 | –2,730.3 | –2,717.0 | 5.0 | 39.3 | 7.9 | 39.5 | 3.9 | –1.7 | 11.3 | –0.7 | –0.1 | 0.7 | M | M |
| Land converted to pre-1990 natural forest | CO2 | 19.0 | 50.6 | 5.0 | 39.3 | 7.9 | 45.8 | 0.1 | –0.1 | –0.2 | 0.0 | 0.0 | 0.0 | M | M |
| Pre-1990 planted forest remaining pre-1990 planted forest | CO2 | 2,042.3 | –8,351.6 | 5.0 | 27.1 | 12.3 | 201.8 | 61.4 | 44.5 | 34.8 | 12.1 | 3.1 | 12.5 | M | M |
| Land converted to pre-1990 planted forest | CO2 | –22,758.1 | –482.1 | 5.0 | 27.1 | 12.3 | 20.7 | 0.4 | –105.2 | 2.0 | –28.5 | –7.4 | 29.4 | M | M |
| Post-1989 forest remaining post-1989 forest | CO2 | 0.0 | –5,920.9 | 8.0 | 8.9 | 10.4 | 55.9 | 12.1 | 24.7 | 24.7 | 2.2 | 2.8 | 3.6 | M | M |
| Land converted to post-1989 planted forest | CO2 | 78.1 | –4,146.2 | 8.0 | 8.9 | 10.4 | 15.5 | 2.3 | 17.6 | 17.3 | 1.6 | 2.0 | 2.5 | M | M |
| G-WB remaining G-WB | CO2 | 34.1 | 27.6 | 83.0 | 75.0 | 7.3 | 86.8 | 0.1 | 0.0 | –0.1 | 0.0 | 0.1 | 0.1 | M | M |
| Land converted to G-WB | CO2 | 22.6 | –70.0 | 83.0 | 75.0 | 7.3 | 85.0 | 0.2 | 0.4 | 0.3 | 0.3 | 0.5 | 0.6 | M | M |
| G-HP remaining G-HP | CO2 | 1,114.4 | 1,186.3 | 8.0 | 75.0 | 5.8 | 90.4 | 3.9 | 0.4 | –4.9 | 0.3 | 0.0 | 0.3 | M | M |
| Land converted to G-HP | CO2 | –726.9 | 1,426.5 | 8.0 | 75.0 | 5.8 | 28.1 | 1.5 | –9.4 | –5.9 | –7.0 | –1.1 | 7.1 | M | M |
| G-LP remaining G-LP | CO2 | 202.1 | 61.8 | 8.0 | 75.0 | 7.3 | 90.4 | 0.2 | 0.7 | –0.3 | 0.5 | 0.1 | 0.5 | M | M |
| Land converted to G-LP | CO2 | –115.0 | 1,549.0 | 8.0 | 75.0 | 7.3 | 16.9 | 1.0 | –7.0 | –6.5 | –5.2 | –0.8 | 5.3 | M | M |
| Cropland – perennial remaining cropland – perennial | CO2 | 73.5 | 44.7 | 8.0 | 75.0 | 14.1 | 188.6 | 0.3 | 0.2 | –0.2 | 0.1 | 0.0 | 0.1 | M | M |
| Land converted to cropland – perennial | CO2 | 52.2 | 21.0 | 8.0 | 75.0 | 14.1 | 162.6 | 0.1 | 0.2 | –0.1 | 0.1 | 0.0 | 0.1 | M | M |
| Cropland – annual remaining cropland – annual | CO2 | 269.1 | 272.4 | 8.0 | 75.0 | 9.7 | 90.4 | 0.9 | 0.1 | –1.1 | 0.1 | 0.0 | 0.1 | M | M |
| Land converted to cropland – annual | CO2 | 73.6 | 41.3 | 8.0 | 75.0 | 9.7 | 23.7 | 0.0 | 0.2 | –0.2 | 0.1 | 0.0 | 0.1 | M | M |
| Wetland – open water remaining wetlands – open water | CO2 | 0.0 | 0.0 | 33.0 | 0.0 | 90.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to wetland – open water | CO2 | –20.3 | –3.0 | 33.0 | 0.0 | 90.0 | 139.5 | 0.0 | –0.1 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Wetland – vegetative non-forest remaining wetland – vegetative non-forest | CO2 | 9.4 | 17.9 | 33.0 | 75.0 | 12.3 | 95.9 | 0.1 | 0.0 | –0.1 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to wetland – vegetative non-forest | CO2 | 0.2 | –1.6 | 33.0 | 75.0 | 12.3 | 49.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Settlements remaining settlements | CO2 | 67.2 | 75.7 | 22.0 | 75.0 | 95.0 | 92.6 | 0.3 | 0.0 | –0.3 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to settlements | CO2 | 7.3 | 37.0 | 22.0 | 75.0 | 95.0 | 73.9 | 0.1 | –0.1 | –0.2 | –0.1 | 0.0 | 0.1 | M | M |
| Other land remaining other land | CO2 | 0.0 | 0.0 | 22.0 | 75.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to other land | CO2 | 12.6 | 48.6 | 22.0 | 75.0 | 70.7 | 28.6 | 0.1 | –0.1 | –0.2 | –0.1 | 0.0 | 0.1 | M | M |
| Harvested wood products | CO2 | –2,072.9 | –10,067.3 | 15.0 | 67.4 | – | 68.2 | 25.0 | 32.0 | 41.9 | 21.6 | 6.8 | 22.6 | M | M |
| LULUCF CH4 (CO2-e) | CO2-e | 93.5 | 76.6 | 30.0 | 48.3 | – | 56.9 | 0.2 | 0.1 | –0.3 | 0.1 | 0.1 | 0.1 | R | R |
| LULUCF N2O (CO2-e) | CO2-e | 214.0 | 110.9 | 30.0 | 48.3 | – | 17.4 | 0.1 | 0.6 | –0.5 | 0.3 | 0.2 | 0.4 | R | R |

Note:	G-HP = high producing grassland; G-LP = low producing grassland; G-WB = grassland with woody biomass; M = measurements; R = national referenced information.

### A3.2.5	LUCAS data management system

The Land Use Carbon Analysis System (LUCAS) data management system stores, manages and archives data for international greenhouse gas reporting for the LULUCF sector. This system is used for managing the land use spatial databases, plot and reference data, and for combining the two sets of data to calculate the numbers required for reporting under the United Nations Framework Convention on Climate Change (UNFCCC) and the Kyoto Protocol (see figure A3.2.1).

The data collected are stored and manipulated within three systems: the Geospatial System, the Gateway, and the Calculation and Reporting Application (CRA).

The main objectives of these systems are to:

provide a transparent system for data storage and carbon calculations

provide a repository for the versioning and validation of plot measurements and land-use data

calculate carbon stocks, emissions and removals per hectare for land uses and carbon pools based on the plot and spatial data collected

calculate biomass burning emissions by land use based on area and emission factors stored in the Gateway

produce the outputs required for the LULUCF sector reporting under the UNFCCC and the Kyoto Protocol

archive all inputs and outputs used in reporting.

The module ‘joint calculations’ refers to the process New Zealand uses to estimate national average carbon values by carbon pool for each land-use category and subcategory.

The joint calculation process is performed within the CRA. Within the joint calculations interface, the user selects the appropriate area data and emission factors. The results of the calculations are carbon gains, losses and net change for all land-use subcategories (whether in a conversion state or land remaining land), by year and by carbon pool.

Figure A3.2.1	New Zealand’s LUCAS data management system

Note:	IPCC = Intergovernmental Panel on Climate Change; LULUCF = Land Use, Land-Use Change and Forestry; LUM = land use map; QA/QC = quality assurance/quality control. Joint calculations are described below.

#### Geospatial System

The Geospatial System consists of hardware and specific applications designed to meet LULUCF reporting requirements. Most of the hardware comprises servers for spatial database storage, management, versioning and running web-mapping applications. The core components of the Geospatial System are outlined in figure A3.2.2.

Figure A3.2.2	New Zealand’s Geospatial System components

Note:	Blue indicates land use mapping data flow. LUCAS = Land Use Carbon Analysis System; 
LUM = land use map.

#### Land use mapping functionality

The land use mapping (LUM) functionality of the Geospatial System largely involves the editing and maintenance of time-stamped land use mapping data. The five main components within the LUM functionality are:

LUM Import/Export Tool – providing functionality to manage importing LUM data in to and exporting LUM data out of the database

LUM Attribute Tool – an extension to the standard ArcGIS Desktop software that facilitates maintenance of and updates to the LUM data by external contractors

LUM Database – a non-versioned geographic information system (GIS) database for interim LUM data

Spatial Gateway Tool – used to validate version data from the LUM database prior to loading into the LUCAS GIS database. Validation business rules are stored in the Spatial Gateway database

LUCAS Database – storing versions of LUM used to derive land-use change reporting.

#### LUCAS Management Studio

The LUCAS Management Studio (see figure A3.2.3) is the package of applications used to store activity data and calculate and report New Zealand’s emissions and removals for LULUCF. The LUCAS Gateway is a data warehouse with the purpose of storing, versioning and validating activity data and emission factors. The CRA sources all data from the Gateway. It then calculates and outputs New Zealand’s emissions and removals for LULUCF for land remaining land and land converted to another land use by pool and year.

Figure A3.2.3 	LUCAS Management Studio

#### LUCAS Gateway

The LUCAS Gateway enables the storage of activity data such as field plot data, land use area, biomass burning and other data needed by the CRA, such as IPCC defaults.

The LUCAS Gateway provides a viewing, querying and editing interface to the source (plot, land use area, carbon and non-carbon) data. It also stores any published or saved results from running the CRA.

All activity data and emission factors are stored within the Gateway database (see figure A3.2.4). It contains the following main components.

- A data and results layer contains all activity data (natural, planted forest, soils, default carbon, non-carbon, land use areas, land-use change and reference tables). The user has the ability to create a ‘snapshot’ in time (a data set archiving system) of the data held in the Gateway. This enables users of the CRA to select from a range of data snapshots and ensures past results can be replicated over time.

- A validation layer allows users to judge the suitability of data for use in the CRA calculations, subsequent to passing primary validation. Where records are deemed not acceptable for use within published reports, they are tagged as ‘invalid’ in the LUCAS Gateway database.

- An audit trail provides a history of any changes to the database tables within the Gateway.

- Versioning at a number of levels ensures any changes to data, schema or the database itself are logged and versioned, while providing the user with the ability to track what changes have been applied and roll back to a previous version if required. The results of saved or published reports within the CRA are also stored within the Gateway for repeatability and reference.

- Primary data validation, both during data capture and during import of the data into the Gateway, ensures only data that has passed acceptability criteria is available for a publishable CRA run.

- Hosting and application support provides hosting services, system security, backup and restore, daily maintenance and monitoring for the Gateway and CRA.

Figure A3.2.4	LUCAS Gateway database

#### Calculation and Reporting Application

The CRA enables users to import carbon and non-carbon data from the Gateway and, by running the various modules, determine emissions and removals by New Zealand’s Forest land, Cropland, Grassland and other land use types. This information, combined with land area data, enables New Zealand to meet its reporting requirements under the UNFCCC and the Kyoto Protocol.

The CRA allows for the inclusion of other data sets, models and calculations without the complete redesign of the applications. All models, data and results are versioned, and the CRA allows the user to alter specific key values within a model or calculation (parameters) without the intervention of a programmer or technical support officer. The CRA is deployed as a client-based application that sources the required data from the Gateway.

The CRA comprises four modules: natural forest, soils, non-carbon and joint calculations. Any of these modules can be run independently or as a group. The results are provided as ‘views’ to the user at the completion of the run.

To activate a module, the user selects the module to run within the CRA, the version of the data set to be used, the model version and other calculation parameters. The natural forest and soil carbon modules use R statistical language as the base program language, while the non-carbon module and joint calculations module are developed in the programming language C Sharp (C#).

Within the joint calculations module, the user has the option of using the carbon results from running the modules or using default carbon estimates (based on published reports) stored within the Gateway. The joint calculations module combines the carbon estimates with the land use area to calculate carbon stock and change following the methodology set out in section 2.3 of volume 4 of the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2006). The results represent carbon stock and change for every ‘from’ and ‘to’ land use combination outlined by the IPCC since 1990.

On completion of running a module, the results can be saved or published back to the Gateway. This provides a versioned and auditable record of the results used for reporting. If the results are saved or published, other information such as the time created, the user’s identification and the module-particular parameters that were used are also saved for tracking and audit control.

The CRA is maintained and supported by Interpine Innovation, a New Zealand-based company that specialises in forestry inventories and related information technology development. Interpine Innovation also provides support services, such as database and application backups, day-to-day issue resolution and enhancement projects to the Gateway or CRA as required.

Any changes to the data or table structure within the Gateway, or to the people accessing the Gateway or CRA, are tracked via audit logs. For any changes to the data within the Gateway, the person making the change, the date, the reason for change and the version are logged and reports are made available to users for review.

#### Quality control management for implementing planned improvements

In 2020 further quality control processes were introduced and formalised for introducing improvements to the national inventory report. This was done to help manage the large number of improvements to the LULUCF sector that were made for the 2021 submission. The quality control process is described in figure A3.2.5.

Figure A3.2.5	Quality control procedure for implementing improvements

#### Document management

All reference material, including scientific reports containing information on methodologies or emission factors used in the production of the LULUCF and Kyoto Protocol estimates, is archived on the Ministry for the Environment’s document management store, Te Puna.

The emission factors and area estimates for published runs are also archived within the Gateway and can be accessed via the Gateway or the CRA.

## Annex 3: References

Some references may be downloaded directly from the following webpage: www.mpi.govt.nz/news-and-resources/statistics-and-forecasting/greenhouse-gas-reporting/agriculture-greenhouse-gas-inventory-reports.

The Ministry for Primary Industries is progressively making reports used for the inventory available on this page, provided copyright permits.

Carran RA, Theobold PW, Evans JP. 1995. Emissions of nitrous oxide from some grazed pasture soils. New Zealand and Australian Journal of Soil Research 33: 341–352.

de Klein CAM, Barton L, Sherlock RR, Li Z, Littlejohn RP. 2003. Estimating a nitrous oxide emission factor for animal urine from some New Zealand pastoral soils. Australian Journal of Soil Research 41: 381–399.

Fick J, Saggar S, Hill J, Giltrap D. 2011. Poultry Management in New Zealand: Production, Manure Management and Emissions Estimations for the Commercial Chicken, Turkey, Duck and Layer Industries within New Zealand. Report prepared for the Ministry of Agriculture and Forestry by Poultry Industry Association, Egg Producers Association, Landcare Research and Massey University. Wellington: Ministry of Agriculture and Forestry.

Hill J. 2012. Recalculate Pork Industry Emissions Inventory. Report prepared for the Ministry of Agriculture and Forestry by Massey University and the New Zealand Pork Industry Board. Wellington: Ministry of Agriculture and Forestry.

IPCC. 1996. Houghton JT, Meira Filho LG, Lim B, Treanton K, Mamaty I, Bonduki Y, Griggs DJ, Callender BA (eds). IPCC/OECD/IEA. Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Bracknell: United Kingdom Meteorological Office.

IPCC. 2000. Penman J, Kruger D, Galbally I, Hiraishi T, Nyenzi B, Emmanuel S, Buendia L, Hoppaus R, Martinsen T, Meijer J, Miwa K, Tanabe K (eds). Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4. Agriculture, Forestry and Other Land Use. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

Kelliher FM, de Klein CAM. Unpublished. Review of New Zealand’s Fertiliser Nitrous Oxide Emission Factor (EF1) Data. Report prepared for the Ministry for the Environment by Landcare Research and AgResearch in 2006.

Lassey K. 2011. Methane Emissions and Nitrogen Excretion Rates for New Zealand Goats. Report for the Ministry of Agriculture and Forestry, National Institute of Water and Atmospheric Research. Wellington: National Institute of Water and Atmospheric Research.

Luo J, van der Weerden T, Hoogendoorn C, de Klein C. 2009. Determination of the N2O Emission Factor for Animal Dung Applied in Spring in Three Regions of New Zealand. Report prepared for the Ministry of Agriculture and Forestry by AgResearch. Wellington: Ministry of Agriculture and Forestry.

Ministry for the Environment. 2010. New Zealand’s Greenhouse Gas Inventory 1990–2008. Wellington: Ministry for the Environment.

Muller C, Sherlock RR, Williams PH. 1995. Direct field measurements of nitrous oxide emissions from urine-affected and urine-unaffected pasture in Canterbury. In: Proceedings of the Workshop on Fertilizer Requirements of Grazed Pasture and Field Crops: Macro and Micronutrients. Currie LD, Loganathan P (eds). Occasional Report No. 8. Palmerston North: Massey University. pp 243–234.

Paul TSH, Beets PN, Kimberley MO. Unpublished(a). Carbon stocks in New Zealand’s Post-1989 Natural Forest – Analysis of the 2018/2019 forest inventory data. Contract report prepared for the Ministry for the Environment by the New Zealand Forest Research Institute Ltd (trading as Scion) in 2020.

Paul TSH, Kimberley MO, Beets PN. 2020. Natural Forests in New Zealand – a large terrestrial carbon pool in a national state of equilibrium. Manuscript submitted for publication.

Paul TSH, Kimberley MO, Dodunski C. Unpublished(b). The 2019 NFI plot analysis Yield tables and carbon stocks at measurement. Contract report prepared for the Ministry for the Environment by New Zealand Forest Research Institute Ltd (trading as Scion) in 2020.

Paul TSH, Wakelin SJ. Unpublished Yield tables and approach for estimating historic carbon stocks in pre-1990 planted forests for greenhouse gas inventory reporting. Contract report prepared for the Ministry for the Environment by New Zealand Forest Research Institute Ltd (trading as Scion) in 2020.

Pickering A, Fick JM. 2015. Detailed methodologies for agricultural greenhouse gas emission calculation. Version 3. Wellington: Ministry for Primary Industries. (Refer to: http://mpi.govt.nz/news-and-resources/statistics-and-forecasting/greenhouse-gas-reporting.)

Sherlock RR, Jewell P, Clough T. 2008. Review of New Zealand Specific FracGASM and FracGASF Emissions Factors. Report prepared for the Ministry of Agriculture and Forestry by Landcare Research and AgResearch. Wellington: Ministry of Agriculture and Forestry.

Suttie J. 2012. Report to the Deer Industry New Zealand: Estimation of Deer Population and Productivity Data 1990 to 2012. Dunedin: Suttie Consulting Limited.

Thomas S, Hume E, Fraser T, Curtin, D. 2011. Factors and Activity Data to Estimate Nitrous Oxide Emissions from Cropping Systems, and Stubble and Tussock Burning. Report prepared for Ministry of Agriculture and Forestry by Plant and Food Research. Wellington: Ministry of Agriculture and Forestry.

Thomas S, Wallace D, Beare M. 2014. Pasture Renewal Activity Data and Factors for New Zealand. Report prepared for the Ministry for Primary Industries by Plant and Food Research. Wellington: Ministry for Primary Industries.

Thomas SM, Fraser T, Curtin D, Brown H, Lawrence E. 2008. Review of Nitrous Oxide Emission Factors and Activity Data for Crops. Report prepared for the Ministry of Agriculture and Forestry by Plant and Food Research. Wellington: Ministry of Agriculture and Forestry.

Thomas SM, Ledgard SF, Francis GS. Unpublished. Appropriateness of IPCC Default Values for Estimating New Zealand’s Indirect Nitrous Oxide Emissions. Report prepared for the Ministry of Agriculture and Forestry in 2003.

Thomas SM, Ledgard SF, Francis GS. 2005. Improving estimates of nitrate leaching for quantifying New Zealand’s indirect nitrous oxide emissions. Nutrient Cycling in Agroecosystems 73: 213–226.

van der Weerden T, Cox N, Luo J, Di HJ, Podolyan A, Phillips RL, Saggar S, de Klein CAM, Ettema P, Rys G. 2016. Refining the New Zealand nitrous oxide emission factor for urea fertiliser and farm dairy effluent. Agriculture Ecosystems & Environment 222: 133–137.

# Annex 4: Methodology and data collection for estimating emissions from fossil fuel combustion

New Zealand emission factors are based on gross calorific value. Energy activity data and emission factors in New Zealand are conventionally reported in gross (higher heating value) terms, with some minor exceptions. The convention adopted by New Zealand to convert gross calorific value to net calorific value follows the Organisation for Economic Co-operation and Development and International Energy Agency assumptions:

Net calorific value = 0.95 × gross calorific value for coal and liquid fuels

Net calorific value = 0.90 × gross calorific value for gas

Net calorific value = 0.80 × gross calorific value for wood

Emission factors for gas, coal, biomass and liquid fuels used by New Zealand are shown in tables A4.1–A4.4. Where Intergovernmental Panel on Climate Change (IPCC) default emission factors are used, a net-to-gross factor as above is used to account for New Zealand activity data representing gross energy figures:

Gross EF = Net EF × Factor

Table A4.1 	Gross carbon dioxide emission factors used for New Zealand’s energy sector in 2019

|  | Emission factor (t CO2/TJ) | Source |
| --- | --- | --- |
| Gas |  |  |
| Maui | 52.26 | 1 |
| Kapuni | 53.57 | 1 |
| McKee | 54.42 | 2 |
| Kaimiro | 63.51 | 2 |
| Ngatoro | 61.81 | 2 |
| TAWN | 52.72 | 2 |
| Mangahewa | 54.34 | 2 |
| Turangi | 55.05 | 2 |
| Pohokura | 54.84 | 1 |
| Rimu/Kauri | 51.46 | 2 |
| Maari | 54.56 | 2 |
| Weighted average | 54.1 |  |
| Kapuni LTS | 86.31 | 1 |
| Methanol – Mixed Feed – to 94 | 62.44 | 2 |
| Methanol – LTS – to 94 | 83.97 | 2 |
| Liquid fuels |  |  |
| Crude oil | 69.67 | 4 |
| Regular petrol | 66.77 | 3 |
| Petrol – premium | 66.95 | 3 |
| Diesel (10 parts (sulphur) per million) | 69.45 | 3 |
| Jet kerosene | 68.33 | 3 |
| Av gas | 65.89 | 3 |
| LPG | 60.79 | 6 |
| Heavy fuel oil | 73.33 | 3 |
| Light fuel oil | 73.02 | 3 |
| Bitumen (asphalt) | 76.43 | 3 |
| Biomass |  |  |
| Biogas | 49.17 | 4 |
| Wood (industrial) | 89.47 | 4 |
| Bioethanol | 64.20 | 5 |
| Biodiesel | 67.26 | 4 |
| Wood (residential) | 85.8 | 4 |
| Coal |  |  |
| All sectors excl. electricity (sub-bituminous) | 91.99 | 6 |
| All sectors (bituminous) | 89.13 | 6 |
| All sectors (lignite) | 93.11 | 6 |

1.	New Zealand Emissions Trading Scheme data.

2.	Specific gas field operator.

3.	Refining NZ.

4.	IPCC Guidelines (2006).

5.	New Zealand Energy Information Handbook (Eng et al., 2008).

6.	Review of Default Emission Factors in Draft Stationary Energy and Industrial Processes Regulations: Coal (CRL Energy, 2009).

Table A4.2 	Consumption-weighted average emission factors used for New Zealand’s
sub-bituminous coal-fired electricity generation for 1990 to 2019

| Year | Emission factor (t CO2/TJ) |
| --- | --- |
| 1990 | 91.20 |
| 1991 | 91.24 |
| 1992 | 91.29 |
| 1993 | 91.33 |
| 1994 | 91.38 |
| 1995 | 91.42 |
| 1996 | 91.47 |
| 1997 | 91.51 |
| 1998 | 91.56 |
| 1999 | 91.60 |
| 2000 | 91.64 |
| 2001 | 91.69 |
| 2002 | 91.73 |
| 2003 | 91.78 |
| 2004 | 91.82 |
| 2005 | 91.87 |
| 2006 | 91.91 |
| 2007 | 92.43 |
| 2008 | 92.31 |
| 2009 | 92.39 |
| 2010 onwards | 92.20 |

Table A4.3 	Methane emission factors used for New Zealand’s energy sector for 1990 to 2019

|  | Emission factor (t CH4/PJ) | Source |
| --- | --- | --- |
| Natural gas |  |  |
| Electricity industries | 0.9 | IPCC 2006 (table 2.2) |
| Commercial | 4.50 | IPCC 2006 (table 2.4) |
| Residential | 4.50 | IPCC 2006 (table 2.5) |
| Domestic transport (CNG) | 82.80 | IPCC 2006 (table 3.2.2) |
| Other stationary (mainly industrial) | 0.9 | IPCC 2006 (table 2.3) |
| Liquid fuels |  |  |
| Stationary sources |  |  |
| Electricity – residual oil | 2.85 | IPCC 2006 (table 2.2) |
| Industrial (including refining) – residual oil | 2.85 | IPCC 2006 (table 2.3) |
| Industrial – LPG | 0.95 | IPCC 2006 (table 2.3) |
| Commercial – residual oil | 9.50 | IPCC 2006 (table 2.4) |
| Commercial – distillate oil | 9.50 | IPCC 2006 (table 2.4) |
| Commercial – LPG | 4.75 | IPCC 2006 (table 2.4) |
| Residential – distillate oil | 9.50 | IPCC 2006 (table 2.5) |
| Residential – LPG | 4.75 | IPCC 2006 (table 2.5) |
| Agriculture – stationary | 2.85 | IPCC 2006 (table 2.5) |
| Mobile sources |  |  |
| LPG | 58.9 | IPCC 2006 (table 3.2.2) |
| Petrol | 28.05 | IPCC 2006 (table 3.2.2) |
| Diesel | 3.71 | IPCC 2006 (table 3.2.2) |
| Navigation (fuel oil and diesel) | 6.65 | IPCC 2006 (table 3.5.3) |
| Aviation fuel/kerosene | 0.48 | IPCC 2006 (table 3.6.5) |
| Coal |  |  |
| Electricity generation | 0.95 | IPCC 2006 (table 2.2) |
| Industry | 9.50 | IPCC 2006 (table 2.3) |
| Commercial | 9.50 | IPCC 2006 (table 2.4) |
| Residential | 285.00 | IPCC 2006 (table 2.5) |
| Biomass |  |  |
| Wood/wood waste | 24 | IPCC 2006 (table 2.3) |
| Wood – fireplaces | 240.00 | IPCC 2006 (table 2.5) wood – residential |
| Bioethanol | 18.00 | IPCC 2006 (table 3.2.2) – ethanol, cars, Brazil |
| Biodiesel | 18.00 | IPCC 2006 (table 3.2.2) – ethanol, cars, Brazil |
| Gas biomass | 0.9 | IPCC 2006 (table 2.2) |

Table A4.4 	Nitrous oxide emission factors used for New Zealand’s energy sector for 1990 to 2019

|  | Emission factor (t N2O/PJ) | Source |
| --- | --- | --- |
| Natural gas |  |  |
| Electricity generation | 0.09 | IPCC 2006 (table 2.2) |
| Commercial | 0.09 | IPCC 2006 (table 2.4) |
| Residential | 0.09 | IPCC 2006 (table 2.5) |
| Domestic transport (CNG) | 2.70 | IPCC 2006 (table 3.2.2) |
| Other stationary (mainly industrial) | 0.09 | IPCC 2006 (table 2.3) |
| Liquid fuels |  |  |
| Stationary sources |  |  |
| Electricity – residual oil | 0.57 | IPCC 2006 (table 2.2) |
| Electricity – distillate oil | 0.57 | IPCC 2006 (table 2.2) |
| Industrial (including refining) – residual oil | 0.57 | IPCC 2006 (table 2.2) |
| Industrial – distillate oil | 0.57 | IPCC 2006 (table 2.3) |
| Commercial – residual oil | 0.57 | IPCC 2006 (table 2.4) |
| Commercial – distillate oil | 0.57 | IPCC 2006 (table 2.4) |
| Residential (all oil) | 0.57 | IPCC 2006 (table 2.5) |
| LPG (all uses) | 0.095 | IPCC 2006 (tables 2.2–2.5) |
| Agriculture – stationary | 0.38 | Tier 2, diesel engines – agriculture |
| Mobile sources |  |  |
| LPG | 0.19 | IPCC 2006 (table 3.22) |
| Petrol | 7.6 | IPCC 2006 (table 3.2.2) |
| Diesel | 3.71 | IPCC 2006 (table 3.2.2) |
| Fuel oil (ships) | 1.90 | IPCC 2006 (table 3.5.3) |
| Aviation fuel/kerosene | 1.90 | IPCC 2006 (table 3.6.5) |
| Coal |  |  |
| Electricity generation | 1.43 | IPCC 2006 (table 2.2) |
| Industry | 1.43 | IPCC 2006 (table 2.3) |
| Commercial | 1.43 | IPCC 2006 (table 2.4) |
| Residential | 1.43 | IPCC 2006 (table 2.5) |
| Biomass |  |  |
| Wood (all uses) | 3.20 | IPCC 2006 (table 2.5) wood/wood waste |
| Gas biomass | 0.09 | IPCC 2006 (table 2.5) |

## A4.1	Emissions from liquid fuels

### A4.1.1	Activity data and uncertainties

The Delivery of Petroleum Fuels by Industry Survey is conducted by the Ministry of Business, Innovation and Employment (MBIE). Because it is a census, there is no sampling error. The only possible sources of error are non-sampling errors (such as respondent error and processing error). The 2019 statistical difference for liquid fuels in the balance table of the publication Energy in New Zealand (MBIE, 2020) was 1.4 per cent. This is used as the activity data uncertainty for liquid fuels in 2019.

### A4.1.2	Emission factors and uncertainties

The carbon dioxide (CO2) emission factors are described in table A4.1. Table A4.5 shows a complete time series of gross calorific values, while table A4.6 shows a complete time series of carbon content of liquid fuels. This information is supplied by Refining New Zealand Ltd and is used in the calculation of annual emission factors for liquid fuels.

A 2009 consultant report (Hale and Twomey, unpublished) to the Ministry for the Environment estimates the uncertainty of CO2 emission factors for liquid fuels at ±0.5 per cent. The uncertainty for methane (CH4) and nitrous oxide (N2O) emission factors is ±50.0 per cent because almost all emission factors are IPCC defaults.

Table A4.5 	Gross calorific values (MJ/kg) for liquid fuels for 1990 to 2019

|  | Premium petrol | Regular petrol | Diesel | Jet kerosene | Heavy fuel oil | Light fuel oil | Bitumen (asphalt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 47.24 | 47.22 | 45.76 | 46.37 | 43.07 | 44.12 | 41.30 |
| 1991 | 47.17 | 47.17 | 45.73 | 46.38 | 43.02 | 44.07 | 41.30 |
| 1992 | 47.18 | 47.14 | 45.75 | 46.41 | 43.03 | 44.14 | 41.30 |
| 1993 | 47.09 | 47.14 | 45.74 | 46.36 | 43.01 | 44.13 | 41.31 |
| 1994 | 47.10 | 47.11 | 45.75 | 46.34 | 43.03 | 44.16 | 41.30 |
| 1995 | 47.07 | 47.14 | 45.59 | 46.31 | 43.03 | 44.01 | 41.30 |
| 1996 | 46.91 | 47.14 | 45.54 | 46.26 | 43.00 | 43.98 | 41.30 |
| 1997 | 46.93 | 47.17 | 45.58 | 46.32 | 42.92 | 43.92 | 41.30 |
| 1998 | 46.89 | 47.12 | 45.64 | 46.27 | 43.06 | 44.02 | 41.27 |
| 1999 | 46.92 | 47.13 | 45.56 | 46.29 | 43.09 | 43.93 | 41.28 |
| 2000 | 46.91 | 47.12 | 45.58 | 46.22 | 43.07 | 43.90 | 41.27 |
| 2001 | 46.92 | 47.15 | 45.64 | 46.25 | 43.08 | 43.96 | 41.27 |
| 2002 | 46.90 | 47.16 | 45.62 | 46.29 | 43.03 | 43.84 | 41.26 |
| 2003 | 46.87 | 47.11 | 45.61 | 46.23 | 43.06 | 43.79 | 41.27 |
| 2004 | 46.91 | 47.10 | 45.59 | 46.25 | 43.04 | 43.90 | 41.30 |
| 2005 | 46.95 | 47.10 | 45.73 | 46.28 | 43.11 | 43.94 | 41.30 |
| 2006 | 46.97 | 47.09 | 45.79 | 46.23 | 42.93 | 43.68 | 41.30 |
| 2007 | 46.97 | 47.10 | 45.77 | 46.23 | 42.97 | 43.72 | 41.30 |
| 2008 | 46.93 | 47.06 | 45.72 | 46.19 | 42.86 | 43.72 | 41.30 |
| 2009 | 46.95 | 47.03 | 45.72 | 46.17 | 42.89 | 43.75 | 41.29 |
| 2010 | 46.96 | 47.03 | 45.69 | 46.17 | 42.95 | 43.70 | 41.29 |
| 2011 | 46.96 | 47.04 | 45.69 | 46.19 | 42.89 | 43.72 | 41.27 |
| 2012 | 46.98 | 47.03 | 45.66 | 46.18 | 43.03 | 43.71 | 41.27 |
| 2013 | 46.99 | 47.05 | 45.71 | 46.23 | 43.05 | 43.84 | 41.26 |
| 2014 | 46.95 | 47.02 | 45.71 | 46.23 | 42.94 | 43.73 | 41.26 |
| 2015 | 46.96 | 47.03 | 45.67 | 46.19 | 42.98 | 43.70 | 41.28 |
| 2016 | 46.92 | 46.99 | 45.79 | 46.29 | 42.99 | 43.91 | 41.29 |
| 2017 | 46.91 | 47.00 | 45.77 | 46.27 | 42.97 | 43.71 | 41.27 |
| 2018 | 46.90 | 46.99 | 45.77 | 46.28 | 42.93 | 43.69 | 41.28 |
| 2019 | 46.91 | 46.97 | 45.75 | 46.25 | 42.98 | 43.67 | 41.28 |

Table A4.6 	Carbon content (per cent mass) for liquid fuels for 1990 to 2019

|  | Premium petrol | Regular petrol | Diesel | Jet kerosene | Heavy fuel oil | Light fuel oil | Bitumen (asphalt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 84.87 | 84.92 | 86.28 | 85.92 | 86.22 | 86.67 | 86.57 |
| 1991 | 85.04 | 85.04 | 86.33 | 85.89 | 86.26 | 86.30 | 86.57 |
| 1992 | 85.03 | 85.13 | 86.29 | 85.84 | 86.25 | 86.18 | 86.57 |
| 1993 | 85.25 | 85.13 | 86.32 | 85.94 | 86.27 | 86.20 | 86.56 |
| 1994 | 85.21 | 85.19 | 86.30 | 85.99 | 86.25 | 86.13 | 86.57 |
| 1995 | 85.30 | 85.13 | 86.63 | 86.05 | 86.25 | 86.39 | 86.57 |
| 1996 | 85.66 | 85.13 | 86.73 | 86.16 | 86.28 | 86.45 | 86.57 |
| 1997 | 85.63 | 85.04 | 86.64 | 86.04 | 86.35 | 86.55 | 86.58 |
| 1998 | 85.72 | 85.17 | 86.52 | 86.14 | 86.22 | 86.39 | 86.63 |
| 1999 | 85.65 | 85.15 | 86.69 | 86.10 | 86.20 | 86.53 | 86.63 |
| 2000 | 85.67 | 85.16 | 86.64 | 86.25 | 86.22 | 86.58 | 86.63 |
| 2001 | 85.65 | 85.09 | 86.53 | 86.18 | 86.21 | 86.49 | 86.64 |
| 2002 | 85.68 | 85.06 | 86.57 | 86.10 | 86.25 | 86.68 | 86.66 |
| 2003 | 85.76 | 85.19 | 86.58 | 86.23 | 86.23 | 86.76 | 86.63 |
| 2004 | 85.66 | 85.22 | 86.62 | 86.20 | 86.24 | 86.58 | 86.58 |
| 2005 | 85.58 | 85.22 | 86.62 | 86.12 | 86.18 | 86.52 | 86.57 |
| 2006 | 85.54 | 85.25 | 86.57 | 86.24 | 86.34 | 86.93 | 86.57 |
| 2007 | 85.54 | 85.23 | 86.61 | 86.24 | 86.30 | 86.87 | 86.57 |
| 2008 | 85.63 | 85.32 | 86.70 | 86.32 | 86.39 | 86.87 | 86.57 |
| 2009 | 85.56 | 85.38 | 86.72 | 86.36 | 86.37 | 86.83 | 86.60 |
| 2010 | 85.54 | 85.40 | 86.77 | 86.35 | 86.31 | 86.90 | 86.59 |
| 2011 | 85.55 | 85.37 | 86.78 | 86.32 | 86.37 | 86.87 | 86.64 |
| 2012 | 85.51 | 85.38 | 86.84 | 86.34 | 86.25 | 86.89 | 86.63 |
| 2013 | 85.49 | 85.35 | 86.73 | 86.22 | 86.24 | 86.68 | 86.65 |
| 2014 | 85.57 | 85.42 | 86.74 | 86.23 | 86.33 | 86.87 | 86.65 |
| 2015 | 85.54 | 85.40 | 86.81 | 86.33 | 86.30 | 86.90 | 86.62 |
| 2016 | 85.66 | 85.48 | 86.56 | 86.11 | 86.28 | 86.58 | 86.60 |
| 2017 | 85.68 | 85.46 | 86.60 | 86.15 | 86.30 | 86.89 | 86.63 |
| 2018 | 85.69 | 85.49 | 86.61 | 86.31 | 86.04 | 86.93 | 86.04 |
| 2019 | 85.66 | 85.53 | 86.65 | 86.19 | 85.97 | 86.96 | 86.04 |

Table A4.7 	Emission factors for European gasoline and diesel vehicles – COPERT IV model (European Environment Agency, 2007)

| Vehicle type and emission standard | N2O emission factors (mg/km) |  |  |  |  | CH4 emission factors (mg/km) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Urban |  | Rural | Highway |  | Urban |  | Rural |  | Highway |
|  | Cold | Hot |  |  |  | Cold | Hot |  |  |  |
| Passenger car |  |  |  |  |  |  |  |  |  |  |
| Gasoline |  |  |  |  |  |  |  |  |  |  |
| pre-Euro | 10.0 | 10.0 | 6.5 | 6.5 |  | 201.0 | 131.0 | 86.0 |  | 41.0 |
| Euro 1 | 18.8 | 26.5 | 10.7 | 5.5 |  | 45.0 | 26.0 | 16.0 |  | 14.0 |
| Euro 2 | 12.6 | 12.7 | 4.9 | 2.7 |  | 94.0 | 17.0 | 13.0 |  | 11.0 |
| Euro 3 | 8.3 | 1.50 | 0.33 | 0.23 |  | 83.0 | 3.0 | 2.0 |  | 4.0 |
| Euro 4 | 5.5 | 1.95 | 0.34 | 0.22 |  | 57.0 | 2.87 | 2.69 |  | 5.08 |
| Euro 5 | 2.15 | 2.22 | 0.19 | 1.20 |  | 57.0 | 2.87 | 2.69 |  | 5.08 |
| Euro 6 | 2.15 | 2.22 | 0.19 | 1.20 |  | 57.0 | 2.87 | 2.69 |  | 5.08 |
| Diesel |  |  |  |  |  |  |  |  |  |  |
| pre-Euro | 0.0 | 0.0 | 0.0 | 0.0 |  | 22.0 | 28.0 | 12.0 |  | 8.0 |
| Euro 1 | 0.0 | 2.0 | 4.0 | 4.0 |  | 18.0 | 11.0 | 9.0 |  | 3.0 |
| Euro 2 | 3.0 | 4.0 | 6.0 | 6.0 |  | 6.0 | 7.0 | 3.0 |  | 2.0 |
| Euro 3 | 15.0 | 9.0 | 4.0 | 4.0 |  | 3.0 | 3.0 | 0.0 |  | 0.0 |
| Euro 4 | 15.0 | 9.0 | 4.0 | 4.0 |  | 1.1 | 1.1 | 0.0 |  | 0.0 |
| Euro 5 | 15.0 | 9.0 | 4.0 | 4.0 |  | 1.1 | 1.1 | 0.0 |  | 0.0 |
| Euro 6 | 9.0 | 9.0 | 4.0 | 4.0 |  | 1.1 | 1.1 | 0.0 |  | 0.0 |
| LPG |  |  |  |  |  |  |  |  |  |  |
| pre-Euro | 0.0 | 0.0 | 0.0 | 0.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Euro 1 | 38.0 | 21.0 | 13.0 | 8.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Euro 2 | 23.0 | 13.0 | 3.0 | 2.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Euro 3 | 9.0 | 5.0 | 2.0 | 1.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Euro 4 | 9.0 | 5.0 | 2.0 | 1.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Euro 5 | 1.8 | 2.1 | 0.2 | 1.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Euro 6 | 1.8 | 2.1 | 0.2 | 1.0 |  | 80.0 | 80.0 | 35.0 |  | 25.0 |
| Light duty vehicles |  |  |  |  |  |  |  |  |  |  |
| Gasoline |  |  |  |  |  |  |  |  |  |  |
| pre-Euro | 10.0 | 10.0 | 6.5 | 6.5 |  | 201.0 | 131.0 | 86.0 |  | 41.0 |
| Euro 1 | 47.3 | 46.3 | 27.5 | 13.8 |  | 45.0 | 26.0 | 16.0 |  | 14.0 |
| Euro 2 | 83.8 | 27.7 | 15.8 | 12.3 |  | 94.0 | 17.0 | 13.0 |  | 11.0 |
| Euro 3 | 17.1 | 8.5 | 1.5 | 1.5 |  | 83.0 | 3.0 | 2.0 |  | 4.0 |
| Euro 4 | 14.1 | 1.17 | 0.36 | 0.36 |  | 57.0 | 2.0 | 2.0 |  | 0.0 |
| Euro 5 | 2.10 | 2.22 | 0.19 | 1.20 |  | 57.0 | 2.0 | 2.0 |  | 0.0 |
| Euro 6 | 2.10 | 2.22 | 0.19 | 1.20 |  | 57.0 | 2.0 | 2.0 |  | 0.0 |
| Diesel |  |  |  |  |  |  |  |  |  |  |
| pre-Euro | 0.0 | 0.0 | 0.0 | 0.0 |  | 22.0 | 28.0 | 12.0 |  | 8.0 |
| Euro 1 | 0.0 | 2.0 | 4.0 | 4.0 |  | 18.0 | 11.0 | 9.0 |  | 3.0 |
| Euro 2 | 3.0 | 4.0 | 6.0 | 6.0 |  | 6.0 | 7.0 | 3.0 |  | 2.0 |
| Euro 3 | 15.0 | 9.0 | 4.0 | 4.0 |  | 3.0 | 3.0 | 0.0 |  | 0.0 |
| Euro 4 | 15.0 | 9.0 | 4.0 | 4.0 |  | 1.1 | 1.1 | 0.0 |  | 0.0 |
| Euro 5 | 15.0 | 9.0 | 4.0 | 4.0 |  | 1.1 | 1.1 | 0.0 |  | 0.0 |
| Euro 6 | 9.0 | 9.0 | 4.0 | 4.0 |  | 1.1 | 1.1 | 0.0 |  | 0.0 |
| Heavy duty truck and bus |  |  |  |  |  |  |  |  |  |  |
| Gasoline all technologies | 6.0 | 6.0 | 6.0 | 6.0 |  | 140.0 | 140.0 | 110.0 |  | 70.0 |
| Diesel |  |  |  |  |  |  |  |  |  |  |
|  |  | GVW≤12t |  |  |  |  | GVW≤12t |  |  |  |
| pre-Euro | 30.0 | 30.0 | 30.0 | 30.0 |  | 85.0 | 85.0 | 23.0 |  | 20.0 |
| Euro I | 6.0 | 6.0 | 5.0 | 3.0 |  | 85.0 | 85.0 | 23.0 |  | 20.0 |
| Euro II | 5.0 | 5.0 | 5.0 | 3.0 |  | 54.4 | 54.4 | 20.0 |  | 18.6 |
| Euro III | 3.0 | 3.0 | 3.0 | 2.0 |  | 47.6 | 47.6 | 21.4 |  | 18.2 |
| Euro IV | 6.0 | 6.0 | 7.2 | 5.8 |  | 2.6 | 2.6 | 1.6 |  | 1.2 |
| Euro V | 15.0 | 15.0 | 19.8 | 17.2 |  | 2.6 | 2.6 | 1.6 |  | 1.2 |
| Euro VI | 18.5 | 18.5 | 19.0 | 15.0 |  | 2.6 | 2.6 | 1.6 |  | 1.2 |
|  |  | 12t<GVW≤16t |  |  |  |  | 12t<GVW≤16t |  |  |  |
| pre-Euro | 30.0 | 30.0 | 30.0 | 30.0 |  | 85.0 | 85.0 | 23.0 |  | 20.0 |
| Euro I | 11.0 | 11.0 | 9.0 | 7.0 |  | 85.0 | 85.0 | 23.0 |  | 20.0 |
| Euro II | 11.0 | 11.0 | 9.0 | 6.0 |  | 54.4 | 54.4 | 20.0 |  | 18.6 |
| Euro III | 5.0 | 5.0 | 5.0 | 4.0 |  | 47.6 | 47.6 | 21.4 |  | 18.2 |
| Euro IV | 11.2 | 11.2 | 13.8 | 11.4 |  | 2.6 | 2.6 | 1.6 |  | 1.2 |
| Euro V | 29.8 | 29.8 | 40.2 | 33.6 |  | 2.6 | 2.6 | 1.6 |  | 1.2 |
| Euro VI | 37.0 | 37.0 | 39.0 | 29.0 |  | 2.6 | 2.6 | 1.6 |  | 1.2 |
|  |  | 16t<GVW≤28t |  |  |  |  | 16t<GVW≤28t |  |  |  |
| pre-Euro | 30.0 | 30.0 | 30.0 | 30.0 |  | 175.0 | 175.0 | 80.0 |  | 70.0 |
| Euro I | 11.0 | 11.0 | 9.0 | 7.0 |  | 175.0 | 175.0 | 80.0 |  | 70.0 |
| Euro II | 11.0 | 11.0 | 9.0 | 6.0 |  | 112.0 | 112.0 | 69.6 |  | 65.1 |
| Euro III | 5.0 | 5.0 | 5.0 | 4.0 |  | 98.0 | 98.0 | 74.4 |  | 63.7 |
| Euro IV | 11.2 | 11.2 | 13.8 | 11.4 |  | 5.3 | 5.3 | 5.6 |  | 4.2 |
| Euro V | 29.8 | 29.8 | 40.2 | 33.6 |  | 5.3 | 5.3 | 5.6 |  | 4.2 |
| Euro VI | 37.0 | 37.0 | 39.0 | 29.0 |  | 5.3 | 5.3 | 5.6 |  | 4.2 |
|  |  | 28t<GVW≤34t |  |  |  |  | 28t<GVW≤34t |  |  |  |
| pre-Euro | 30.0 | 30.0 |  | 30.0 | 30.0 | 175.0 | 175.0 |  | 80.0 | 70.0 |
| Euro I | 17.0 | 17.0 |  | 14.0 | 10.0 | 175.0 | 175.0 |  | 80.0 | 70.0 |
| Euro II | 17.0 | 17.0 |  | 14.0 | 10.0 | 112.0 | 112.0 |  | 69.6 | 65.1 |
| Euro III | 8.0 | 8.0 |  | 8.0 | 6.0 | 98.0 | 98.0 |  | 74.4 | 63.7 |
| Euro IV | 17.4 | 17.4 |  | 21.4 | 17.4 | 5.3 | 5.3 |  | 5.6 | 4.2 |
| Euro V | 45.6 | 45.6 |  | 61.6 | 51.6 | 5.3 | 5.3 |  | 5.6 | 4.2 |
| Euro VI | 56.5 | 56.5 |  | 59.5 | 44.5 | 5.3 | 5.3 |  | 5.6 | 4.2 |
|  |  | GVW>34t |  |  |  |  | GVW>34t |  |  |  |
| pre-Euro | 30.0 | 30.0 |  | 30.0 | 30.0 | 175.0 | 175.0 |  | 80.0 | 70.0 |
| Euro I | 18.0 | 18.0 |  | 15.0 | 11.0 | 175.0 | 175.0 |  | 80.0 | 70.0 |
| Euro II | 18.0 | 18.0 |  | 15.0 | 10.0 | 112.0 | 112.0 |  | 69.6 | 65.1 |
| Euro III | 9.0 | 9.0 |  | 9.0 | 7.0 | 98.0 | 98.0 |  | 74.4 | 63.7 |
| Euro IV | 19.0 | 19.0 |  | 23.4 | 19.2 | 5.3 | 5.3 |  | 5.6 | 4.2 |
| Euro V | 49.0 | 49.0 |  | 66.6 | 55.8 | 5.3 | 5.3 |  | 5.6 | 4.2 |
| Euro VI | 61.0 | 61.0 |  | 64.0 | 48.0 | 5.3 | 5.3 |  | 5.6 | 4.2 |
| Urban bus or coach |  | All types |  |  |  |  | All types |  |  |  |
| pre-Euro | 30.0 | 30.0 |  | 30.0 | 30.0 | 175.0 | 175.0 |  | 80.0 | 70.0 |
| Euro I | 12.0 | 12.0 |  | 9.0 | 7.0 | 175.0 | 175.0 |  | 80.0 | 70.0 |
| Euro II | 12.0 | 12.0 |  | 9.0 | 6.0 | 113.8 | 113.8 |  | 52.0 | 45.5 |
| Euro III | 6.0 | 6.0 |  | 5.0 | 4.0 | 103.3 | 103.3 |  | 47.2 | 41.3 |
| Euro IV | 12.8 | 12.8 |  | 13.8 | 11.4 | 5.3 | 5.3 |  | 2.4 | 2.1 |
| Euro V | 33.2 | 33.2 |  | 40.2 | 33.6 | 5.3 | 5.3 |  | 2.4 | 2.1 |
| Euro VI | 41.5 | 41.5 |  | 39.0 | 29.0 | 5.3 | 5.3 |  | 2.4 | 2.1 |
| CNG |  |  |  |  |  |  |  |  |  |  |
| pre-Euro |  |  |  |  |  | 6,800 | 6,800 |  | 6,800 | 6,800 |
| Euro I |  |  |  |  |  | 6,800 | 6,800 |  | 6,800 | 6,800 |
| Euro II |  |  |  |  |  | 4,500 | 4,500 |  | 4,500 | 4,500 |
| Euro III |  |  |  |  |  | 1,280 | 1,280 |  | 1,280 | 1,280 |
| Euro IV and later |  |  |  |  |  | 980 | 980 |  | 980 | 980 |
| Power two wheeler |  |  |  |  |  |  |  |  |  |  |
| Gasoline |  |  |  |  |  |  |  |  |  |  |
| <50 cm3 | 1.0 | 1.0 |  | 1.0 | 1.0 | 219 | 219 |  | 219 | 219 |
| >50 cm3 2-stroke | 2.0 | 2.0 |  | 2.0 | 2.0 | 150 | 150 |  | 150 | 150 |
| >50 cm3 4-stroke | 2.0 | 2.0 |  | 2.0 | 2.0 | 200 | 200 |  | 200 | 200 |

## A4.2	Emissions from solid fuels

### A4.2.1	Activity data and uncertainties

The New Zealand Quarterly Statistical Return of Coal Production and Sales conducted by MBIE has near coverage of the sector, meaning that sampling error is small. The only other possible sources of error are non-sample errors (such as respondent error and processing error). The 2019 statistical difference for solid fuels in the balance table of the publication Energy in New Zealand (MBIE, 2020) was 5.0 per cent. This is used as the activity data uncertainty for solid fuels in 2019.

### A4.2.2	Emission factors and uncertainties

The estimated uncertainty in CO2 emission factors for solid fuels is ±2.2 per cent. This is based on the difference between the range of updated emission factors for the three different ranks of coal used in New Zealand. The uncertainty for CH4 and N2O emission factors is ±50.0 per cent because almost all emission factors are IPCC defaults.

## A4.3	Emissions from gaseous fuels

### A4.3.1	Activity data

Through the various surveys and information it collects, MBIE has full coverage of the natural gas sector. This means that there is no sampling error in natural gas statistics and the only possible sources of error include those such as respondent error and processing error. The 2019 statistical difference for gaseous fuels in the balance table of the publication Energy in New Zealand (MBIE, 2020) was 9.3 per cent. This is used as the activity data uncertainty for gaseous fuels in 2019.

### A4.3.2	Emission factors

The estimated uncertainty in CO2 emission factors for gaseous fuels is ±2.8 per cent. This is based on the difference between the range of emission factors for three large gas fields in New Zealand. Together, these gas fields made up over 55 per cent of New Zealand’s total gas supply in 2019. The uncertainty for CH4 and N2O emission factors is ±50.0 per cent because almost all emission factors are IPCC defaults.

## A4.4	Energy balance

Detailed and up-to-date energy balance tables for New Zealand are available online: www.mbie.govt.nz/building-and-energy/energy-and-natural-resources/energy-statistics-and-modelling/energy-statistics/energy-balances.

Further information can be found within the publication Energy in New Zealand (MBIE, 2020), also available online: www.mbie.govt.nz/building-and-energy/energy-and-natural-resources/energy-statistics-and-modelling/energy-publications-and-technical-papers/energy-in-new-zealand.

Table A4.8 gives a time series of energy use versus non-energy use of natural gas.

Table A4.8 	Split of energy use and non-energy use of natural gas in petajoules

|  | Energy use | Non-energy use |
| --- | --- | --- |
| 1990 | 129.5 | 14.2 |
| 1991 | 143.9 | 22.1 |
| 1992 | 152.6 | 18.8 |
| 1993 | 148.0 | 21.1 |
| 1994 | 137.7 | 25.8 |
| 1995 | 127.4 | 36.2 |
| 1996 | 147.7 | 47.5 |
| 1997 | 170.4 | 48.9 |
| 1998 | 146.2 | 46.6 |
| 1999 | 168.5 | 54.2 |
| 2000 | 173.9 | 61.8 |
| 2001 | 190.6 | 55.4 |
| 2002 | 177.1 | 57.8 |
| 2003 | 151.9 | 26.1 |
| 2004 | 129.8 | 32.1 |
| 2005 | 136.4 | 13.0 |
| 2006 | 137.2 | 15.0 |
| 2007 | 148.6 | 15.4 |
| 2008 | 138.2 | 18.4 |
| 2009 | 134.6 | 25.5 |
| 2010 | 150.0 | 25.6 |
| 2011 | 134.0 | 24.5 |
| 2012 | 146.2 | 32.0 |
| 2013 | 145.1 | 40.3 |
| 2014 | 149.2 | 60.7 |
| 2015 | 144.3 | 51.4 |
| 2016 | 134.1 | 59.1 |
| 2017 | 145.6 | 53.8 |
| 2018 | 131.4 | 45.3 |
| 2019 | 139.7 | 51.2 |

## A4.5	Carbon dioxide reference approach for the Energy sector

### A4.5.1	Estimation of carbon dioxide using the IPCC reference approach

The reference approach uses a country’s energy supply data to calculate the CO2 emissions from the combustion of fossil fuels using the apparent consumption equation. The apparent consumption in the reference approach is derived from production, import and export data. This information is included as a check for combustion-related emissions calculated from the sectoral approach.

The apparent consumption for primary fuels in the reference approach is obtained from ‘calculated’ energy-use figures (see annex 2 and section A4.4). These are derived as a residual figure from an energy balance equation comprising production, imports, exports, stock change and international transport on the supply side according to the IPCC Guidelines (IPCC, 2006).

The majority of the CO2 emission factors for the reference approach are specific to New Zealand. Most emission factors for liquid fuels are based on annual carbon content and the gross calorific value data provided by New Zealand’s only oil refinery, Refining New Zealand Ltd. Where these data are not available, an IPCC default is used. The natural gas emission factor is based on a production-derived, weighted average of emission factors from all gas production fields. The CO2 emission factors for solid fuels were updated for the 2014 inventory submission following analysis to verify default emission factors used for the New Zealand Emissions Trading Scheme. For more information on this improvement, see chapter 3, section 3.3.2.

#### Solid fuels in iron and steel manufacture

As mentioned in chapter 3, section 3.2.3, some of the coal production activity data in the reference approach are used in steel production. The Industrial Processes and Product Use sector accounts for the CO2 emissions from this coal in the sectoral approach, as recommended by the IPCC Guidelines (IPCC, 2006); therefore they are not included in the common reporting format table 1.AA Fuel combustion – sectoral approach.

For simplicity, all feedstock carbon is excluded from the reference approach according to the IPCC Guidelines (IPCC, 2006). Without taking into account the use of by-product gases, this can create some discrepancies between the reference and sectoral approaches.

### A4.5.2	Comparison of the IPCC reference approach with the New Zealand sectoral methodology

For 2019, CO2 emissions estimated with the sectoral approach were 2.6 per cent higher than those estimated with the reference approach. Figure A4.1 shows the results for the two approaches for the period 1990 to 2019.

In some years, differences exist between the reference and sectoral approaches. Much of this is due to the statistical differences found in the energy balance tables (MBIE, 2020) that are used as the basis for the reference and sectoral approach. Since 2000, the standard of national energy data has improved significantly, due to increased resources and focus. In 2008, Stats NZ delegated responsibility for the collection and analysis of national energy data to MBIE. Before 2008, various energy statistics were collected by Stats NZ or MBIE. The change resulted in a more consistent and transparent approach to energy data collection because one agency collected data across the supply chain.

Figure A4.1 	Reference and sectoral approach carbon dioxide by fuel type (kt CO2)

###### Sources of differences

For gaseous fuels, the field-specific emission factors are used for natural gas supplied for industrial processes, while the reference approach uses an average emission factor.

For liquid fuels, the energy balance is mass balanced but not carbon balanced. The fuel category ‘other oil’ is an aggregation of several fuel types, and so it is difficult to quantify a reliable carbon emission factor for the reference approach.

In the sectoral approach, sector- or even plant-specific calorific values are used to calculate energy consumption, whereas in the reference approach, average (country-specific) calorific values are applied.

## Annex 4: References

CRL Energy Ltd. 2009. Reviewing Default Emission Factors in Draft Stationary Energy and Industrial Processes Regulations: Coal. Contract report prepared for the Ministry for the Environment. Wellington: Ministry for the Environment.

Eng G, Bywater I, Hendtlass C. 2008. New Zealand Energy Information Handbook. Christchurch: New Zealand Centre for Advanced Engineering.

European Environment Agency. 2007. EMEP/CORINAIR Emission Inventory Guidebook – 2007. Copenhagen: European Environment Agency.

Hale R, Twomey I. Unpublished. Reviewing default emission factors for liquid fossil fuels adopted by the New Zealand Emissions Trading Scheme. Report by consultants Hale & Twomey Limited prepared for the Ministry for the Environment in 2009.

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 2. Energy. IPCC National Greenhouse Gas Inventories Programme. Japan: Institute for Global Environmental Strategies for IPCC.

Ministry of Business, Innovation and Employment. 2020. Energy in New Zealand 2020. Wellington: Ministry of Business, Innovation and Employment.

# Annex 5: Supplementary information for the KP-LULUCF sector

## A5.1	Technical corrections to the FMRL

### A5.1.1	Introduction

For the second commitment period, reporting on Forest management under the Kyoto Protocol is mandatory. Accounting for Forest management during the second commitment period is relative to a forest management reference level (FMRL) (Decision 2/CMP.7, UNFCCC, 2012).

New Zealand’s FMRL was initially set at 11.15 million tonnes carbon dioxide equivalent (Mt CO2-e) on average per year for the period 2013 to 2020 (New Zealand Government, 2011). This value was constructed using a business-as-usual projection of pre-1990 planted forest growth and harvest for the period 2013 to 2020. It was based on yield tables and statistics on the area in each age class of pre-1990 planted forest from the National Exotic Forest Description (NEFD) as at 2009 (Ministry for Primary Industries, 2009).

The 2011 FMRL included the following assumptions:

pre-1990 natural forests were in steady state

no pre-1990 planted forest deforestation would occur between 2013 and 2020 (pre-1990 natural forests were excluded from the analyses; post-1989 forest deforestation is reported under Article 3.3 – Deforestation)

between 2013 and 2020, 2,000 hectares per year would be converted to non-forest land, and the equivalent forest would be planted elsewhere (i.e., 2,000 hectares per year would be reported as carbon equivalent forest (CEF) and be accounted for under Forest management)

while harvest of post-1989 planted forest will increase over the period, pre-1990 planted forests will still make up a substantial proportion of total forest harvest

all carbon is assumed to be instantly emitted at the time of harvest (emissions and removals by the Harvested wood products pool were not considered)

no allowance was made for the impacts of potential natural disturbances beyond background levels captured in the carbon stock yield tables.

The FMRL also reflects the following New Zealand legislation (including amendments) and current policies:

the Forest Act 1949, which regulates the removal of timber from natural indigenous forests

the South Island Landless Natives Act 1906, which transferred 17,000 hectares of natural indigenous forest to South Island Māori. The harvesting of this forest is also subject to the Resource Management Act 1991

the Climate Change Response Act 2002, which makes owners of pre-1990 forest who deforest liable for the emissions associated with that activity

the New Zealand’s biofuels policy of the time (under which it was thought most feedstock for biofuel was likely to be derived from non-forest sources).

It was assumed that this legislation and these policies would prevent any significant deforestation of pre-1990 forests, and that the New Zealand Emissions Trading Scheme would encourage harvest in pre-1990 planted forests over post-1989 forest.

The 2011 FMRL was determined by modelling the pre-1990 planted forest estate using a Forestry-Oriented Linear Programming Interpreter (FOLPI). As mentioned above, the model developed in FOLPI was based on an age-class distribution of pre-1990 planted forest as at 2009 from the NEFD, and simulated expected harvesting and replanting of this forest. Some additional modelling of decay of residues from harvest events was also carried out in MS Excel.

Since the 2011 FMRL was submitted, supplementary guidance has been prepared that describes the circumstances that would trigger a technical correction to the FMRL (IPCC, 2014). Changes to policies that affect harvest rate (as listed above) cannot be corrected for, but corrections can be made to reflect changes to the method for reporting against the FMRL and to address recommendations made by United Nations Framework Convention on Climate Change (UNFCCC) expert review teams (ERTs).

A technical assessment of New Zealand’s reference level submission was carried out by an ERT in 2011 (UNFCCC, 2011). The ERT noted a number of items for New Zealand to address either through the provision of additional data or through applying technical corrections. These included (UNFCCC, 2011, pp 6–10):

maintaining consistency in the fraction of harvested biomass instantaneously oxidised when estimating emissions from harvest in the FMRL and in reporting against it (paragraph 21)

ensuring consistency between the National Inventory Report (NIR) and the FMRL and, therefore, the updating of the current FMRL when new data or information become available (paragraph 22)

making efforts to disaggregate gains and losses by biomass pool (paragraph 35);

providing further information on how forest owners will be able to move from historical and current harvesting practice to the longer rotation length projected in the FOLPI model (paragraph 36)

explaining in more detail how the difference in both harvested areas and harvesting age as calculated by FOLPI could be achieved (paragraph 36)

comparing the results provided in its submission with a rerun of the FOLPI model in which the harvesting of over-mature forests (over 32 years of age) is constrained, and modify the reference level accordingly if necessary (paragraph 36)

if estimates for natural forests are included in future NIR submissions, making a technical adjustment of the FMRL (paragraph 37)

agreeing that in the future a technical correction should be made to incorporate the Harvested wood products pool (paragraph 38).

### A5.1.2	Technical corrections required

For the 2016 submission, the following technical corrections were made to meet IPCC guidance and address recommendations by the UNFCCC ERT. These aimed to:

- ensure consistency between the method used for greenhouse gas reporting of Forest management and that used to calculate the FMRL (Kyoto Protocol Supplement, IPCC, 2014, sections 2.7.5.2 and 2.7.6). This involved making changes to:

  - align forest area estimates

  - align CEF emissions calculation methods

  - include overplanting estimates (pre-1990 natural forest conversions to pre-1990 planted forest)

  - include non-carbon emissions

- include an estimate for pre-1990 natural forest emissions following completion of the re-measurement of the pre-1990 natural forest inventory and subsequent analysis

- address new elements of Decision 2/CMP.7 including

  - accounting for Harvested wood products

  - the application of the natural disturbances provision.

For the 2019 submission, an additional technical correction was applied to the FMRL to capture recent improvements to the Harvested wood products estimates on exported, unprocessed logs.

Another technical correction was applied to the 2021 submission. The technical correction was made to address a number of methodological inconsistencies between emissions estimated for Forest management and those used to calculate the FMRL, including:

- aligning pre-1990 natural forest methods

- aligning the pre-1990 planted forest yield tables

- correcting a model assumption to align the harvesting projections with the Harvested wood products estimates

- correcting the background disturbance level and aligning this calculation with the FMRL.

#### Technical corrections: Addressing methodological inconsistencies 
between the 2011 FMRL and Forest management reporting

The first step taken to calculate technical corrections to the FMRL was to replicate the FMRL as submitted in 2011, applying the same policy assumptions, but using the reporting system and historical data that are used to report on Forest management in the inventory.

This technical correction addresses two of the findings of the technical assessment (listed above) by:

- maintaining consistency in the fraction of harvested biomass instantaneously oxidised

- ensuring consistency between the emissions reported in the inventory for Forest management and the FMRL.

This is achieved by using the harvest and deforestation data from 1990 to 2008 from the 2013 inventory (Ministry for the Environment, 2015) as the starting point for the revised projections. Harvesting and deforestation areas for 2009 to 2020, sourced from the Ministry for Primary Industries, are the same as those used for the 2011 FMRL. The first year of projected data within New Zealand’s 2011 FMRL submission was 2009, as data were required to be projected forward from 2009 to enable the projections for the 2013 to 2020 period to be made.

Minor adjustments were made to these data as outlined below.

The forest area estimate was aligned to match it to the area of forest that is included under the definition of forest used for UNFCCC reporting.

Pre-1990 harvesting data (average harvest age) from 2009 to 2020 have been altered from those used in the 2011 FMRL. The average age at harvest has been adjusted down to 28 years, to address the issue raised by the ERT in its technical assessment of the 2011 FMRL. The area of harvest, however, has been kept the same.

The age distribution of plantation forest under Forest management was incorrectly assumed for Forest land prior to 2010. We know this because the age-class of Forest land harvested post 2010 would not have been possible had our harvest age assumptions made for the FMRL projection been correct. A technical correction to the forest age harvest assumptions needed to be made because otherwise there would be a methodological inconsistency between the age-classes being harvested under Forest management and the input age-class data used for the FMRL projection. This technical correction is made in alignment with 2013 Kyoto Protocol Supplement, section 2.7.5.2, page 2.97, item (ii). Note that this text has been revised to address ERT review comment KL.13, 2019 (FCCC/ARR/2019/NZL, UNFCCC, 2019).

Changes to CEF reporting were made to reflect updated guidance for this reporting released after the 2011 FMRL submission.

##### Aligning forest area estimates

The 2011 FMRL submission was based on data derived from the NEFD (Ministry for Primary Industries, 2015). The NEFD is an annual survey of forest owners that represents the ‘net stocked area’ of the planted production forest estate established with the primary intention of producing wood or fibre. The Land Use and Carbon Analysis System (LUCAS) that is used for reporting emissions for Forest management in the inventory uses complete wall-to-wall mapping to estimate forest area. This means LUCAS maps to a ‘gross stocked area’ where harvested areas, skid sites, forest roads and unstocked gullies are included in the mapped forest area. This gross stocked area is also the basis for the national sampling system used for deriving emission factors for the Forest land use classes. For modelling emissions for reporting under the UNFCCC, LUCAS has isolated the net stocked area from the mapped gross stocked area so the modelled area is compatible between the two data sources (LUCAS and NEFD). The LUCAS gross stocked area of pre-1990 planted forest area is 1.47 million hectares as at 2009. The LUCAS net stocked area, as at 2009, is estimated to be 1.25 million hectares (a 12.4 per cent difference). This compares with 1.14 million hectares from the NEFD as at 2009. Because the 2011 FMRL did not take into account differences in the data sources due to the two purposes for which the data are collected, a technical correction is required to correct the original NEFD-based FMRL to the LUCAS mapped area estimates used for reporting for Forest management.

The need for this adjustment extends to estimates of the area of CEF and deforestation, meaning these original net stocked area values need to have an unstocked area component added to them (the same adjustment of 12.4 per cent is used). The harvest areas, however, remain unchanged because both approaches harvest a net stocked area.

##### Harvest data

Pre-1990 planted forest harvesting uses projections from 2009 to 2020 at an average age of (approximately) 28 years (see table A5.1.1). This is to address the issue raised by the ERT that harvest ages in the projection were older than those observed historically and there were no policies in place that would influence rotation length or change the average harvest ages of planted forests. While the average harvest ages used for the technical correction do not match, the harvest areas match the 2011 FMRL harvest areas.

Table A5.1.1 	Pre-1990 planted forest data used to estimate emissions for the technically corrected FMRL

| Year | Pre-1990 planted forest deforestation (kha) | Pre-1990 planted forest harvested (kha) | Pre-1990 planted forest harvest average age (years) |
| --- | --- | --- | --- |
| 1990 | – | 19.369 | 28.0 |
| 1991 | – | 19.883 | 28.0 |
| 1992 | – | 22.639 | 28.0 |
| 1993 | – | 23.275 | 28.0 |
| 1994 | – | 25.000 | 28.0 |
| 1995 | – | 29.275 | 28.0 |
| 1996 | – | 31.250 | 28.0 |
| 1997 | – | 32.175 | 28.0 |
| 1998 | – | 31.575 | 28.0 |
| 1999 | – | 34.075 | 28.0 |
| 2000 | 2.305 | 35.551 | 28.0 |
| 2001 | 2.225 | 39.371 | 28.0 |
| 2002 | 1.616 | 46.149 | 28.0 |
| 2003 | 3.137 | 40.428 | 28.0 |
| 2004 | 6.777 | 33.867 | 28.0 |
| 2005 | 13.186 | 27.198 | 28.0 |
| 2006 | 16.596 | 27.036 | 28.0 |
| 2007 | 22.022 | 22.175 | 28.0 |
| 2008 | 4.103 | 37.243 | 28.0 |
| 2009 | 2.389 | 29.218 | 27.8 |
| 2010 | 2.383 | 33.086 | 28.4 |
| 2011 | 2.396 | 37.479 | 28.2 |
| 2012 | 2.378 | 41.354 | 27.8 |
| 2013 | 2.378 | 46.112 | 27.7 |
| 2014 | 2.247 | 50.021 | 27.8 |
| 2015 | 2.247 | 49.697 | 28.0 |
| 2016 | 2.247 | 49.724 | 28.1 |
| 2017 | 2.247 | 50.018 | 28.5 |
| 2018 | 2.247 | 49.967 | 28.9 |
| 2019 | 2.247 | 45.817 | 29.8 |
| 2020 | 2.247 | 43.817 | 28.9 |

#### Carbon equivalent forests

The method used to calculate the emissions from the application of CEF in the 2011 FMRL was inconsistent with the provisions of Decision 2/CMP.7 (UNFCCC, 2012) and the guidance for reporting (Kyoto Protocol Supplement, IPCC, 2014). The correct method for calculating emissions for CEF is to model the events by applying the same methods as would apply to deforestation and afforestation events but report all emissions (and removals) under Forest management.

##### Carbon equivalent forest harvested and converted

The estimate for carbon equivalent forest harvested and converted (CEFhc) included in the technical correction uses projections of land-use change from 2009 to 2020 at an average age of (approximately) 28 years and at the rate of 2,247 hectares per annum (the net stocked area from the 2011 FMRL of 2,000 hectares, plus an unstocked proportion of 247 hectares, which contains a much lower carbon stock as explained above under ‘Aligning forest area estimates’).

##### Carbon equivalent forest newly established

Carbon equivalent forest newly established (CEFne) land is replanted at an equivalent annual area (2,000 hectares net stocked area plus 247 hectares that is unstocked), and the post-1989 planted forest yield table is applied to the net stocked area. The post-1989 planted forest yield table is deemed appropriate because the new forest is established on Grassland and the history of this newly planted land is most similar to post-1989 planted forest land.

In the technical correction to the FMRL, CEF land is modelled as going to and coming from the three Grassland types (low producing grassland, high producing grassland and grassland with woody biomass) in equal amounts. Soil emissions resulting from conversion and establishment of CEF land are also now included in the FMRL.

#### Overplanting

The 2011 FMRL did not model emissions from overplanting that occurs on Forest management land. This activity occurs when pre-1990 natural forest is converted to planted forest. Overplanting has been occurring since 1990, as demonstrated in the common report format (CRF) data. The system used for national greenhouse gas reporting for the sector reports the area and emissions associated with that practice within the Forest management category. Overplanting should have been captured in the original FMRL projection but was omitted due to an error. There has been no management practice change that has encouraged overplanting to develop over the reporting period. To maintain consistency with Forest management reporting, a technical correction was applied. This technical correction results in the addition of 0.039 kilotonnes carbon dioxide (kt CO2) emissions to the annual estimate of emissions in the FMRL. Note that this text has been revised to address review recommendation KL.14, 2019 (FCCC/ARR/2019/NZL, UNFCCC, 2019).

#### Non-carbon emissions

Non-carbon emissions were not included in the 2011 FMRL submission; therefore, a technical correction is required to include these emissions. Non-carbon emissions are estimated based on the average controlled burning from 1990 to 2009 and the minimum historical level for wildfire.

##### Controlled burning

Emissions from the burning of pre-1990 planted forest harvest residues are now included. The harvest rate is as per the FMRL, and the proportion burned is that applied to the LULUCF Forest land remaining forest land category during the first commitment period of the Kyoto Protocol.

Burning of residues associated with conversions of pre-1990 natural forest to pre-1990 planted forest are included and are assumed to occur at the same rate as reported during the first commitment period.

##### Wildfire emissions

Wildfires are hard to predict and are influenced by inter-annual climatic conditions and regional drought. To estimate emissions from wildfire, the default methodology described in the Kyoto Protocol Supplement, IPCC, 2014, section 2.3.9.6 has been applied:

the value of the mean [of natural disturbance time series data] plus two times the standard deviation is calculated using the entire time series of data in the calibration period. Any outlier value (ie above mean plus two times the standard deviation) is removed. This process is repeated until there are no outliers.

The default method calibration period has been applied between 1990 and 2009. This approach is taken to be consistent with New Zealand’s background level of natural disturbance.

##### Nitrous oxide emissions

It is assumed that there are no nitrous oxide emissions from fertilisation of forests within the FMRL. These are minor and captured within the Agriculture sector.

#### Natural disturbance

Emissions from natural disturbance events were not originally considered in the calculation of the 2011 FMRL. New Zealand has reported its intention to apply the natural disturbance provision, and, for Forest management, the background level has been set using the default method described in section 2.3.9.6 of the Kyoto Protocol Supplement (IPCC, 2014). This is included in the estimate of the non-carbon emissions as described above.

However, emissions from, and associated with, salvage logging cannot be excluded from accounting during the second commitment period. This means that, when developing the natural disturbance background level, historical emissions from natural disturbances should exclude these emissions. New Zealand has not excluded these emissions from the historical data used to calculate its background level of natural disturbance emissions under its technically corrected FMRL. If New Zealand applies the provision to exclude emissions from natural disturbances from its accounting, the background level will then be adjusted to remove these salvage logging emissions.

#### Pre-1990 planted forest

To improve the accuracy of the Forest management emissions estimates, multiple yield tables have been applied to the pre-1990 planted forest estate in the 2021 submission. Previously only one yield table was applied to all pre-1990 planted forest. However, due to improved genetics and management practices, a single yield table was found not to be representative of the entire pre-1990 planted forest estate. Therefore, in the 2021 submission three yield tables have been applied: one for all years prior to 1990; one for 1990 to 2009; and a third for 2010 to 2019.

The application of multiple yield tables to the pre-1990 planted forest estate creates an inconsistency between the methods used to calculate Forest management emissions and those used to calculate the FMRL. Therefore, a technical correction is required. The FMRL has been corrected using two yield tables for the pre-1990 planted forest estate: one for all years prior to 1990; and one for 1990 to 2009. The third yield table, 2010 to 2019, has not been included in the FMRL technical correction, as it includes developments in management practices that would not have been known at the time of the FMRL construction. Therefore, the projected FMRL from 2009 onwards uses the 1990 to 2009 yield table.

#### Pre-1990 natural forest

Emissions and removals by pre-1990 natural forest were not included in the 2011 FMRL submission. Because pre-1990 natural forest is now included in New Zealand’s reporting of emissions for Forest management land, a technical correction is required. The rate of carbon change used for this technical correction is consistent with that reported from 1990 to 2013 in the 2015 inventory.

When projections of pre-1990 natural forest emissions are incorporated into the technically corrected FMRL, the area under Forest management is reduced to factor in the projected deforestation of these forests. This deforested land will be reported under Article 3.3 
– Deforestation. The business-as-usual projection of pre-1990 natural forest deforestation is based on the historical rate seen between 1990 and 2009.

In the 2020 submission, an updated, improved method to measure stock change in natural forests was applied. This updated methodology was not applied to the methodology used to calculate the FMRL. This methodological inconsistency between the methods used to calculate the pre-1990 natural forest under Forest Management and that used in the FMRL has been addressed in the 2021 submission, resulting in a correction of –2.711 Mt CO2-e yr-1 to the FMRL.

#### Harvested wood products

Emissions and removals for the Harvested wood products pool were not included in the 2011 FMRL submission. The technical correction for this uses the same spreadsheet model as that used for New Zealand’s Forest management reporting with minor modifications, in order to enable reporting to 2020. The technical correction made reflects that there were no government policies either in place, or being planned, that would increase wood use and/or domestic production between 2013 and 2020.

To estimate emissions from harvested wood products from 2013 to 2020, the activity data time series was investigated for trends from 1990 to 2009. Production of products with relatively flat trends through the time series (i.e., pulp and paper) was held at 2009 rates between 2009 and 2020, and products whose production had been increasing over the period (i.e., panels and sawn wood) were increased at the projected rate of population increase (1 per cent; sourced from Stats NZ). Changes in the harvesting rate between 2013 and 2020 have no impact on the production of domestic harvested wood products because wood that is not processed in New Zealand is assumed to be exported.

Previously, exported harvested wood products were not considered in the FMRL. In the 2016 NIR (2018 submission), New Zealand revised its harvested wood products model to include products made from exported logs based on an export markets study. This investigated the use and discard rate of harvested wood products produced from raw materials of New Zealand origin. The inclusion of exported harvested wood products is in line with paragraph 27 of Decision 2/CMP.7 and follows the methodology provided for in table 12.1, chapter 12, volume 4 of the 2006 IPCC Guidelines.

To ensure consistency between the method used for greenhouse gas reporting of exported harvested wood products derived from Forest management activities and that used to calculate the FMRL (Kyoto Protocol Supplement, IPCC, 2014, sections 2.7.5.2 and 2.7.6), a technical correction was applied to the FMRL in the 2019 submission.

However, the inclusion of exported harvested wood products in the FMRL calculation considered that the volume of this export commodity would not change. A technical correction has been applied to the 2021 submission to correct this assumption. This correction has been applied to ensure methodological consistency between the emissions reported under Forest management and those used to calculate the FMRL. The FMRL projects that harvesting under Forest management increases through time. Given the capacity for domestic production of harvested wood products is limited, it should have been assumed that an increase in harvest would therefore lead to an increase in exported harvested wood products. This assumption has been corrected.

Harvested wood products from pre-1990 natural forest are excluded. The volume produced from the harvesting of pre-1990 natural forests is less than 0.1 per cent of New Zealand’s total harvest volume (Ministry for Primary Industries, 2015).

### A5.1.3	Technical corrections and their impact

The impact of the technical corrections made in the 2016, 2019 and 2021 submissions to the FMRL is summarised in table A5.1.2.

Table A5.1.2	Summary of the technical corrections to the FMRL

|  | Emissions (Mt CO2-e yr-1) |
| --- | --- |
| FMRL | 11.150 |
| Technical corrections |  |
| Carbon equivalent forest (CEF) | –0.074 |
| Pre-1990 planted forest | –9.245 |
| Non-carbon (including natural disturbance) | 0.009 |
| Pre-1990 natural forest | –2.711 |
| Harvested wood products | –9.839 |
| Sum of technical corrections | –21.860 |
| FMRLcorr | –10.710 |

Note: 	FMRL = forest management reference level; FMRLcorr = technically corrected forest management reference level.

Figure A5.1.1 provides a breakdown of the various components of the technical corrections over the time series.

Figure A5.1.1	Technical corrections to the FMRL by category

Note: 	Non-carbon emissions are 0.009 Mt CO2-e per year, which is too small to display at this scale. FM = forest management; FMRL = forest management reference level.

Figure A5.1.2 provides a comparison of recalculated estimates with previous estimates. This illustrates the time-series consistency of the estimates.

Figure A5.1.2	Comparison of the 2011 FMRL and total of technical corrections over the period to 2020

Note: 	FMRL = forest management reference level; FMRLCORR = technically corrected forest management reference level.

## A5.2 	Natural disturbance

New Zealand has chosen to apply the default method described in section 2.3.9.6 of the Kyoto Protocol Supplement (IPCC, 2014) for calculating its background level of natural disturbances for both Afforestation and reforestation and Forest management. This method has been applied following ERT recommendation KL.10, 2019 (FCCC/ARR/2019/NZL, UNFCCC, 2019).

Types of natural disturbances New Zealand intends to exclude from the accounting are:

wildfires

- invertebrate and vertebrate pests and diseases

- extreme weather events

- geological disturbances.

In all cases except fire, New Zealand assumes a zero baseline between 1990 and 2009. While other natural disturbance events occurred throughout the calibration period, assumptions were made for the purposes of calculating the background level.

For planted forests reported under Afforestation and reforestation and Forest management, salvage logging is considered to take place in all disturbed forests.

In the case of pre-1990 natural forests, the ground plot measurement programme captures emissions from natural disturbances implicitly, and the emissions from natural disturbance events, apart from wildfires, cannot be separated from other disturbance events. The stock change estimates reported for natural forests include background levels of small-scale natural disturbance events.

Only direct oxidation of biomass in wildfires is considered for the purposes of calculating a background level of natural disturbance for both Afforestation and reforestation and Forest management land, regardless of forest type. The data used are as reported under the UNFCCC for the period 1990 to 2009 (see chapter 6, section 6.11.5).

### A5.2.1 	Afforestation and reforestation

New Zealand may choose to apply the provision for the treatment of natural disturbance emissions to its Afforestation and reforestation accounting (Ministry for the Environment, 2015). Due to the nature of Afforestation and reforestation accounting and reporting methods, the background level of carbon dioxide emissions from natural disturbance is already captured implicitly within the reported estimates. New Zealand separately estimates and reports the non-carbon emissions from natural disturbances. The background level has been calculated using the default method described in section 2.3.9.6 of the Kyoto Protocol Supplement (IPCC, 2014). However, both the post-1989 forest area and the carbon stock increase during the calibration period. To account for the annual change, background level for the calibration period is calculated as a proportion of the post-1989 forest estate. This proportion is then multiplied by the carbon stock in post-1989 forest for each year in the reporting period (2013 to 2020). This approach provides the background level and corrects for the increasing area and age (and therefore carbon stock exposed to natural disturbance) in post-1989 forests.

The Afforestation and reforestation background level for 2019 was 2.414 kilotonnes carbon dioxide equivalent (kt CO2-e).

##### Avoiding the expectation of net credits or net debits for the application of the natural disturbance provision: Afforestation and reforestation

The background level is calculated using the default methodology described in section 2.3.9.6 of the Kyoto Protocol Supplement (IPCC, 2014). The proportion from the calibration period is then multiplied by the carbon stock in post-1989 forest for each year in the reporting period (2013 to 2020). This approach is taken because:

a trend is observed in natural disturbance emissions during the calibration period for Afforestation and reforestation. Emissions from natural disturbances have been increasing throughout the calibration period as the age of these forests and, therefore, biomass increase through time. This trend has continued during the second commitment period. The calibration period was used to obtain an annual emissions value by proportion of carbon stocks and then used to calculate the background level for the 2013 year onwards, based on the carbon stocks of Afforestation and reforestation lands in each year

gross:net accounting applies to Afforestation and reforestation activities. Emissions from natural disturbances occurring during any year of the commitment period, which fall below the background level, are not excluded from the accounting. Emissions from natural disturbances that are greater than the background level in any year of the commitment period are able to be excluded from the accounting if a Party chooses

if emissions from natural disturbances are greater than the background level, they can be excluded from the accounting and there is no expectation of net debits arising. If emissions are less than the background level in any year of the commitment period, all emissions from natural disturbance will still be accounted for. There is no expectation of net debits in this scenario. Under gross:net accounting for Afforestation and reforestation activities, it would not be possible to expect net credits when applying this approach to excluding the emissions from natural disturbances.

### A5.2.2	Forest management

The background level of natural disturbance for Forest management was calculated as 9.249 kt CO2-e.

##### Avoiding the expectation of net credits or net debits for the application of the natural disturbance provision: Forest management

The background level has been calculated using the default methodology described in section 2.3.9.6 of the Kyoto Protocol Supplement (IPCC, 2014). Using this method, the expectation of net credits or net debits for the application of the natural disturbance provision is avoided because:

there is no observed trend in natural disturbance emissions during the calibration period for Forest management and therefore none can be expected during the second commitment period

any emissions from natural disturbances during the commitment period that fall below the background level are not excluded from the accounting. During the commitment period, emissions from natural disturbances that are above the background level are, subject to New Zealand’s discretion, able to be excluded from the accounting

the accounting for Forest management is against a projected business-as-usual FMRL. The background level is included implicitly within the FMRL, and any emissions greater than the background level can be excluded from the accounting.

## A5.3	Carbon equivalent forests

Information on carbon equivalent forests is provided in aggregated form in CRF table 4(KP-I)B.1.2. Details of each application that make up the reported estimates are provided in table A5.3.1.

Table A5.3.1	Breakdown of carbon equivalent forests by domestic scheme application from 2014 to 2019

| Scheme ID | Management type | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CEF – 31 | Newly established (ha) | – | – | – | – | 10.19 | – |
|  | Harvested and converted (ha) | – | – | – | 7.17 | – | – |
|  | Net change (tC) | – | – | – | –1.81 | –0.02 | 0.00 |
| CEF – 20 | Newly established (ha) | – | – | – | 14.47 | – | – |
|  | Harvested and converted (ha) | – | 7.69 | – | – | – | – |
|  | Net change (tC) | – | –1.67 | 0.01 | –0.04 | –0.00 | 0.01 |
| CEF – 11 | Newly established (ha) | – | – | – | 771.43 | 992.04 | – |
|  | Harvested and converted (ha) | 3.36 | 76.81 | 409.17 | 488.34 | 235.11 | 9.37 |
|  | Net change (tC) | –0.74 | –16.49 | –87.87 | –124.05 | –58.41 | –1.87 |
| CEF – 25 | Newly established (ha) | – | – | – | – | 279.64 | – |
|  | Harvested and converted (ha) | – | – | – | 79.63 | – | 5.24 |
|  | Net change (tC) | – | – | – | –20.16 | –0.42 | –1.37 |
| CEF – 3 | Newly established (ha) | – | – | 189.93 | – | 247.19 | – |
|  | Harvested and converted (ha) | 42.96 | 373.95 | 1.43 | – | – | – |
|  | Net change (tC) | –9.21 | –80.52 | 0.17 | 0.19 | 0.50 | 0.37 |
| CEF – 17 | Newly established (ha) | – | – | – | 8.61 | – | – |
|  | Harvested and converted (ha) | – | – | 6.60 | – | – | – |
|  | Net change (tC) | – | – | –1.43 | –0.07 | 0.00 | 0.01 |
| CEF – 36 | Newly established (ha) | – | – | – | – | – | 225.10 |
|  | Harvested and converted (ha) | – | – | 104.12 | 59.62 | 32.65 | – |
|  | Net change (tC) | – | – | –22.62 | –15.03 | –8.16 | 0.37 |
| CEF – 35 | Newly established (ha) | – | – | – | – | – | 9.72 |
|  | Harvested and converted (ha) | – | 6.11 | – | – | – | – |
|  | Net change (tC) | – | –1.32 | 0.00 | 0.00 | 0.00 | 0.01 |
| CEF – 15 | Newly established (ha) | – | – | – | 194.01 | – | – |
|  | Harvested and converted (ha) | – | – | 47.83 | 89.18 | – | – |
|  | Net change (tC) | – | – | –10.38 | –24.11 | 0.07 | 0.18 |
| CEF – 2 | Newly established (ha) | – | – | – | 302.95 | – | – |
|  | Harvested and converted (ha) | 5.70 | 62.70 | 148.37 | 56.57 | 27.28 | – |
|  | Net change (tC) | –1.22 | –13.40 | –31.69 | –16.83 | –6.75 | 0.35 |
| CEF – 24 | Newly established (ha) | – | – | – | 22.47 | – | – |
|  | Harvested and converted (ha) | – | – | – | 17.89 | – | – |
|  | Net change (tC) | – | – | – | –4.52 | 0.00 | 0.01 |
| CEF – 40 | Newly Established (ha) | – | – | – | – | – | 36.57 |
|  | Harvested and Converted (ha) | – | – | – | 36.08 | – | – |
|  | Net change (tC) | – | – | – | –9.13 | 0.02 | –0.21 |
| CEF – 38 | Newly established (ha) | – | – | – | – | – | 11.35 |
|  | Harvested and converted (ha) | – | – | – | 10.37 | – | – |
|  | Net change (tC) | – | – | – | –2.62 | 0.01 | –0.08 |
| CEF – 42 | Newly established (ha) | – | – | – | – | – | 86.55 |
|  | Harvested and converted (ha) | – | – | – | – | 82.96 | – |
|  | Net change (tC) | – | – | – | – | –21.00 | –0.03 |
| CEF – 43 | Newly established (ha) | – | – | – | – | – | 49.57 |
|  | Harvested and converted (ha) | – | – | – | – | – | 41.78 |
|  | Net change (tC) | – | – | – | – | – | –10.75 |
| CEF – 39 | Newly established (ha) | – | – | – | – | – | 135.53 |
|  | Harvested and converted (ha) | – | – | 7.58 | 103.48 | – | – |
|  | Net change (tC) | – | – | –1.65 | –26.19 | 0.07 | –0.17 |
| CEF – 44 | Newly established (ha) | – | – | – | – | – | 20.75 |
|  | Harvested and converted (ha) | – | – | – | – | 19.63 | – |
|  | Net change (tC) | – | – | – | – | –4.97 | 0.03 |
| CEF – 9 | Newly established (ha) | – | – | 26.15 | – | – | – |
|  | Harvested and Converted (ha) | – | 4.01 | 19.49 | – | – | – |
|  | Net change (tC) | – | –0.86 | –4.14 | 0.00 | 0.02 | 0.04 |
| CEF – 13 | Newly established (ha) | – | – | 111.53 | – | – | – |
|  | Harvested and converted (ha) | – | 1.61 | 106.49 | – | – | – |
|  | Net change (tC) | – | –0.35 | –22.65 | 0.02 | 0.08 | 0.19 |
| CEF – 12 | Newly established (ha) | – | – | 168.21 | – | – | – |
|  | Harvested and converted (ha) | – | – | 167.54 | – | – | – |
|  | Net change (tC) | – | – | –35.69 | 0.03 | 0.13 | 0.28 |
| CEF – 21 | Newly established (ha) | – | – | – | 180.17 | – | – |
|  | Harvested and converted (ha) | – | 1.78 | 67.81 | 104.54 | – | – |
|  | Net change (tC) | – | –0.38 | –14.50 | –26.23 | 0.03 | 0.13 |
| CEF – 41 | Newly established (ha) | – | – | – | – | – | 4.58 |
|  | Harvested and converted (ha) | – | – | – | – | 6.78 | – |
|  | Net change (tC) | – | – | – | – | –1.72 | 0.01 |
| CEF – 14 | Newly established (ha) | – | – | – | 153.61 | – | – |
|  | Harvested and converted (ha) | – | 2.42 | 148.44 | – | – | – |
|  | Net change (tC) | – | –0.53 | –32.19 | 0.14 | 0.03 | 0.12 |
| CEF – 18 | Newly established (ha) | – | – | – | – | 130.00 | – |
|  | Harvested and converted (ha) | – | 5.00 | 124.80 | – | – | – |
|  | Net change (tC) | – | –1.09 | –27.11 | 0.09 | –0.07 | 0.03 |
| TOTAL | Newly established (ha) | – | – | 612.34 | 1,685.68 | 1,795.02 | 579.73 |
|  | Harvested and converted (ha) | 52.02 | 543.40 | 1,470.80 | 1,181.27 | 404.40 | 56.39 |
|  | Net change (tC) | –11.17 | –116.89 | –315.71 | –302.84 | –100.36 | –12.12 |

Note: 	CEF = carbon equivalent forest; ha = hectares; tC = tonnes carbon.

## Annex 5: References

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4. Agriculture, Forestry and Other Land Use. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

IPCC. 2014. Hiraishi T, Krug T, Tanabe K, Srivastava N, Baasansuren J, Fukuda M, Troxler TG (eds). 2013 Revised Supplementary Methods and Good Practice Guidance Arising from the Kyoto Protocol. Switzerland: IPCC.

Ministry for Primary Industries. 2009. National Exotic Forest Description as at 1 April 2009. Wellington: Ministry for Primary Industries.

Ministry for Primary Industries. 2015. Log and Roundwood Removal Statistics. Retrieved from: www.mpi.govt.nz/news-and-resources/statistics-and-forecasting/forestry (July 2015).

Ministry for the Environment. 2015. New Zealand’s Greenhouse Gas Inventory 1990–2013. Wellington: Ministry for the Environment.

New Zealand Government. 2011. Forest Management Reference Level Submission. Retrieved from unfccc.int/files/meetings/ad_hoc_working_groups/kp/application/pdf/newzealand_frml.pdf (22 February 2016).

UNFCCC. 2011. Report of the technical assessment of the forest management reference level submission of New Zealand submitted in 2011. Retrieved from http://unfccc.int/ resource/docs/2011/tar/nzl01.pdf (22 February 2016).

UNFCCC. 2012. Report of the Conference of the Parties serving as the meeting of the Parties to the Kyoto Protocol on its seventh session, held in Durban from 28 November to 11 December 2011: Addendum – Part 2: Action taken by the Conference of the Parties serving as the meeting of the Parties to the Kyoto Protocol at its seventh session. FCCC/KP/CMP/2011/10/Add.1.

UNFCCC. 2019. Report on the individual review of the annual submission of New Zealand submitted in 2019. Note by the expert review team. FCCC/ARR/2019/NZL.

# Annex 6: Additional information on the inventory system and completeness

## A6.1 	Quality assurance and quality control processes

The quality assurance and quality control (QA/QC) processes have a significant role in the preparation of the inventory, to ensure the core principles of transparency, accuracy, completeness, comparability and consistency are achieved. Table A6.1.1 describes the main QA/QC processes used in the preparation of the inventory. These processes are under continual review and improvement to ensure they are fit for purpose.

Table A6.1.1	Quality assurance and quality control processes used in preparation of the inventory

| ID | QA/QC process or activity description |
| --- | --- |
| QA file | All external reviews of the whole or part of the inventory are documented in the QA file. Reviews are performed by qualified personnel, and the review records are included in the submission of the inventory to the United Nations Framework Convention on Climate Change. These reviews help identify improvements to the inventory. |
| QC 1 | Planned recalculations and improvements are approved by the reporting governance group that oversees all climate change reporting by the New Zealand Government. The role of this group is further described in chapter 1. |
| QC 2 | Planned improvements are peer reviewed before being implemented, when they affect the emission factor, parameter, methodology or activity data source. Some sectors have a dedicated panel of experts that review improvements. |
| QC 3 | Tier 1 checklist QC sheets are completed to ensure transparency, accuracy, completeness, comparability and consistency principles are met. Examples are included in the submission of the inventory. |
| QC 4 | The chapter text for each sector is peer reviewed and follows the checklist provided, to ensure that the peer review is comprehensive and consistent. |
| QC 5 | Recalculations that exceed a certain threshold (see figure A6.1.1) are analysed and clearly documented. This includes changes resulting from planned improvements, errors, recommendations from the expert review team, and changes to guidelines. |
| QC 7 | All sectors in the inventory are approved by the applicable member of the reporting governance group that oversees all international climate change reporting by the New Zealand Government before being submitted to the National Inventory Compiler. |
| QC 10 | Common reporting format QC tools identify any potential issues with the data and are used to ensure the data integrity standards are met. |
| Sector submission checks | Sector submissions are checked against the data integrity standards and chapter formatting standards by the inventory agency before sector submission. Any issues must be resolved before submitting. This enables the remainder of the inventory compilation to proceed smoothly because quality is assured. |

Figure A6.1.1 shows how these QA/QC processes align with the overall preparation of the inventory.

Figure A6.1.1 	How the quality assurance and quality control processes and products align with the preparation of the inventory

Planned improvements	Sectoral compilation

Plus unplanned improvements

Prepare common reporting format data and report sections, and carry out improvements as applicable.

Sector quality control:

- QC 3 Tier 1 checklist for common reporting format data

- QC 4 chapter checklist for peer review of chapter

- QC 10 CRF QC tools for data quality.

Deliver sector to the National Inventory Compiler:

- QC 7 approval of sector from reporting governance group manager

- include QC 2 and QC 5 forms as applicable.

Start with an improvement. External reviews as documented in the ‘QA file’ are key sources of improvements.

Get improvements peer reviewed and complete a QC 2 peer-review change form if it affects:

- emission factor, parameter, methodology or activity data source.

Include all planned improvements in QC 1 planned recalculations and improvements for the reporting governance group.

Fill out QC 5 recalculation form if it is a change to:

- emissions >500 kt CO2-e

- methodology

- an emission factor

- emissions that result in the trend reversing.

Unplanned improvements, including recommendations from the expert review team, and resolution of any other issues as required.

Sector undergoes quality checks by the inventory agency.

Chapter and data checked against the chapter formatting standards and data integrity standards respectively, any issues are resolved before receiving confirmation of quality from the inventory agency.

Figure A6.1.2 presents an overview of the compilation process for Tokelau, and it is integrated into New Zealand’s inventory. It also shows where QA/QC steps are applied.

Figure A6.1.2 	Data processing, quality assurance and quality control processes during the inventory preparation of the data from Tokelau into New Zealand’s inventory

| CRF | Common reporting format |
| --- | --- |
| MFAT | Ministry of Foreign Affairs and Trade |
| MfE | Ministry for the Environment (New Zealand) |
|  | QA/QC procedures performed by Tokelau NSO |
|  | QA/QC procedures performed by MfE |
|  | QA/QC procedures performed by third parties |
| NIR | National inventory report |
| NSO | National Statistics Office (Tokelau) |
| QA/QC | Quality assurance and quality control |
| RGG | Reporting Governance Group |

## A6.2 	General assessment of completeness

### A6.2.1	Emissions reported as ‘NE’ (not estimated)

According to the United Nations Framework Convention on Climate Change (UNFCCC) reporting guidelines (UNFCCC, 2013), the notation key ‘NE’ (not estimated) signifies that emissions and/or removals occur but have not been estimated or reported. It can be applied for the following reasons:

if emissions of a gas from a category are insignificant, that is, they should not exceed 0.05 per cent of the national total greenhouse gas (GHG) emissions, and do not exceed 500 kilotonnes carbon dioxide equivalent (kt CO2-e) (paragraph 37(b) of the UNFCCC reporting guidelines)

the total national aggregate of estimated emissions for all gases and categories considered insignificant shall remain below 0.1 per cent of the national total GHG emissions (paragraph 37(b) of the UNFCCC reporting guidelines)

- when an activity occurs in the Party but the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2006) do not provide methodologies to estimate emissions and removals (footnote 6 of the UNFCCC reporting guidelines (UNFCCC, 2013)). If this is the case, the category is considered to be non-mandatory, providing the emissions from the category have not been reported previously.

The UNFCCC reporting guidelines also state that, once emissions from a specific category have been reported in a previous submission, emissions from this specific category shall be reported in subsequent GHG inventory submissions (UNFCCC, 2013).

New Zealand’s gross emissions were 82,317.9 kt CO2-e in 2019. The threshold of 0.1 per cent for New Zealand’s 2021 submission is 82.3 kt CO2-e and the threshold of 0.05 per cent is 41.2 kt CO2-e. Both values are below 500 kt CO2-e.

Table A6.2.1 summarises New Zealand’s direct GHG emissions reported as ‘NE’ (not estimated) in the 2021 submission.

Table A6.2.1 	Summary of NE (not estimated) entries in 2021 submission

| CRF category code | Category | Gas | Explanation |
| --- | --- | --- | --- |
| Energy |  |  |  |
| 1.B.1.a.1.iii | Abandoned underground mines | CO2, CH4 | Methane (CH4) emissions from this category do not occur in the North Island of New Zealand and are not estimated for the South Island. Because the historical information is not available, New Zealand does not have any reliable information on activities related to CH4 emissions from abandoned mines to reliably report on it. A project focusing on collating and digitising mine data for the South Island commenced in December 2019 and is ongoing. The data still requires significant manual processing before it will be usable for a meaningful assessment of fugitive emissions. Further progress will be reported in the 2022 submission. |
| 1.B.2.a.5 | Distribution of oil products | CO2, CH4 | According to the article 37(b) of the United Nations Framework Convention on Climate Change (UNFCCC) reporting guidelines (Decision 24/CP.19), this category is not mandatory: the 2006 IPCC Guidelines do not provide the default Intergovernmental Panel on Climate Change (IPCC) emission factor for calculating Tier 1 estimates of CH4 emissions from the distribution of refined oil products. New Zealand did not report an emissions estimate from this category prior to the 2018 submission. |
| 1.B.2.b.3 | Processing | CO2, CH4 | Fugitive emissions of carbon dioxide (CO2) and CH4 have not been formally estimated, though a rough estimate of the likely level of emissions indicates that they are insignificant. While emissions from the Kapuni Gas Treatment Plant may include traces of CH4, the level of these emissions has been determined to be insignificant in comparison with national emissions: a conservative estimate (using default emission factors from the 2006 IPCC Guidelines) gives approximately 1.5 kilotonnes carbon dioxide equivalent (kt CO2-e) per year. CH4: 625 Mm3 (Kapuni field production) * 9.7e-5 * 25 = 1.5 kt CO2-e. The conservative estimated value is below 0.05 per cent of New Zealand’s gross emissions. This would keep the national total aggregate of estimated emissions for all gases and categories considered insignificant below 0.1 per cent of the national total greenhouse gas emissions, which is in line with paragraph 37(b) of the UNFCCC reporting guidelines. Carbon dioxide from gas processing is mostly associated with direct venting through a stack and, therefore, is reported under 1.B.2.c.1, as recommended in the 2017 assessment review report. However, there is a possibility of the presence of trace amounts of CO2 from processing due to leakage, which is estimated to be no higher than 0.1 per cent of vented CO2. A conservative estimate of 0.1 per cent of vented CO2 from all categories is 0.26 kt, which is below 0.05 of the gross emissions and thus can be considered insignificant. |
| 1.B.2.c.1.ii | Venting | CH4 | No meaningful activity data are available. According to the article 37(b) of the UNFCCC reporting guidelines, this category is not mandatory: the 2006 IPCC Guidelines do not provide the default IPCC emission factor for calculating Tier 1 estimates of CH4 emissions from raw CO2 venting. |
| Agriculture |  |  |  |
| 3.A.4 (for both New Zealand and Tokelau) | Poultry | CH4 | According to the article 37(b) of the UNFCCC reporting guidelines, this category is not mandatory: the 2006 IPCC Guidelines state (page 10.27, vol 4-2) that the Tier 1 method for estimating CH4 emissions from enteric fermentation for poultry is not developed. Also, table 10.10 (page 10.28, vol 4-2) indicates that there is insufficient research to establish a CH4 emission factor for poultry for either developed or developing countries. |
| 3.B.2.5 | Indirect N2O emissions | N2O | According to footnote 6 in paragraph 37(b) of the UNFCCC reporting guidelines (Decision 24/CP.19), this category is not mandatory for reporting. The 2006 IPCC Guidelines for determining indirect nitrous oxide (N2O) emissions do not provide a methodology for estimating emissions from leaching and run-off. In addition, indirect N2O emissions from leaching and run-off are insignificant in New Zealand, because almost all livestock are kept outdoors all year around on pasture. |
| 3.B.2.5 | N2O emissions per MMS | N2O | Direct N2O emissions from anaerobic lagoons (dairy and swine) and daily spread (swine) are reported under Agricultural soils. The 2006 IPCC Guidelines assume that negligible direct N2O emissions occur in anaerobic lagoons and daily spread, and only occur once the stored effluent is spread onto agricultural soil. For more information, see chapter 5, sections 5.3.2 (Direct nitrous oxide emissions from manure management) and 5.5.2 (Urine and dung deposited by grazing animals) of the NIR. According to footnote 6 in paragraph 37(b) of the UNFCCC reporting guidelines (Decision 24/CP.19), this category is not mandatory for reporting. |
| 3.D.1.2.c | Other organic fertilisers applied to soils | N2O | Emissions from ‘Other organic fertilisers applied to soils’ are not estimated due to their insignificance as defined in accordance with the UNFCCC reporting guidelines (Decision 24/CP.19, paragraph 37(b)). Emissions are roughly estimated to be 20 kt CO2-e (van der Weerden et al., 2014). Emissions are below the threshold of 0.05 per cent of the national total greenhouse gas emissions and do not exceed 500 kt CO2-e. |
| 3.I | Other carbon-containing fertilisers | CO2 | According to the UNFCCC reporting guidelines (Decision 24/CP.19, paragraph 37), this category is not mandatory because the 2006 IPCC Guidelines do not provide guidance for reporting on other carbon-containing fertilisers. Other carbon-containing synthetic fertilisers besides limestone, dolomite and urea are not applied to agricultural land in New Zealand. |
| Land Use, Land-Use Change and Forestry |  |  |  |
| 4.D.1 | Forest land, cropland, grassland and wetlands: Drainage and rewetting and other management of organic and mineral soils | CH4, N2O | No methodology is provided in the 2006 IPCC Guidelines for estimating emissions from this source category. According to footnote 6 in paragraph 37(b) of the UNFCCC reporting guidelines (Decision 24/CP.19), this category is not mandatory for reporting. |
| 4.B.1 | Cropland remaining cropland/4(V) Biomass burning/ Wildfires/Cropland remaining cropland | CH4, N2O | New Zealand does not have sufficient information on biomass burning activities to reliably report on it. |
| 4.B.2 | Land converted to cropland/4(V) Biomass burning/ Wildfires/Land converted to cropland | CH4, N2O | New Zealand does not have sufficient information on biomass burning activities to reliably report on it. |
| 4.D.1 | Wetlands remaining wetlands/4(V) Biomass burning/ Wildfires/Wetland remaining wetland | CH4, N2O | According to the article 37(b) of the UNFCCC reporting guidelines (Decision 24/CP.19), this category is not mandatory because no IPCC guidance is provided for calculating Tier 1 estimates of carbon stock changes in organic soils for this land use category. New Zealand does not have sufficient information on biomass burning activities to reliably report on it. |
| Waste |  |  |  |
| 5.C.2.2.a | Incineration of municipal solid waste | CO2, CH4 and N2O | Approximately 100–200 rural schools in New Zealand still incinerate their waste production. Estimates indicate that this practice emits 0.04 kt CO2-e per year. NE (not estimated) is used because New Zealand does not have sufficient information regarding the practice of incinerating waste in schools, and the amount is negligible. This is in accordance with paragraph 37(b) of the UNFCCC reporting guidelines (Decision 24/CP.19). |
| 5.D.1 and 5.D.2 | Domestic wastewater and Industrial wastewater | Amount of CH4 flared and for energy recovery | NE (not estimated) is used for activity data, because New Zealand does not have any information regarding the CH4 flaring in this source category. The amount of CH4 flared does not contribute to New Zealand’s total emissions because it produces biogenic CO2 (as per the 2006 IPCC Tier 1 methodology provided in table 5D of the common reporting format tables). |

The estimate of emissions for all of New Zealand’s source categories marked as ‘NE’ results in 21.8 kt CO2-e, which is below the 0.1 per cent of the total emissions threshold (82.3 kt CO2-e).

### A6.2.2	Emissions reported as ‘IE’ (included elsewhere)

According to the UNFCCC reporting guidelines (UNFCCC, 2013), the notation key ‘IE’ (included elsewhere) signifies that emissions and/or removals for this activity or category are estimated and included in the inventory but not presented separately for this category.

Table A6.2.2 details where the notation key ‘IE’ has been used in this submission of the inventory.

Table A6.2.2 	Emissions reported using the ‘IE’ (included elsewhere) notation key

| CRF category code | Category | Reported under the following source category: | Notation key explanation |
| --- | --- | --- | --- |
| 1.A.2.a | Iron and steel – liquid fuels | 1.A.2.g.viii – Other – Liquid fuels | Liquid fuels activity data for this category do not exist. |
| 1.A.2.a | Iron and steel – solid fuels | 2.C.1 – Iron and steel production | All emissions from the use of coal in this category are included in the Industrial Processes and Product Use sector because the primary purpose of the coal is to produce iron. |
| 1.A.2.f | Non-metallic minerals – biomass | 1.A.2.g.viii – Other – Biomass | Activity data for this category do not exist. |
| 1.A.2.g.v | Construction – all fuels | 1.A.2.g.iii – Mining | Disaggregated data do not exist. |
| 1.A.3.b.ii–iv | Road transportation (other than ‘Cars’) – all fuels | 1.A.3.b.i – Cars | Disaggregated data do not exist. Disaggregation of carbon dioxide (CO2) emissions is included in the plan, but the implementation has not yet been completed. |
| 1.A.4.c.ii–iii | Agriculture/forestry/fishing – Off-road vehicles and other machinery | 1.A.4.c.i – Agriculture/ forestry/fishing – Stationary | Agriculture/forestry/fishing has not been disaggregated into stationary, mobile and fishing: data are not available. |
| 1.B.2.b.1 | Natural gas/exploration | 1.B.2.a.1 – Oil exploration | In New Zealand, exploration is not specifically aimed at obtaining oil or gas, that is, oil exploration is not separated from gas exploration by planning, processes, equipment or resources. Thus the exploratory wells are drilled without distinction of their purpose, that is, whether the expected outcome is oil, gas, both or none and there is no reliable way to predict which it would be to estimate proportions of mostly oil and mostly gas wells. In that sense, disaggregated data for oil and gas exploration do not exist. Considering that available emission factors for well drilling and testing also do not distinguish between oil and gas, all emissions from oil and gas exploration are placed in the same category. |
| 1.B.2.c.1.i–ii | Venting/oil and Venting/gas | 1.B.2.c.1.iii – Venting/combined | The fields produce both oil and gas and, therefore, are reported as combined. Disaggregated data do not exist. |
| 1.B.2.c.1.i–ii | Flaring/oil and Flaring/gas | 1.B.2.c.1.iii – Flaring/combined | The fields produce both oil and gas and, therefore, are reported as combined. Disaggregated data do not exist. |
| 2.A.3 | Glass production | 2.A.4.b – Other process uses of carbonates/Other uses of soda ash | Carbon dioxide emissions are reported in 2.A.4.b because this aggregates emissions from glass production with other uses of carbonates, due to confidentiality concerns for both glass and aluminium production. A very small number of firms in New Zealand are involved in these activities and use carbonates. |
| 3.A.4 | Enteric fermentation/ other/buffalo | 3.A.1.A – Dairy cattle | A small herd of around 200 buffalo was brought into New Zealand around 2007 for specialised cheese and dairy production. These buffalo are reported within the dairy herd so the notation key ‘IE’ is used from 2007 onwards. |
| 3.B.1.4 & 3.B.2.4 | Manure management/ other/buffalo | 3.B.1.A – Dairy cattle 3.B.2.A – Dairy cattle | For both nitrous oxide (N2O) and CH4 emissions, the notation key ‘NO’ (not occurring) is used up to 2006 because no buffalo were recorded in New Zealand before 2007. A small herd of around 200 buffalo was brought into New Zealand around 2007 for specialised cheese and dairy production. See notation key explanation for 3.A.4. For more information, see chapter 5, section 5.1.4 (Minor livestock categories) of this national inventory report (NIR). |
| 3.B.2.5 | N2O emissions per MMS | 3.D – Agricultural soils | Direct N2O emissions from anaerobic lagoons (dairy and swine) and daily spread (swine) are reported under Agricultural soils. |
| 3.D.1.2.b | Sewage sludge applied to soils | Included under the Waste sector 5.A.1.a | Direct N2O emissions from sewage sludge are reported under 5.A.1.a in the Waste sector. Sewage sludge activity data are obtained from water treatment industry surveys and do not disaggregate the amount of sludge used for different purposes. Due to the small amount of emissions coming from sewage sludge, further disaggregation of the activity data is considered resource prohibitive. Sewage sludge is a very small source of nitrogen (van der Weerden et al., 2014). |
| 3.E | Prescribed burning of savannas | Biomass burning (table 4(V) of LULUCF), category C Grassland | Prescribed burning of savanna is reported under the Land Use, Land-Use Change and Forestry (LULUCF) sector. See chapter 6, section 6.11.5 (Biomass burning (table 4(V) of LULUCF), category C Grassland). |
| 4.A.1/4(V) | Controlled burning/Forest land remaining forest land | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. |
| 4.A.2/4(V) | Controlled burning/Land converted to forest land Wildfires/Land converted to forest land | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. |
| 4.B.1/4(V) | Controlled burning/Cropland remaining cropland | Included under the Agriculture sector | Carbon dioxide and CH4 emissions from burning of crop stubble are reported in the Agriculture sector. |
| 4.B.1/4(V) | Wildfires/Cropland remaining cropland | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
| 4.B.2/4(V) 4.C.1/4(V) 4.C.2/4(V) 4.D.2/4(V) | Wildfires/Land converted to cropland Wildfires/Grassland remaining grassland Wildfires/Land converted to grassland Wildfires/Land converted to wetlands | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
|  | Direct N2O emissions from nitrogen (N) inputs to managed soils/Inorganic N fertilisers and Direct N2O emissions from N inputs to managed soils/Organic N fertilisers from the following categories: | Included under the Agriculture sector. | New Zealand does not disaggregate data on nitrogen fertiliser by land use, therefore, all N2O emissions from organic and inorganic fertilisers are reported in the Agriculture sector. |
| 4.A.1/4(I) | Forest land remaining forest land | Included under the Agriculture sector. Included under the Agriculture sector. | New Zealand does not disaggregate data on nitrogen fertiliser by land use, therefore, all N2O emissions from organic and inorganic fertilisers are reported in the Agriculture sector. All emissions from burning of crop stubble are reported in the Agriculture sector. |
| 4.D.1/4(I) | Wetlands remaining wetlands Land converted to wetlands |  |  |
| 4.D.2/4(I) | Settlements remaining settlements Land converted to settlements |  |  |
| 4.E.1/4(I) | Settlements remaining settlements |  |  |
| 4.E.2/4(I) | Land converted to settlements |  |  |
| 4.B.1/4(V) | Controlled burning/Cropland remaining cropland |  |  |
| 4.C.1/4(V) 4.D.1/4(V) | Controlled burning/Grassland remaining grassland Controlled burning/Wetland remaining wetland Wildfires/Wetland remaining wetland | If due to temperate climate and rainfall, any CO2 emissions from burning on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the fire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | This is not a significant activity in New Zealand due to its temperate climate and rainfall distribution, and any CO2 emissions from burning on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the fire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
| 4.C.2/4(V) 4.D.2/4(V) 4.E/4(V) | Controlled burning/Land converted to grassland Controlled burning/Land converted to wetlands Biomass burning/Land converted to settlements | Carbon dioxide emissions from the controlled burning of land converted to this category are captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | Carbon dioxide emissions from the controlled burning of land converted to this category are captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
| 5.D.1 | Domestic wastewater | 5.A Solid waste | Activity data – sludge amounts are included under solid waste disposal because sludge is disposed to landfill. |
| 5.D.2 | Industrial wastewater | 5.A Solid waste | Activity data – sludge amounts are included under solid waste disposal because sludge is disposed to landfill. |
| 5.D.2 | Industrial wastewater | 1.A.2.e Food processing, beverages and tobacco – Biomass | Emissions from CH4 recovered for energy generation are known to occur at the Tirau dairy processing plant, and emissions of CH4 and N2O from the combustion of biogas are reported under 1.A.2.e Food processing, beverages and tobacco – Biomass. |
| Within the Tokelau sector 6, categories 1.A.3.b.i and 1.A.4.c.iii were reported elsewhere | Road transport/Gasoline and diesel oil | Domestic navigation | The number of petrol cars has, until recently, been very small in Tokelau (in 2018 only about 40 cars, 30 motorbikes, with an entire road network less than 10 kilometres). Census 2001 and prior record only four registered cars. Aluminium boats are the main means of family transport: there were, on average, about 100 outboard motors travelling both outside and within the large lagoons. Therefore, any petrol use for road transport is far outweighed by Domestic navigation, and is included there. |
| Within the Tokelau sector 6, category 1.A.4.b is reported elsewhere | Residential (1.A.4.b) liquid fuels | Domestic navigation | Only gas used for cooking is listed here. Amounts of liquid fuel use are miniscule compared with Domestic navigation and are included there. |

## Annex 6: References

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

UNFCCC. 2013. FCCC/CP/2013/10/Add.3. Report of the Conference of the Parties on its nineteenth session, held in Warsaw from 11 to 23 November 2013, Addendum; Decision 24/CP.19 Revision of the UNFCCC reporting guidelines on annual inventories for Parties included in Annex I to the Convention.

van der Weerden A, de Klein C, Kelliher F, Rollo M. 2014. Reporting to 2006 IPCC Guidelines for N2O Emissions from Additional Sources of Organic N: Final Report. MPI Technical Report. Wellington: Ministry for Primary Industries.

# Annex 7: Tokelau

## A7.1	Emissions estimate data and relevant supporting information by category for Tokelau

Tokelau CRF Table 1.A.1.a: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 |
| Liquid fuels | TJ | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 | 3.27 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 |
| Liquid fuels | kt | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 | 0.23 |
| CH4 | kt | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 |
| Liquid fuels | kt | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 |
| N2O | kt | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 |
| Liquid fuels | kt | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 |

Tokelau CRF Table 1.A.1.a: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 3.27 | 3.27 | 3.27 | 3.27 | 9.81 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 |
| Liquid fuels | TJ | 3.27 | 3.27 | 3.27 | 3.27 | 9.81 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.23 | 0.23 | 0.23 | 0.23 | 0.69 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |
| Liquid fuels | kt | 0.23 | 0.23 | 0.23 | 0.23 | 0.69 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |
| CH4 | kt | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000279 | 0.0000466 | 0.0000466 | 0.0000466 | 0.0000466 | 0.0000466 |
| Liquid fuels | kt | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000279 | 0.0000466 | 0.0000466 | 0.0000466 | 0.0000466 | 0.0000466 |
| N2O | kt | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000056 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 |
| Liquid fuels | kt | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000019 | 0.0000056 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 | 0.0000093 |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 |

Tokelau CRF Table 1.A.1.a: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 16.34 | 16.34 | 12.97 | 2.86 | 2.86 | 2.86 | 3.05 | 3.24 | 3.42 | 3.61 |
| Liquid fuels | TJ | 16.34 | 16.34 | 12.97 | 2.86 | 2.86 | 2.86 | 3.05 | 3.24 | 3.42 | 3.61 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.15 | 1.15 | 0.91 | 0.20 | 0.20 | 0.20 | 0.21 | 0.23 | 0.24 | 0.25 |
| Liquid fuels | kt | 1.15 | 1.15 | 0.91 | 0.20 | 0.20 | 0.20 | 0.21 | 0.23 | 0.24 | 0.25 |
| CH4 | kt | 0.0000466 | 0.0000466 | 0.0000370 | 0.0000082 | 0.0000082 | 0.0000082 | 0.0000087 | 0.0000092 | 0.0000098 | 0.0000103 |
| Liquid fuels | kt | 0.0000466 | 0.0000466 | 0.0000370 | 0.0000082 | 0.0000082 | 0.0000082 | 0.0000087 | 0.0000092 | 0.0000098 | 0.0000103 |
| N2O | kt | 0.0000093 | 0.0000093 | 0.0000074 | 0.0000016 | 0.0000016 | 0.0000016 | 0.0000017 | 0.0000018 | 0.0000020 | 0.0000021 |
| Liquid fuels | kt | 0.0000093 | 0.0000093 | 0.0000074 | 0.0000016 | 0.0000016 | 0.0000016 | 0.0000017 | 0.0000018 | 0.0000020 | 0.0000021 |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 7.40 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 |

Tokelau CRF Table 1.A.3.b.i: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] (Part 1 of 3)

| [1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

Note:	This category is included under 1.A.3.d. For explanation please refer to section 8.2.5.

Tokelau CRF Table 1.A.3.b.i: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] (Part 2 of 3)

| [1.A.3 Transport] [1.A.3.b Road Transportation] [1.A.3.b.i Cars][Gasoline] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/T | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

Tokelau CRF Table 1.A.3.b.i: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] (Part 3 of 3)

| [1.A.3 Transport] [1.A.3.b Road Transportation] [1.A.3.b. Cars][Gasoline] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/T | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.b.i Diesel Oil: 	[1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport] [1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.b.i Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.b.i Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 12.76 | 12.98 | 13.21 | 13.43 | 13.66 | 13.89 | 14.11 | 14.34 | 14.56 | 14.79 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.90 | 0.91 | 0.93 | 0.95 | 0.96 | 0.98 | 0.99 | 1.01 | 1.03 | 1.04 |
| CH4 | kt | 0.00008 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00010 | 0.00010 | 0.00010 |
| N2O | kt | 0.00002 | 0.00002 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 | kg/TJ | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 |
| N2O | kg/TJ | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 |

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 15.01 | 15.24 | 15.47 | 15.69 | 15.92 | 16.14 | 16.37 | 16.59 | 16.82 | 17.04 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.06 | 1.07 | 1.09 | 1.10 | 1.12 | 1.14 | 1.15 | 1.17 | 1.18 | 1.20 |
| CH4 | kt | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 |
| N2O | kt | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 | kg/TJ | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 |
| N2O | kg/TJ | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 |

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 17.27 | 17.50 | 17.72 | 17.95 | 18.17 | 18.03 | 18.89 | 19.88 | 21.08 | 30.91 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.22 | 1.23 | 1.25 | 1.26 | 1.28 | 1.27 | 1.33 | 1.40 | 1.48 | 2.18 |
| CH4 | kt | 0.00011 | 0.00012 | 0.00012 | 0.00012 | 0.00012 | 0.00012 | 0.00013 | 0.00013 | 0.00014 | 0.00021 |
| N2O | kt | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00004 | 0.00004 | 0.00004 | 0.00006 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 | kg/TJ | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 |
| N2O | kg/TJ | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 |

CRF Table 1.A.4.b: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 |
| Liquid fuels | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | TJ | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Gaseous fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 |
| CH4 | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| N2O | kt | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | t/TJ | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |

CRF Table 1.A.4.b: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 |
| Liquid fuels | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | TJ | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Gaseous fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 |
| CH4 | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| N2O | kt | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | t/TJ | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |

CRF Table 1.A.4.b: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 1.25 |
| Liquid fuels | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | TJ | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 2.16 | 1.25 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Gaseous fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.07 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.12 | 0.07 |
| CH4 | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00007 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00007 |
| N2O | kt | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000002 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000004 | 0.0000002 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | t/TJ | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing] [1.A.4.c.iii Fishing]
[Gas/Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing][Gas/Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing] [Gas/Diesel Oil] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing][Gas/Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing]
[Gas/Diesel Oil] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.4 Other Sectors] [1.A.4.c Agriculture/Forestry/Fishing] [1.A.4.c.iii Fishing][Gas/Diesel Oil] | Unit | 2010 (kt CO2-equivalent) | 2011 (kt CO2-equivalent) | 2012 (kt CO2-equivalent) | 2013 (kt CO2-equivalent) | 2014 (kt CO2-equivalent) | 2015 (kt CO2-equivalent) | 2016 (kt CO2-equivalent) | 2017 (kt CO2-equivalent) | 2018 (kt CO2-equivalent) | 2019 (kt CO2-equivalent) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.AB Gasoline: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gasoline] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Gasoline] | Unit | Base year (1990) (kt CO2-equivalent) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| CO2 | kt | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |

CRF Table 1.AB Gasoline: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gasoline] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Gasoline] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| CO2 | kt | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |

CRF Table 1.AB Gasoline: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gasoline] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Gasoline] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.80 | 9.39 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.17 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.17 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.17 |
| CO2 | kt | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.62 |

CRF Table 1.AB Gas Diesel Oil: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Gas / Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 5.83 | 6.06 | 6.29 | 6.51 | 6.74 | 6.96 | 7.19 | 7.41 | 7.64 | 7.86 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.11 | 0.12 | 0.12 | 0.12 | 0.13 | 0.13 | 0.14 | 0.14 | 0.15 | 0.15 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.11 | 0.12 | 0.12 | 0.12 | 0.13 | 0.13 | 0.14 | 0.14 | 0.15 | 0.15 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.11 | 0.12 | 0.12 | 0.12 | 0.13 | 0.13 | 0.14 | 0.14 | 0.15 | 0.15 |
| CO2 | kt | 0.4104824 | 0.4263607 | 0.4422390 | 0.4581173 | 0.4739956 | 0.4898739 | 0.5057522 | 0.5216305 | 0.5375088 | 0.5533871 |

CRF Table 1.AB Gas Diesel Oil: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Gas / Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 8.09 | 8.32 | 8.54 | 8.77 | 15.53 | 22.29 | 22.52 | 22.74 | 22.97 | 23.19 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.16 | 0.16 | 0.16 | 0.17 | 0.30 | 0.43 | 0.43 | 0.44 | 0.44 | 0.45 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.16 | 0.16 | 0.16 | 0.17 | 0.30 | 0.43 | 0.43 | 0.44 | 0.44 | 0.45 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.16 | 0.16 | 0.16 | 0.17 | 0.30 | 0.43 | 0.43 | 0.44 | 0.44 | 0.45 |
| CO2 | kt | 0.5692654 | 0.5851437 | 0.6010220 | 0.6169003 | 1.0927211 | 1.5685419 | 1.5844201 | 1.6002984 | 1.6161767 | 1.6320550 |

CRF Table 1.AB Gas Diesel Oil: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Gas / Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.02 | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.02 | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 23.42 | 23.65 | 20.50 | 10.62 | 10.84 | 10.70 | 11.74 | 12.93 | 14.31 | 24.98 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.45 | 0.45 | 0.39 | 0.20 | 0.21 | 0.21 | 0.23 | 0.25 | 0.27 | 0.48 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.45 | 0.45 | 0.39 | 0.20 | 0.21 | 0.21 | 0.23 | 0.25 | 0.27 | 0.48 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.45 | 0.45 | 0.39 | 0.20 | 0.21 | 0.21 | 0.23 | 0.25 | 0.27 | 0.48 |
| CO2 | kt | 1.6479333 | 1.6638116 | 1.4425810 | 0.7471324 | 0.7630107 | 0.7529823 | 0.8262569 | 0.9095598 | 1.0067726 | 1.7579367 |

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Other Kerosene] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Other Kerosene] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Other Kerosene] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.0002 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.0002 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.67 | 0.23 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 | 18.62 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.004 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.004 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.004 |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.02 |

CRF Table 1.AB LPG: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Liquefied Petroleum Gases (LPG)] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 1.AB LPG: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels] [Liquefied Petroleum Gases (LPG)] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 1.AB LPG: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.49 | 1.02 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.06 |

CRF Table 1.AB Lubricants: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach] [Liquid Fuels] [Lubricants] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 1.AB Lubricants: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach] [Liquid Fuels] [Lubricants] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 1.AB Lubricants: [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion – Reference Approach][Liquid Fuels][Lubricants] | Unit | 2010 (kt CO2-equivalent) | 2011 (kt CO2-equivalent) | 2012 (kt CO2-equivalent) | 2013 (kt CO2-equivalent) | 2014 (kt CO2-equivalent) | 2015 (kt CO2-equivalent) | 2016 (kt CO2-equivalent) | 2017 (kt CO2-equivalent) | 2018 (kt CO2-equivalent) | 2019 (kt CO2-equivalent) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Imports | PJ | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0002 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0002 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.39 | 0.15 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 |
| CO2 | kt | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.01 |

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning] [ 2.F.1.b Domestic Refrigeration][ HFC-134a] (Part 1 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | 0.02 | 0.04 | 0.07 | 0.09 | 0.11 | 0.13 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning] [ 2.F.1.b Domestic Refrigeration][ HFC-134a] (Part 2 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] | Unit | 2000 (kt CO2-equivalent) | 2001 (kt CO2-equivalent) | 2002 (kt CO2-equivalent) | 2003 (kt CO2-equivalent) | 2004 (kt CO2-equivalent) | 2005 (kt CO2-equivalent) | 2006 (kt CO2-equivalent) | 2007 (kt CO2-equivalent) | 2008 (kt CO2-equivalent) | 2009 (kt CO2-equivalent) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.14 | 0.16 | 0.20 | 0.25 | 0.27 | 0.29 | 0.32 | 0.32 | 0.31 | 0.31 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.02 | 0.02 | 0.03 | 0.04 | 0.04 | 0.04 | 0.05 | 0.05 | 0.05 | 0.05 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.02 | 0.02 | 0.03 | 0.04 | 0.04 | 0.04 | 0.05 | 0.05 | 0.05 | 0.05 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] (Part 3 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.31 | 0.31 | 0.29 | 0.27 | 0.25 | 0.23 | 0.21 | 0.21 | 0.21 | 0.21 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air Conditioning]
(Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| HFCs | t CO2-e | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-32 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-125 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-134a | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFCs and PFCs | t CO2-e | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| Aggregated F-gases | t CO2-e | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning]
(Part 2 of 3)

| [2. Industrial Processes and Product Use] [2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| HFCs | t CO2-e | NO | NO | NO | NO | NO | NO | NO | 17.06 | 34.12 | 51.17 |
| HFC-32 | t | NO | NO | NO | NO | NO | NO | NO | 0.003 | 0.006 | 0.009 |
| HFC-125 | t | NO | NO | NO | NO | NO | NO | NO | 0.004 | 0.008 | 0.013 |
| HFC-134a | t | NO | NO | NO | NO | NO | NO | NO | 0.0003 | 0.001 | 0.001 |
| HFCs and PFCs | t CO2-e | NO | NO | NO | NO | NO | NO | NO | 17.06 | 34.12 | 51.17 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| Aggregated F-gases | t CO2-e | NO | NO | NO | NO | NO | NO | NO | 17.06 | 34.12 | 51.17 |

CRF Table 2.F.1.f: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning]
(Part 3 of 3)

| [2. Industrial Processes and Product Use] [2.F Product Uses as Substitutes for ODS] [2.F.1 Refrigeration and Air conditioning] [2.F.1.f Stationary Air conditioning] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| HFCs | t CO2-e | 68.23 | 85.29 | 102.35 | 119.41 | 136.46 | 153.52 | 170.58 | 170.58 | 170.58 | 170.58 |
| HFC-32 | t | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| HFC-125 | t | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| HFC-134a | t | 0.001 | 0.001 | 0.002 | 0.002 | 0.002 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 |
| HFCs and PFCs | t CO2-e | 68.23 | 85.29 | 102.35 | 119.41 | 136.46 | 153.52 | 170.58 | 170.58 | 170.58 | 170.58 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| Aggregated F-gases | t CO2-e | 68.23 | 85.29 | 102.35 | 119.41 | 136.46 | 153.52 | 170.58 | 170.58 | 170.58 | 170.58 |

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-32] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-32] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-32] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-32] | Unit | 2000 (kt CO2-equivalent) | 2001 (kt CO2-equivalent) | 2002 (kt CO2-equivalent) | 2003 (kt CO2-equivalent) | 2004 (kt CO2-equivalent) | 2005 (kt CO2-equivalent) | 2006 (kt CO2-equivalent) | 2007 (kt CO2-equivalent) | 2008 (kt CO2-equivalent) | 2009 (kt CO2-equivalent) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | 0.02 | 0.04 | 0.06 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-32] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-32] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.08 | 0.10 | 0.12 | 0.14 | 0.16 | 0.18 | 0.20 | 0.20 | 0.20 | 0.20 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-125: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-125] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-125] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-125: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-125] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-125] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | 0.03 | 0.06 | 0.08 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-125: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-125] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-125] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.11 | 0.14 | 0.17 | 0.19 | 0.22 | 0.25 | 0.28 | 0.28 | 0.28 | 0.28 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-134a] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] [HFC-134a] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | 0.002 | 0.004 | 0.01 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | 0.0003 | 0.001 | 0.001 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | 0.0003 | 0.001 | 0.001 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.001 | 0.001 | 0.002 | 0.002 | 0.002 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.001 | 0.001 | 0.002 | 0.002 | 0.002 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.4.a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] (Part 1 of 3)

| [2. Industrial Processes and Product Use] [2.F Product Uses as Substitutes for ODS] [2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | D | D | D | D | D |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  | NO | NO | NO | NO | NO | 0.0001 | 0.001 | 0.001 | 0.002 | 0.002 |
| HFCs | t CO2 equivalent | NO | NO | NO | NO | NO | 0.12 | 0.70 | 1.14 | 1.69 | 2.44 |
| HFC-134a | t | NO | NO | NO | NO | NO | 0.0001 | 0.0005 | 0.001 | 0.001 | 0.002 |
| HFC-227ea | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Aggregated F-gases | t CO2 equivalent | NO | NO | NO | NO | NO | 0.12 | 0.70 | 1.14 | 1.69 | 2.44 |

CRF Table 2.F.4.a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | D | D | D | D | D | D | D | D | D | D |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions | kt CO2-e | 0.003 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| HFCs | t CO2-e | 2.85 | 6.09 | 11.54 | 13.90 | 13.66 | 13.66 | 13.35 | 13.62 | 14.18 | 14.76 |
| HFC-134a | t | 0.002 | 0.004 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| HFC-227ea | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Aggregated F-gases | t CO2-e | 2.85 | 6.09 | 11.54 | 13.90 | 13.66 | 13.66 | 13.35 | 13.62 | 14.18 | 14.76 |

CRF Table 2.F.4.a: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | D | D | D | D | D | D | D | D | D | D |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions | kt CO2-e | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| HFCs | t CO2-e | 15.46 | 17.22 | 18.93 | 19.10 | 19.14 | 19.47 | 19.50 | 19.15 | 19.01 | 19.01 |
| HFC-134a | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| HFC-227ea | t | NO | 0.0004 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Aggregated F-gases | t CO2-e | 15.46 | 17.22 | 18.93 | 19.10 | 19.14 | 19.47 | 19.50 | 19.15 | 19.01 | 19.01 |

CRF Table 2.F.4.a HFC-134a: [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] (Part 1 of 3)

| [ 2. Industrial Processes and Product Use] [ 2.F Product Uses as Substitutes for ODS] [ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers] [ HFC-134a] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | 0.0001 | 0.0005 | 0.001 | 0.001 | 0.002 |
| Emissions | t | NO | NO | NO | NO | NO | 0.0001 | 0.0005 | 0.001 | 0.001 | 0.002 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | 0.0001 | 0.0005 | 0.001 | 0.001 | 0.002 |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.F.4.a HFC-134a: [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] (Part 2 of 3)

| [ 2. Industrial Processes and Product Use] [ 2.F Product Uses as Substitutes for ODS] [ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers] [ HFC-134a] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.002 | 0.004 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Emissions | t | 0.002 | 0.004 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.002 | 0.004 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.F.4.a HFC-134a: [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] (Part 3 of 3)

| [ 2. Industrial Processes and Product Use] [ 2.F Product Uses as Substitutes for ODS] [ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers] [ HFC-134a] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Emissions | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] (Part 1 of 3)

| [2. Industrial Processes and Product Use] [2.F Product Uses as Substitutes for ODS] [2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] [HFC-227ea] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] (Part 2 of 3)

| [2. Industrial Processes and Product Use] [2.F Product Uses as Substitutes for ODS] [2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] [HFC-227ea] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] (Part 3 of 3)

| [2. Industrial Processes and Product Use] [2.F Product Uses as Substitutes for ODS] [2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] [HFC-227ea] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | 0.0004 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Emissions | t | NO | 0.0004 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | 0.0004 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.G.3.a: [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] (Part 1 of 3)

| [2. Industrial Processes and Product Use] [2.G Other Product Manufacture and Use] [2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Activity data |  |  |  |  |  |  |  |  |  |  |  |
| N2O imported | kt | 0.0002 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | 0.0002 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| N2O | t/t | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 |

CRF Table 2.G.3.a: [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] (Part 2 of 3)

| [2. Industrial Processes and Product Use] [2.G Other Product Manufacture and Use] [2.G.3 N2O from Product Uses] [2.G.3.a Medical Applications] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Activity data |  |  |  |  |  |  |  |  |  |  |  |
| N2O imported | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00004 | 0.00004 | 0.00004 | 0.00005 | 0.00005 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00005 | 0.00004 | 0.00004 | 0.00005 | 0.00005 | 0.00005 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| N2O | t/t | 1.03 | 1.03 | 1.03 | 1.04 | 1.04 | 1.05 | 1.05 | 0.89 | 0.94 | 1.06 |

CRF Table 2.G.3.a: [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] (Part 3 of 3)

| [2. Industrial Processes and Product Use] [2.G Other Product Manufacture and Use] [2.G.3 N2O from Product Uses] [2.G.3.a Medical Applications] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Activity data |  |  |  |  |  |  |  |  |  |  |  |
| N2O imported | kt | 0.00005 | 0.00005 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | 0.00005 | 0.00005 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| N2O | t/t | 0.94 | 1.07 | 0.91 | 1.02 | 1.00 | 0.97 | 1.05 | 0.92 | 0.85 | 0.85 |

CRF Table 3.A.3 Tokelau Swine: [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] (Part 1 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.293 | 2.500 | 2.395 | 2.290 | 2.186 | 2.081 | 1.976 | 2.111 | 2.247 | 2.382 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.003 | 0.004 | 0.004 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.004 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| Feeding situation |  | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Swine: [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] (Part 2 of 3)

| [ 3. Agriculture][ 3.1 Livestock] [ 3.A Enteric Fermentation] [ 3.A.3 wine][ Other (please specify)] [ Tokelau_Swine] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.518 | 2.653 | 2.633 | 2.613 | 2.592 | 2.572 | 2.552 | 2.514 | 2.476 | 2.438 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| Feeding situation |  | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Swine: [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] (Part 3 of 3)

| [ 3. Agriculture][ 3.1 Livestock] [ 3.A Enteric Fermentation][ 3.A.3 Swine ] [ Other (please specify)][ Tokelau_Swine] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.400 | 2.362 | 2.219 | 2.076 | 1.933 | 1.790 | 1.647 | 1.647 | 1.647 | 1.647 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.004 | 0.004 | 0.003 | 0.003 | 0.003 | 0.003 | 0.002 | 0.002 | 0.002 | 0.002 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| Feeding situation |  | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.4 Tokelau Poultry [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] (Part 1 of 3)

| [3. Agriculture][ 3.1 Livestock] [ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 3.439 | 3.500 | 3.394 | 3.288 | 3.182 | 3.076 | 2.970 | 2.840 | 2.709 | 2.579 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Feeding situation |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.4 Tokelau Poultry [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] (Part 2 of 3)

| [3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.448 | 2.318 | 2.229 | 2.140 | 2.052 | 1.963 | 1.874 | 1.712 | 1.550 | 1.388 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Feeding situation |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.4 Tokelau Poultry [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] (Part 3 of 3)

| [3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock] [ Tokelau_Poultry] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 1.226 | 1.064 | 0.976 | 0.888 | 0.801 | 0.713 | 0.625 | 0.625 | 0.625 | 0.625 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Feeding situation |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.B.1.3 Tokelau Swine: [3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] (Part 1 of 3)

| [ 3. Agriculture][ 3.1 Livestock] [ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.293 | 2.500 | 2.395 | 2.290 | 2.186 | 2.081 | 1.976 | 2.111 | 2.247 | 2.382 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| VS daily excretion (average) | kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.04 | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 |

CRF Table 3.B.1.3 Tokelau Swine: [3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] (Part 2 of 3)

| [ 3. Agriculture][ 3.1 Livestock] [ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine] [ Other (please specify)][ Pigs] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.518 | 2.653 | 2.633 | 2.613 | 2.592 | 2.572 | 2.552 | 2.514 | 2.476 | 2.438 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| VS daily excretion (average) | kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 |

CRF Table 3.B.1.3 Tokelau Swine: [3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] (Part 3 of 3)

| [ 3. Agriculture][ 3.1 Livestock] [ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine] [ Other (please specify)][ Pigs] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.400 | 2.362 | 2.219 | 2.076 | 1.933 | 1.790 | 1.647 | 1.647 | 1.647 | 1.647 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 101.00 | 102.00 |
| Typical animal mass (average) | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| VS daily excretion (average) | kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 |

CRF Table 3.B.1.4 Tokelau Poultry: [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] (Part 1 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock] [ Tokelau_Poultry] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 3.439 | 3.500 | 3.394 | 3.288 | 3.182 | 3.076 | 2.970 | 2.840 | 2.709 | 2.579 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Cool | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Temperature | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| VS daily excretion (average) | dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 3.B.1.4 Tokelau Poultry: [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] (Part 2 of 3)

| [ 3. Agriculture][ 3.1 Livestock] [ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock] [ Tokelau_Poultry] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.448 | 2.318 | 2.229 | 2.140 | 2.052 | 1.963 | 1.874 | 1.712 | 1.550 | 1.388 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Cool | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Temperature | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| VS daily excretion (average) | dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00005 | 0.00004 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 3.B.1.4 Tokelau Poultry: [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] (Part 3 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock] [ Tokelau_Poultry] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 1.226 | 1.064 | 0.976 | 0.888 | 0.801 | 0.713 | 0.625 | 0.625 | 0.625 | 0.625 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Cool | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Temperature | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| VS daily excretion (average) | Kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00004 | 0.00003 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 5.A: [5. Waste][5.A Solid Waste Disposal] (Part 1 of 3)

| [5. Waste][5.A Solid Waste Disposal] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.39 | 0.39 | 0.39 | 0.39 | 0.38 | 0.38 | 0.38 | 0.38 | 0.37 | 0.37 |
| CO2 | kt | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.01 | 0.01 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |

CRF Table 5.A: [5. Waste][5.A Solid Waste Disposal] (Part 2 of 3)

| [5. Waste][5.A Solid Waste Disposal] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.37 | 0.37 | 0.36 | 0.36 | 0.35 | 0.35 | 0.34 | 0.33 | 0.32 | 0.31 |
| CO2 | kt | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |

CRF Table 5.A: [5. Waste][5.A Solid Waste Disposal] (Part 3 of 3)

| [5. Waste][5.A Solid Waste Disposal] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.31 | 0.31 | 0.30 | 0.30 | 0.30 | 0.30 | 0.30 | 0.30 | 0.31 | 0.31 |
| CO2 | kt | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |

CRF Table 5.A.3: [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] (Part 1 of 3)

| [5. Waste][5.A Solid Waste Disposal][5.A.3 Uncategorized Waste Disposal Sites] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Annual waste at the SWDS | kt | 0.54 | 0.53 | 0.53 | 0.53 | 0.52 | 0.52 | 0.52 | 0.52 | 0.51 | 0.51 |
| McF |  | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 |
| DOcF | % | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.01 | 0.01 |
| Amount of CH4 flared | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Amount of CH4 for energy re | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/t | 0.029 | 0.030 | 0.029 | 0.029 | 0.029 | 0.029 | 0.029 | 0.029 | 0.029 | 0.029 |

CRF Table 5.A.3: [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] (Part 2 of 3)

| [5. Waste][5.A Solid Waste Disposal][5.A.3 Uncategorized Waste Disposal Sites] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Annual waste at the SWDS | kt | 0.50 | 0.50 | 0.48 | 0.46 | 0.44 | 0.42 | 0.40 | 0.40 | 0.40 | 0.41 |
| McF |  | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 |
| DOcF | % | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Amount of CH4 flared | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Amount of CH4 for energy re | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/t | 0.029 | 0.029 | 0.030 | 0.031 | 0.032 | 0.033 | 0.034 | 0.033 | 0.032 | 0.031 |

CRF Table 5.A.3: [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] (Part 3 of 3)

| [5. Waste][5.A Solid Waste Disposal] [5.A.3 Uncategorized Waste Disposal Sites] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Annual waste at the SWDS | kt | 0.41 | 0.42 | 0.42 | 0.43 | 0.43 | 0.44 | 0.44 | 0.44 | 0.44 | 0.44 |
| McF |  | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 | 0.60 |
| DOcF | % | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Amount of CH4 flared | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Amount of CH4 for energy recovery | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/t | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | t/t | 0.030 | 0.030 | 0.029 | 0.028 | 0.028 | 0.028 | 0.027 | 0.027 | 0.028 | 0.028 |

CRF Table 5.C.2.2.a: [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic][5.C.2.2.a Municipal Solid Waste] (Part 1 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.54 | 0.53 | 0.53 | 0.53 | 0.52 | 0.52 | 0.52 | 0.52 | 0.51 | 0.51 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.004 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 |
| N2O | kt | 0.00005 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 |
| CH4 | kg/t | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 |
| N2O | kg/t | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 5.C.2.2.a: [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic][5.C.2.2.a Municipal Solid Waste] (Part 2 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.50 | 0.50 | 0.48 | 0.46 | 0.44 | 0.42 | 0.40 | 0.40 | 0.40 | 0.41 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.04 | 0.04 |
| CH4 | kt | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 |
| N2O | kt | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00003 | 0.00003 | 0.00003 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 |
| CH4 | kg/t | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 |
| N2O | kg/t | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 5.C.2.2.a: [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic][5.C.2.2.a Municipal Solid Waste] (Part 3 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open burned | kt | 0.41 | 0.42 | 0.42 | 0.43 | 0.43 | 0.44 | 0.44 | 0.44 | 0.44 | 0.44 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 | 0.003 |
| N2O | kt | 0.00003 | 0.00003 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 | 86.73 |
| CH4 | kg/t | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 | 6.50 |
| N2O | kg/t | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 5.D: 	[5. Waste][5.D Wastewater Treatment and Discharge] (Part 1 of 3)

| [5. Waste][5.D Wastewater Treatment and Discharge] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.17 | 0.17 | 0.17 | 0.18 | 0.18 | 0.19 | 0.19 | 0.21 | 0.22 | 0.23 |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00005 | 0.00005 | 0.00004 | 0.00004 | 0.00003 | 0.00003 |
| Nox | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Population | 1000s | 1.568 | 1.537 | 1.531 | 1.525 | 1.519 | 1.513 | 1.507 | 1.495 | 1.484 | 1.472 |
| Protein consumption | kg/person/yr | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 |
| Fraction of nitrogen in protein |  | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| Factor of non-consumed protein added to the wastewater |  | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 |
| Factor of industrial and commercial co-discharged protein into the sewer system |  | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 |
| Degree of utilization of modern, centralized WWT plants | % | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.D: 	[5. Waste][5.D Wastewater Treatment and Discharge] (Part 2 of 3)

| [5. Waste][5.D Wastewater Treatment and Discharge] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.24 | 0.26 | 0.25 | 0.24 | 0.23 | 0.22 | 0.21 | 0.22 | 0.22 | 0.22 |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kt | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Population | 1000s | 1.461 | 1.449 | 1.389 | 1.330 | 1.270 | 1.211 | 1.151 | 1.162 | 1.173 | 1.183 |
| Protein consumption | kg/person/yr | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 |
| Fraction of nitrogen in protein |  | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| Factor of non-consumed protein added to the wastewater |  | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 |
| Factor of industrial and commercial co-discharged protein into the sewer system |  | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 |
| Degree of utilization of modern, centralized WWT plants | % | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.D: 	[5. Waste][5.D Wastewater Treatment and Discharge] (Part 3 of 3)

| [5. Waste][5.D Wastewater Treatment and Discharge] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.23 | 0.23 | 0.24 | 0.24 | 0.25 | 0.26 | 0.26 | 0.26 | 0.26 | 0.26 |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kt | 0.00001 | 0.00001 | 0.00001 | 0.000004 | 0.000003 | 0.000001 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Population | 1000s | 1.194 | 1.205 | 1.221 | 1.237 | 1.253 | 1.269 | 1.285 | 1.285 | 1.285 | 1.285 |
| Protein consumption | kg/person/yr | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 |
| Fraction of nitrogen in protein |  | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| Factor of non-consumed protein added to the wastewater |  | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 |
| Factor of industrial and commercial co-discharged protein into the sewer system |  | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 |
| Degree of utilization of modern, centralized WWT plants | % | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 | 2.00 |

CRF Table 5.D.1: 	[5. Waste][5.D Wastewater Treatment and Discharge] [5.D.1 Domestic Wastewater] (Part 1 of 3)

| [5. Waste][5.D Wastewater Treatment and Dischargel][5.D.1 Domestic wastewater] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Total organic product | kt DC | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| Sludge removed | kt DC | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| N in effluent | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.005 | 0.004 | 0.003 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Amount of CH4 flared | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Amount of CH4 for energy re | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| N2O | kt | 0.0001 | 0.0001 | 0.0001 | 0.0001 | 0.00005 | 0.00005 | 0.00004 | 0.00004 | 0.00003 | 0.00003 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/kg DC | 0.14 | 0.14 | 0.15 | 0.16 | 0.16 | 0.17 | 0.18 | 0.19 | 0.21 | 0.22 |
| N2O | kg N2O-N/kg N | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |

CRF Table 5.D.1: 	[5. Waste][5.D Wastewater Treatment and Discharge] [5.D.1 Domestic Wastewater] (Part 2 of 3)

| [5. Waste][5.D Wastewater Treatment and Dischargel] [5.D.1 Domestic wastewater] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Total organic product | kt DC | 0.04 | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Sludge removed | kt DC | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| N in effluent | kt | 0.003 | 0.002 | 0.002 | 0.002 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Amount of CH4 flared | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Amount of CH4 for energy re | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| N2O | kt | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/kg DC | 0.24 | 0.25 | 0.26 | 0.26 | 0.26 | 0.26 | 0.27 | 0.27 | 0.27 | 0.27 |
| N2O | kg N2O-N/kg N | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |

CRF Table 5.D.1: 	[5. Waste][5.D Wastewater Treatment and Discharge] [5.D.1 Domestic Wastewater] (Part 3 of 3)

| [5. Waste][5.D Wastewater Treatment and Dischargel] [5.D.1 Domestic wastewater] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Total organic product | kt DC | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.04 | 0.04 |
| Sludge removed | kt DC | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| N in effluent | kt | 0.001 | 0.001 | 0.001 | 0.0005 | 0.0003 | 0.0002 | NO | NO | NO | NO |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Amount of CH4 flared | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Amount of CH4 for energy recovery | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| N2O | kt | 0.00001 | 0.00001 | 0.00001 | 0.000004 | 0.000003 | 0.000001 | NO | NO | NO | NO |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/kg DC | 0.28 | 0.28 | 0.28 | 0.29 | 0.29 | 0.30 | 0.30 | 0.30 | 0.30 | 0.30 |
| N2O | kg N2O-N/kg N | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | NO | NO | NO | NO |

# Annex 8: Agricultural emissions from fertilisers and by livestock type

## A8.1 	Agricultural emissions from fertilisers

Fertilisers provide the nutrients to grow and nourish pastures and crops. Nitrogen, phosphate, potassium and sulphur are the four most important nutrients for pasture and crop yields and sustainable food production.

New Zealand’s farmers use both organic and synthetic nitrogen (N) fertilisers. The main types of synthetic N fertilisers used in New Zealand include urea, diammonium phosphate (DAP) and ammonium sulphate. Urea fertiliser accounts for the majority of the synthetic N fertiliser used in New Zealand. It is mainly applied to dairy pasture land to boost pasture growth during the autumn and spring months.

All nitrogen fertilisers provide N inputs to agricultural soils that result in direct and indirect emissions of nitrous oxide (N2O) (see figure 5.5.1 in chapter 5). Urea also releases carbon dioxide (CO2).

Emissions from organic fertilisers come solely from animal manure. Most animal manure in New Zealand is excreted directly onto pasture, but some manure from dairy farms is kept in manure management systems and applied to soils as an organic fertiliser (see table 5.3.2 in chapter 5, for further details). Some manure is also collected but not stored; rather, it is spread directly onto pasture daily (e.g., swine manure and some dairy manure).

Emissions of N2O from all synthetic (including urea) N fertilisers are reported in categories 3.D.1.1 and 3.D.1.2 respectively. Emissions of CO2 from urea are not included under synthetic N fertilisers and are reported under a dedicated category 3.H.

##### 2019

In 2019, the combined effect of synthetic and organic N fertilisers amounted to 23.6 per cent of emissions from the Agricultural soils category and 6.1 per cent from all agricultural emissions (when CO2-e from urea is included).

Table A8.1.1 shows comparisons of both N2O and CO2 emissions from fertilisers to New Zealand’s national totals for each gas and New Zealand’s gross emissions.

Table A8.1.1 	Direct and indirect emissions by fertiliser in 2019

| Fertiliser type | Emissions |  | Percentage of |  |
| --- | --- | --- | --- | --- |
|  |  |  | N2O emissions from Agriculture soils by gas | all emissions from Agriculture |
|  | Gas/source | kt CO2-e | (%) | (%) |
| Synthetic N fertiliser | Direct N2O emissions | 1,429.3 | 18.4 | 3.6 |
|  | Urea | 989.1 | 12.7 | 2.5 |
|  | Other synthetic N fertilisers | 440.2 | 5.7 | 1.1 |
|  | Indirect N2O emissions from all synthetic N fertilisers | 296.0 | 3.8 | 0.7 |
|  | All N2O (direct + indirect) from synthetic N fertilisers | 1,725.3 | 22.2 | 4.4 |
|  | CO2 from urea | 570.7 | NA | 1.4 |
| Organic fertiliser | Direct N2O emissions | 76.9 | 1.0 | 0.2 |
|  | Indirect N2O emissions | 30.7 | 0.4 | 0.1 |
|  | All N2O (direct + indirect) from organic fertilisers | 107.7 | 1.4 | 0.3 |

Note: 	NA = not applicable. Columns may not add up due to rounding.

##### 1990–2019

The total amount of fertilisers applied to agricultural soils in New Zealand has significantly increased since 1990. Synthetic N fertiliser applied to agricultural land has increased by 662.7 per cent since 1990, while the use of organic fertiliser has grown by 174.8 per cent (table A8.1.2).

Table A8.1.2 	Use of fertilisers in New Zealand in 1990 and 2019

| Fertiliser type | 1990 |  |  | 2019 |  |  | Change in the use between 1990 and 2019 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Application | Percentage of |  | Application | Percentage of |  |  |  |
|  | tonnes (N) | synthetic N fertiliser (%) | all fertilisers (%) | tonnes (N) | synthetic N fertiliser (%) | all fertilisers (%) | tonnes (N) | (%) |
| Synthetic N fertiliser (ammonium phosphates, for example, DAP) | 34,679 | 58.5 | 46.3 | 94,000 | 20.8 | 19.0 | 59,321 | 171.1 |
| Urea | 24,586 | 41.5 | 32.8 | 358,000 | 79.2 | 72.3 | 333,414 | 1,356.1 |
| Total synthetic N fertilisers (urea + ammonium phosphates) | 59,265 | 100.0 | 79.1 | 452,000 | 100.0 | 91.3 | 392,735 | 662.7 |
| Organic fertilisers (animal manure applied to soils) | 15,644 | NA | 20.9 | 42,992 | NA | 8.7 | 27,348 | 174.8 |

Note: 	DAP = diammonium phosphate; NA = not applicable. Columns may not add up due to rounding.

Between 1990 and 2019, N2O emissions from synthetic N fertiliser (both direct and indirect emissions, including urea) have increased by 532.8 per cent, while total emissions from these fertilisers (N2O and CO2) have increased by 636.3 per cent. For the same period, total emissions from organic fertilisers increased by 126.9 per cent (see table A8.1.3).

In 1990 and 2019 respectively, 0.8 per cent and 4.4 per cent of total agricultural emissions originated from N2O from synthetic N fertiliser. Total emissions from synthetic N fertiliser (including urea) have increased from 0.9 per cent to 5.8 per cent of total agricultural emissions for 1990 and 2019 respectively (see chapter 5 for further details).

Table A8.1.3 	Emissions from fertilisers in 1990 and 2019

|  |  |  | Synthetic N fertilisers | Organic fertilisers |
| --- | --- | --- | --- | --- |
| 1990 | N2O emissions | kt CO2-e | 272.6 | 47.4 |
|  | CO2 emissions | kt | 39.2 | NA |
|  | Total emissions | kt CO2-e | 311.8 | 47.4 |
| 2019 | N2O emissions | kt CO2-e | 1,725.3 | 107.7 |
|  | CO2 emissions | kt | 570.7 | NA |
|  | Total emissions | kt CO2-e | 2,296.1 | 107.7 |
| Change in N2O emissions between 1990 and 2019 |  | kt CO2-e | 1,452.7 | 60.2 |
| Percentage change in N2O emissions between 1990 and 2019 |  | % | 532.8 | 126.9 |
| Change in all emissions between 1990 and 2019 |  | kt CO2-e | 1,984.21 | 60.2 |
| Percentage change in all emissions between 1990 and 2019 |  | % | 636.3 | 126.9 |

Note: 	NA = not applicable.

## A8.2 	Agricultural emissions by livestock type

This section covers distribution of greenhouse gas emissions from the Agriculture sector by livestock type in 1990, 2018 and 2019, including the changes in emissions. Table A8.2.1 shows total emissions of all greenhouse gases across all categories of the Agriculture sector. For further details on emissions by gas and by category, refer to the common reporting format (CRF) tables (sector 3 – Agriculture).

Table A8.2.1	Total emissions by livestock type in 1990, 2018 and 2019

| Livestock type | 1990 | 2018 | 2019 | 1990–2019 |  | 2018–19 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | kt CO2-e |  |  | kt CO2-e | (%) | kt CO2-e | (%) |
| Dairy cattle | 8,006.4 | 18,533.9 | 18,460.1 | 10,453.7 | 130.6 | –73.8 | –0.4 |
| Beef cattle | 7,040.6 | 6,646.3 | 7,000.8 | –39.8 | –0.6 | 354.6 | 5.3 |
| Sheep | 16,278.4 | 9,745.3 | 9,595.8 | –6,682.6 | –41.1 | –149.5 | –1.5 |
| Deer | 517.4 | 585.8 | 568.5 | 51.0 | 9.9 | –17.3 | –3.0 |
| Swine | 102.0 | 80.5 | 71.5 | –30.5 | –29.9 | –9.0 | –11.2 |
| Goats | 262.8 | 26.5 | 27.9 | –234.9 | –89.4 | 1.4 | 5.4 |
| Horses | 78.4 | 33.8 | 32.0 | –46.3 | –59.1 | –1.7 | –5.1 |
| Alpaca | 0.1 | 2.4 | 2.8 | 2.7 | 2,465.5 | 0.4 | 18.2 |
| Mules and asses | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Poultry (including all types of poultry) | 26.3 | 56.3 | 59.4 | 33.1 | 125.7 | 3.1 | 5.6 |
| Total, all livestock types | 32,312.5 | 35,710.6 | 35,818.9 | 3,506.4 | 9.8 | 108.3 | 0.3 |

Note: 	Columns may not add up due to rounding.

Value is for 2019. In 1990, the value was EF 7.4 kg CH4/head/year. Values for the intermediate years between 1990 and 2018 are calculated based on the estimated proportion of dairy goats in the overall goat population.

As was reported in the 2010 submission, that is, the first year that alpacas were included in New Zealand’s Greenhouse Gas Inventory (Ministry for the Environment, 2010).

Paragraph 33(c) of annex to Decision 2/CMP.7 contained in document FCCC/KP/CMP/2011/10/Add.1, p 18.

MMS stands for a manure management system (see chapter 5).

MMS stands for a manure management system (see chapter 5).

The category names and CRF codes for source categories are consistent with New Zealand’s CRF tables. Only the tables that include reported emissions (by value, IE or NE) are included. For explanations and methodological issues, please refer to chapter 8.